VULCRAFT®

STEEL BAR GRATING

Powerful Partnerships. Powerful Results

# CONTENTS

|  | Introduction Why Grating? 2 Who We Are 4 Sales Contacts 5 |
| --- | --- |
|  | Engineering Nomenclature 6 Controlling Standards 10 Fabrication 11 How to Specify 12 Safe Installation 17 Manufacturing Tolerances 19 Welding Standards 21 Anchorage Details 24 |
|  | Load Tables Standard Duty Imperial 26 19W4 - SD 28 15W4 - Close-Mesh 30 11W4 - ADA 32 7W4 - Heels-Friendly 34 Metric 36 30W102 - SD 38 24W102 - Close-Mesh 40 17W102 - ADA 42 11W102 - Heels-Friendly 44 |
|  | Load Tables Heavy Duty Imperial 46 Vehicular Loads 48 19HW4 - HD 52 15HW4 - HD Close-Mesh 56 30HW4 - Wide-Gap 60 38HW4 - Extra-Wide Gap 64 Metric 69 Vehicular Loads 71 30HW102 - HD 74 24HW102 - HD Close-Mesh 78 48HW102 - Wide-Gap 82 60HW102 - Extra-Wide Gap 91 |
|  | Load Tables Stair Treads Imperial 90 Metric 92 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                 1

# INTRODUCTION

## WHY GRATING?

•     Easy Installation and Fabrication

•    Adaptable to Complex Floor Layouts

•    Allows Passage of Light and Air

•     High Strength-to-Weight Ratio

•     Durable and Long Lasting Surface

APPLICATIONS

•     Industrial Flooring

•    Walkways

•     Bridge Decks

•     Trenches

•     Steel Mills

•    Warehouses

•     Chemical Plants

•    Water Treatment Plants

•    Power Plants

•     Oil Refineries

•    Agricultural Facilities

STANDARD DUTY (SD)

Standard Duty

Standard Duty Grating is the most common  type of grating used in the industrial flooring market. The open grid construction provides for maximum passage for light, air circulation and drainage.

Close Mesh

When a certain bar depth must be held but

standard duty is not sufficient, Close Mesh

moves the bars closer to gain more strength and stiffness. This may also be warranted if the bar gap on Standard Duty is wider than desired.

ADA-Complaint

When the Grating needs to adhere to the

guidelines of the Americans-with-Disabilities

Act (ADA), an even-smaller bar gap is required.

Heel Friendly

Also ADA-Compliant, Heel-Friendly grating

provides an even tighter bar spacing to ensure that high heels, canes, and other narrow items will be difficult to get wedged in the gaps.

2                                                                                                                VULCRAFT GRATING  |  www.vulcraft.com/grating

INTRODUCTION

HEAVY DUTY (HD)

Heavy-Duty Welded Grating has the strength for heavy-duty load areas such as airfields,

industrial plants, truck and bus terminals,

parking lots and railroad yards. Common uses are flooring, driveways, subway and tunnel

ventilation grilles, curb inlet grates, ramps, docks, etc.

STAIR TREADS

Welded steel stair treads are the most widely used for their strength and ease of installation and are universally used in most industrial

and commercial applications. Stair Treads can be ordered with a serrated surface for additional safety.

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                 3

# INTRODUCTION | WHO WE ARE

| Nucor offers steel products that range from bar grating to the heaviest hot rolled beam sections produced in North America. Through implementation and refinement of best practices, we continue to grow as a company. Nucor’s pay-for-performance policy reflects a commitment to manufacturing the highest quality products while maintaining a safety record that is the envy of the industry. Nucor serves the agricultural, automotive, construction, energy, furniture, machinery, metal building, railroad, recreational equipment, shipbuilding, heavy truck, and trailer industries. Which is to say, we are integral to North American industry. | Nucor and its subsidiary divisions manufacture: • Bars (carbon and alloy steel) • Sheets/Flatrolled • Open Web Steel Joists • Steel Decks • Cold finished steel • Metal building systems • Steel Grating • Wire and wire mesh • Conduit • Beams • Plates • Joist Girders • Fabricated concrete reinforcing steel • Steel fasteners • Piling • Tube |
| --- | --- |

4                                                                                                                VULCRAFT GRATING  |  www.vulcraft.com/grating

INTRODUCTION

SALES CONTACTS

| FOR PANEL SALES: WEST GratingSalesWest@vulcraft.com | EAST GratingSalesEast@vulcraft.com |
| --- | --- |

FOR PROJECTS WITH GRATING IN PLATFORMS, WALKWAYS, STAIRS, ETC. VISIT WWW.VULCRAFT.COM/CONTACT-US

TO FIND YOUR LOCAL VULCRAFT SALES OFFICE.

tip

If you are interested in a grating type not represented in this manual, please contact a sales representative.

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                 5

# ENGINEERING | NOMENCLATURE

Span

Bearing /Bars

Cross Bars

Cross Bar Spacing

Surface Type

Bearing Bar Spacing

Bar

Depth

Bearing Bars  load carrying, span of grating

Cross Bars  connecting bars, runs along width of grating

Finish  coating applied-painted black, galvanized Serrated  notching process for non-slip surface

Widths  overall dimension measured perpendicular to bearing bars Span  distance running along direction of bearing bars

Banding  bar of the same size as bearing bar used to close open ends Kick Plate  flat bar welded to outer edge with projection above grating

| Bar Spacing — # of 1/16” from center to center of vertical bearing bars Construction Type — W = Welded | Cross Bar — Spacing in inches of the cross bars Bar Size — Size of bearing bars | Serration — Nothing = Smooth SER = Mechanical Serrated MSR = Mill Serrated Finish — Nothing = Unpainted GAL = Galvanized |
| --- | --- | --- |

BLK = Black Paint

6                                                                                                                VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD-CARRYING CAPABILITY SPAN

Usually this space has structural supports to carry the load. The distance between supports is called the span. Bearing bars run in the direction of the span. Deflection is generally limited to ¼-inch (6mm) only

in order to limit an uncomfortable movement of the traffic surface.

ENGINEERING

END

DIMENSION

BEARING BARS

CROSS BARS

CROSS BAR SPACING

PANEL SPAN

EDGE ANGLE

PANEL WIDTH                        SUPPORT

BEARING BAR SPACING

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                 7

# ENGINEERING | NOMENCLATURE

STANDARD DUTY GRATING (1/8”, 3/16” BAR THICKNESS)

| Standard TYPE 19W4 4” CTRS. 1” Close-Mesh TYPE 15W4 4” CTRS. 15 16 ADA-Compliant TYPE 11W4 4” CTRS. 11_ ” 16 Heels-Friendly TYPE 7W4 4” CTRS. | CROSS RODS CROSS RODS CROSS RODS _1 ” 2 CROSS RODS | 19W4 % Open Area 1/8” Bars 3/16” Bars; 80.0% 74.2%. 15W4 % Open Area 1/8” Bars 3/16” Bars; 76.8% 69.5%. 11W4 % Open Area 3/16” Bars 63.3% 7W4 % Open Area 3/16” Bars 47.7% |
| --- | --- | --- |

_7  ”

16

4                                                                                                                            tip

Cross Bars are not load bearing

The bearing bars are the bars that carry the load and the

cross bars hold the bearing bars in place creating the shape of the panel.  In order to function properly, make sure that

both ends of each bearing bar is supported by the load bearing structure and avoid the situation depicted in the picture to the left.

_1 ”

8                                                                                                                VULCRAFT GRATING  |  www.vulcraft.com/grating

HEAVY-DUTY GRATING (1/4”, 3/8” BAR THICKNESS)

Heavy-Duty

ENGINEERING

19HW4 % Open Area

| 1/4” Bars 3/8” Bars | 65.3% 51% |
| --- | --- |

TYPE 19HW4      4” CTRS.             CROSS RODS

1”

HD Close-Mesh

15HW4 % Open Area

| 1/4” Bars 3/8” Bars | 59.1% 41.7% |
| --- | --- |

TYPE 15HW4       4” CTRS.              CROSS RODS

15_ ”

16

Wide-Gap

30HW4 % Open Area

| 1/4” Bars 3/8” Bars | 73.7% 63.5% |
| --- | --- |

TYPE 30HW4      4” CTRS.            CROSS RODS

1  ”

Extra-Wide Gap

38HW4 % Open Area

| 1/4” Bars 3/8” Bars | 75.8% 66.7% |
| --- | --- |

TYPE 38HW4      4” CTRS.             CROSS RODS

2 ”

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                9

## ENGINEERING | CONTROLLING STANDARDS                                  FABRICATION

All specification and engineering data provided in this document is based on the below National Associations of Architectural Metal Manufacture’s (NAAMM) Guides.

See the NAAMM guides for addtional information such as Standard Specifications and Code of

Standard Practice.

NOT JUST GRATING FABRICATION

Your partner for structural steel as well as grating

detailing and fabrication. Vulcraft has the capability and capacity to handle even the toughest structural steel detailing and fabrication projects.

Contact us today with how we can assist you in meeting your customer’s needs.

ANSI/NAAMM HMMA MBG 531-17 - Metal Bar Grating Manual, 8th Ed

ANSI/NAAMM HMMA MBG 532-19

Heavy Duty Metal Bar Grating Manual, 6th Ed

ANSI/NAAMM MBG 533-09

Welding Standards for Fabrication of Steel,

Stainless Steel and Aluminum Bar Grating, 3rd Ed

NAAMM MBG 534-12

Metal Bar Grating Engineering Design Manual

ENGINEERING

10                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                                11

## ENGINEERING | HOW TO SPECIFY

DRAWING AND GENERAL NOTES EXAMPLE

Standard Duty

•      Description of Grating (see standard marking system on pp. 6-9)

•      Drawing Showing:

-   area to be covered, with cutouts

-   span with direction of bearing

-    method of support

-    all critical dimensions

•    Type of Anchorage

•     Finish

•    Shipping Instructions

Stair Treads

•      Description of Grating (see standard marking system on pp. 6-9)

•      Type of Nosing

•      Dimensions (length and width of tread)

•      Number of Treads

•       Finish

•      Shipping Instructions

Heavy Duty

•      Description of Grating (see standard marking system on pp. 6-9)

•      Drawing Showing:

-    area to be covered, with cutouts

-    span with direction of bearing

-    method of support

-    all critical dimensions

-    serrated or plain surface

•      Type of Anchorage

•       Finish

•      Shipping Instructions

ENGINEERING

| General Notes Examples The following are examples of suggested notes that should be included on drawings where Grating products are being specified: Grating Unless otherwise noted on the drawings, use steel grating consisting of 19W4 -1 1/4” deep grating, to be smooth and painted black and 1 1/2” deep grating is to be serrated and galvanized. Attachment is clip type or indicate that grating to be welded to supports. Treads Unless otherwise noted on the drawings, treads are to be the same grating type as walkway grating, stair treads are to have checkered plate nosing. | Edge Sections: Edge conditions sections are a very important part of proper grating detailing and fabrication. Items to be shown in section drawings should be: • Grating edge dimension in relation to grid line or support • Kick Plates • Hand Rail Conditions Sections should also be shown for any cutouts that need to be created in the grating panels. Sections can be shown as independent sections specific to grating or as a part of the typical construction section drawings, as long as the grating requirements are clearly shown. The following pages show examples of some typical grating sections. These sections can be downloaded from our website www.vulcraft.com/grating. |
| --- | --- |

12                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  | www.vulcraft.com/grating                                                                                                               13

Example Grating Edge Sections:                                                                                                                                                    Guidance for Grating Plan View:

ENGINEERING

Guidance for Specifying Cutouts:

14                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               15

## ENGINEERING | HOW TO SPECIFY                                             ENGINEERING | SAFE INSTALLATION

Grating Specification/Detailing Checklist

。 Grating Description

。 Grating Type (19W4, 15W4, etc.)

。 Bar Size

。 Finish

。 Smooth/Serrated

。 Banding/Kick Plate/Hand Rails

。 Stair Tread Description

。 Grating Type (19W4, 15W4, etc.)

。 Bar Size

。 Finish

。 Smooth/Serrated

。 Nosing Type

。 Attachment

。 Clips

。 Welds

。 Lugs

。 Layout

。 Area to be Covered (See Drawing Example)

。 Edge Conditions

。 Cutouts

。 Span

。 Supports

。Approvals

。RFI’s

。Sequencing/Shipping

ENGINEERING

| • Grating panels must be installed with cross bars on top side. • It is NOT recommended to notch bearing bars at supports to maintain proper elevation. If notching is required for installation, manufacturer should be consulted. • Metal should always be used for grating supports. • A minimum of 1” bearing shall be provided for Light Duty Steel Grating. For Heavy Duty Steel Grating, 1” minimum bearing shall be provided for bearing bar depths up to 2-1/4”, and 2” minimum bearing shall be provided for depths of 2-1/2” and over. This bearing surface does not include the support angle fillet radius noted above (ref. to support details). • Banding may have less depth than bearing bars for trench grating to allow drainage. Full depth banding will be provided unless otherwise specified. | • Clearances shown in Figure 1 (below) are recommended, but may vary in accordance with dimensional tolerances. • Standard and Heavy duty grating should be designed to have structural support under each bearing bar at cutouts. • As shown in Figure 2 (next page), all rectangular cutouts are made to the next bearing bar beyond the penetration with a clearance not to exceed bearing bar spacing. • Cutouts for circular obstructions are recommended to be at least 2” larger in diameter than the obstruction. It is further recommended that cutouts for all piping 4” or less be made in the field. |
| --- | --- |

Figure 1

16                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               17

## ENGINEERING | SAFE INSTALLATION                                        ENGINEERING | MANUFACTURING TOLERANCES

Guidance for Specifying Cutouts:

SPECIFIED CUTOUT - 4"x6"

ACTUAL CUTOUT - 5-3/4"X6"

BEARING BAR SPACING = 1 -3/16"

Figure 2

18                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating

BEARING BAR AND CROSS ROD TOLERANCES

12

Cross Rod Spacing and Alignment

Cross rods should not vary more than a slope of 1/8” to 12” in either direction from perpendicular alignment with bearing

ENGINEERING

bars. The tolerance of the cross bar spacing for 5, in length is +/- 1/4”

+/- 1/4" C-C END CROSS BARS IN 5'- 0 " LENGTH

Cross Rod Position

The top the cross rod should not

LSD - 1/16" HD - 1/8"

projected more than 1/16” above the top

of the bearing bars for standard grating

CROSS RODS —

(1/8” for heavy-duty) and should not extend more than 1/8” from side of bearing bars.

BEARING BARS—

Bearing Bar Lean

Bearing bar lean must not exceed a slope of 1-1/4” to 12.

2

1 - 1/4

1 2

Cross Bar Lean

Cross bar lean must not exceed a slope of 2-1/2” to 12.

2 - 1/2

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               19

## ENGINEERING | MANUFACTURING TOLERANCES                     WELDING STANDARDS

TOE PLATES

PANEL TOLERANCES

Longitudinal Bow

Longitudinal bow should be less than 1/200 of the length for the standard

grating (1/240 for heavy duty).

ENGINEERING

Traverse Bow

Before banding, the traverse bow should be less than 1/8” per foot or width.

Attachment to Length of Bearing Bar

Toe plate to be welded with alternating 1/8” fillet welds, 1” long every 12.”

Overall Dimension and Squareness

D = Overall diagonal dimension

W = Width of panel including cross rod extensions outside of bearing bars

L =  Length of bearing bars

Attachment to End of Bearing Bar

Load-carrying toe plates to be welded at each bearing bar with a fillet weld the size of the bearing bar

thickness (t), 3/4” long. Non-load-carrying toe plates to be welded to bearing bars with 1/8” fillet weld (3/16” for HD), 3/4” long every 5”.

SWEEP 1/200 OF SPAN

20                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               21

ENGINEERING | WELDING STANDARDS

BANDING                                                                                                                                  STAIR TREADS

Load Banding (must specify)

Load carrying end band must be welded with fillet weld the size of bearing bar thickness (t) and the length of the bearing bar depth (d)

at each bearing bar. This spec is standard for grating. Refer to NAAMM MBG 532 for HD specification.

22                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              23

## ENGINEERING | ANCHORAGE DETAILS

ANCHORAGE TYPES

CLIP (SD)

The clip bridges two bearing bars and is

attached with 1/4” self-tapping bolt, 1/4” self

threader, 1/4” weld stud or 1/4” bolt and nut

when hole is drilled through supporting flange. Fasteners should be  specified separate from clips. (Bolts not included)

WELDED LUG (HD)

Weld lugs may be used for installations where grating is subject to removal. Weld lugs are

shop fabricated and must be specified at time

of grating order. Fasteners are 1/4” minimum and supplied by others.

FIELD WELDED

Recommended for all permanently installed

grating panels. Welds should be 3/16” fillet

welds, 3/4” long located approximately 6” from

each side of panel (4 locations) and one weld in middle of panel at each intermediate support.

24

VULCRAFT GRATING  |  www.vulcraft.com/grating

CLIP TYPES

F-10 CLIPS

F-10 Clips are available for use with 19 space grating and can work with any bar depths from 1” to 6”. Different Tek-screw lengths are required for different bar depths.

G CLIPS

G Clips can be used with 19, 15, 30 and 38 space grating, and can work with bar depths from 1” to 2”.

G-CLIPS

| Bar Spacing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bar Depth | 11 | 15 | 19 | 30 | 38 |
| 1” | RSCGC-1A or RSCGC-2A | GN-1A or GN-2A | GG-1A or GG-2A | FFGN-1A or FFGN-2A | FFGN-1A or FFGN-2A |
| 11/4” | RSCGC-1B or RSCGC-2B | GN-1B or GN-2B | GG-1B or GG-2B | FFGN-1B or FFGN-2B | FFGN-1B or FFGN-2B |
| 11/2” | RSCGC-1C or RSCGC-2C | GN-1C or GN-2C | GG-1C or GG-2C | FFGN-1C or FFGN-2C | FFGN-1C or FFGN-2C |
| 13/4” | RSCGC-1D or RSCGC-2D | GN-1D or GN-2D | GG-1D or GG-2D | FFGN-1D or FFGN-2D | FFGN-1D or FFGN-2D |
| 2” | RSCGC-1E or RSCGC-2E | GN-1E or GN-2E | GG-1E or GG-2E | FFGN-1E or FFGN-2E | FFGN-1E or FFGN-2E |

“1” Clips (I.E., GG-1A) are for use with flange members up to 3/4” thick

“2” Clips (I.E., GG-2A) are for use with flange members from 3/4” to 1 1/2” thick

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              25

## LOAD TABLES | STANDARD DUTY, IMPERIAL

OVERVIEW

# Standard

Standard Duty Grating is the most common type of grating  used in the industrial flooring  market. The open grid construction provides for maximum passage for light, air circulation and drainage.

Close-Mesh

When a certain bar depth must be held but standard duty is not sufficient, Close Mesh moves the bars closer to gain more strength and stiffness. This may also be warranted if the bar gap on Standard Duty is wider than desired.

ADA-Compliant

When the Grating needs to adhereto the guidelines of  the  Americans-with-Disabilities  Act  (ADA),  an even-smaller bar gap is required. The ADA requires no more than a 1/2” opening as part of Chapter 3, Section 302.3 of the ADA Guide published by the Unites States Access Board.

Heels-Friendly

Also ADA-Compliant, Heel-Friendly grating provides an  even tighter  bar  spacing to  ensure that  high heels, canes, and other narrow items will be difficult to get wedged in the gaps.

DESIGN CRITERIA

The following tables of safe loads have been calculated using these design parameters:

LOAD TABLES - SD IMPERIAL

|  | Uniform Load |  | Concentrated Load |  |
| --- | --- | --- | --- | --- |
| Determine M: | M = FS 12 |  | M = FS 12 |  |
| Determine U or C: | 8M U = L2 |  | C = |  |
| Check D*: | D= | 5UL(L x 12)3 384 EI | D= | C(L x 12)3 48 EI |

*Deflection should be limited to 1/4” under 100lb/ft2  uniform load to afford pedestrian comfort.

U =   Uniform Load - lbs/ft2

C =   Concentrated Load - lbs/ft of grating width

S =   Section Modulus - in3/ft of grating width I =   Moment of Inertia - in4/ft of grating width

L =   Simple Clear Span - feet D =   Deflection - inches

E =   Modulus of Elasticity

tip

F =   Allowable Bending Stress M =   Bending Moment

Vulcraft Online Bar Grating Design Tools

If you are uncertain about which type of grating to use, try our on-line calculator. Enter your project specs

and our calculator will recommend the appropriate grating. This powerful on-line tool allows you to compare alternatives so you can optimize weight, cost, or both. It saves you valuable time by eliminating manual

calculations and the risk of errors. Visit www.vulcraft.com/design-tools/bar-grating-design-aid

26

VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               27

LOAD TABLES | STANDARD DUTY, IMPERIAL

LOAD TABLES - SD

| Grating Type: 19W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ⅛" | 5.00 | 51 | U Du C Dc | 632 0.07 632 0.06 | 404 0.12 505 0.09 | 281 0.17 421 0.13 | 206 0.23 361 0.18 | 158 0.30 316 0.24 | 125 0.38 281 0.30 |  |  |  |  |  |  |  | 0.211 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.105 |
| 1" x ³/16" | 7.15 | 57 | U Du C Dc | 947 0.07 947 0.06 | 606 0.12 758 0.09 | 421 0.17 632 0.13 | 309 0.23 541 0.18 | 237 187 0.30 0.38 474 421 0.24 0.30 |  | 152 0.47 379 0.37 |  |  |  |  |  |  | 0.316 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.158 |
| 1 ¼" x ⅛" | 6.08 | 61 | U Du C Dc | 987 0.06 987 0.05 | 632 0.09 789 0.07 | 439 0.13 658 0.11 | 322 0.18 564 0.15 | 247 195 0.24 0.30 493 439 0.19 0.24 |  | 158 0.37 395 0.30 | 130 0.45 359 0.36 |  |  |  |  |  | 0.329 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.206 |
| 1 ¼" x ³/16" | 8.76 | 67 | U Du C Dc | 1,480 0.06 1,480 0.05 | 947 0.09 1,184 0.07 | 658 0.13 987 0.11 | 483 0.18 846 0.15 | 370 292 0.24 0.30 740 658 0.19 0.24 |  | 237 0.37 592 0.30 | 196 0.45 538 0.36 | 164 0.54 493 0.43 |  |  |  |  | 0.493 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.308 |
| 1 ½" x ⅛" | 7.15 | 70 | U Du C Dc | 1,421 0.05 1,421 0.04 | 909 0.08 1,137 0.06 | 632 0.11 947 0.09 | 464 0.15 812 0.12 | 355 281 0.20 0.25 711 632 0.16 0.20 |  | 227 0.31 568 0.25 | 188 0.38 517 0.30 | 158 0.45 474 0.36 |  |  |  |  | 0.474 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.355 |
| 1 ½" x ³/16" | 10.37 | 77 | U Du C Dc | 2,132 0.05 2,132 0.04 | 1,364 0.08 1,705 0.06 | 947 0.11 1,421 0.09 | 696 0.15 1,218 0.12 | 533 421 0.20 0.25 1,066 947 0.16 0.20 |  | 341 0.31 853 0.25 | 282 0.38 775 0.30 | 237 0.45 711 0.36 | 202 0.52 656 0.42 |  |  |  | 0.711 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.533 |
| 1 ¾" x ³/16" | 12.17 | 87 | U Du C Dc | 2,901 0.04 2,901 0.03 | 1,857 0.07 2,321 0.05 | 1,289 0.10 1,934 0.08 | 947 0.13 1,658 0.10 | 725 573 0.17 0.22 1,451 1,289 0.14 0.17 |  | 464 0.27 1,161 0.21 | 384 0.32 1,055 0.26 | 322 0.38 967 0.31 | 275 0.45 893 0.36 | 237 0.52 829 0.42 | 206 0.60 774 0.48 |  | 0.967 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.846 |
| 2" x ³/16" | 13.78 | 96 | U Du C Dc | 3,789 0.04 3,789 0.03 | 2,425 0.06 3,032 0.05 | 1,684 0.08 2,526 0.07 | 1,237 0.11 2,165 0.09 | 947 749 0.15 0.19 1,895 1,684 0.12 0.15 |  | 606 0.23 1,516 0.19 | 501 0.28 1,378 0.23 | 421 0.34 1,263 0.27 | 359 0.39 1,166 0.31 | 309 0.46 1,083 0.36 | 269 0.52 1,011 0.42 | 237 0.60 947 0.48 | 1.263 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.263 |
| 2 ½" x ³/16" | 17.00 | 113 | U Du C Dc | 5,921 0.03 5,921 0.02 | 3,789 0.05 4,737 0.04 | 2,632 0.07 3,947 0.05 | 1,933 0.09 3,383 0.07 | 1,480 1,170 0.12 0.15 2,961 2,632 0.10 0.12 |  | 947 0.19 2,368 0.15 | 783 0.23 2,153 0.18 | 658 0.27 1,974 0.21 | 561 0.31 1,822 0.25 | 483 0.36 1,692 0.29 | 421 0.42 1,579 0.34 | 370 0.48 1,480 0.38 | 1.974 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.467 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 19W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 1-3/8 | 2-9/16 | 3-3/4 | 4-15/16 | 6-1/8 | 7-5/16 | 8-1/2 | 9-11/16 | 10-7/8 | 12-1/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 13-1/4 | 14-7/16 | 15-5/8 | 16-13/16 | 18 | 19-3/16 | 20-3/8 | 21-9/16 | 22-3/4 | 23-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 25-1/8 | 26-5/16 | 27-1/2 | 28-11/16 | 29-7/8 | 31-1/16 | 32-1/4 | 33-7/16 | 34-5/8 | 35-13/16 |

Deduct 1/16” for 1/8” bearing bars

28                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - SD IMPERIAL

LOAD TABLES - SD

| Grating Type: 19W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ⅛" | 5.00 | 42 | U Du C Dc | 355 0.10 355 0.08 | 227 0.16 284 0.12 | 158 0.22 237 0.18 | 116 0.30 203 0.24 |  |  |  |  |  |  |  |  |  | 0.118 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.044 |
| 1" x ³/16" | 7.15 | 46 | U Du C Dc | 533 0.10 533 0.08 | 341 0.16 426 0.12 | 237 174 0.22 0.30 355 305 0.18 0.24 |  | 133 0.40 266 0.32 |  |  |  |  |  |  |  |  | 0.178 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.067 |
| 1 ¼" x ⅛" | 6.08 | 51 | U Du C Dc | 632 0.07 632 0.06 | 404 0.12 505 0.09 | 281 206 0.17 0.23 421 361 0.13 0.18 |  | 158 0.30 316 0.24 | 125 0.38 281 0.30 |  |  |  |  |  |  |  | 0.211 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.105 |
| 1 ¼" x ³/16" | 8.76 | 57 | U Du C Dc | 947 0.07 947 0.06 | 606 0.12 758 0.09 | 421 309 0.17 0.23 632 541 0.13 0.18 |  | 237 0.30 474 0.24 | 187 0.38 421 0.30 | 152 0.47 379 0.37 |  |  |  |  |  |  | 0.316 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.158 |
| 1 ½" x ⅛" | 7.15 | 61 | U Du C Dc | 987 0.06 987 0.05 | 632 0.09 789 0.07 | 439 322 0.13 0.18 658 564 0.11 0.15 |  | 247 0.24 493 0.19 | 195 0.30 439 0.24 | 158 0.37 395 0.30 | 130 0.45 359 0.36 |  |  |  |  |  | 0.329 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.206 |
| 1 ½" x ³/16" | 10.37 | 67 | U Du C Dc | 1,480 0.06 1,480 0.05 | 947 0.09 1,184 0.07 | 658 483 0.13 0.18 987 846 0.11 0.15 |  | 370 0.24 740 0.19 | 292 0.30 658 0.24 | 237 0.37 592 0.30 | 196 0.45 538 0.36 | 164 0.54 493 0.43 |  |  |  |  | 0.493 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.308 |
| 1 ¾" x ³/16" | 11.98 | 77 | U Du C Dc | 2,132 0.05 2,132 0.04 | 1,364 0.08 1,705 0.06 | 947 696 0.11 0.15 1,421 1,218 0.09 0.12 |  | 533 0.20 1,066 0.16 | 421 0.25 947 0.20 | 341 0.31 853 0.25 | 282 0.38 775 0.30 | 237 0.45 711 0.36 | 202 0.52 656 0.42 |  |  |  | 0.711 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.533 |
| 2" x ³/16" | 13.78 | 87 | U Du C Dc | 2,901 0.04 2,901 0.03 | 1,857 0.07 2,321 0.05 | 1,289 947 0.10 0.13 1,934 1,658 0.08 0.10 |  | 725 0.17 1,451 0.14 | 573 0.22 1,289 0.17 | 464 0.27 1,161 0.21 | 384 0.32 1,055 0.26 | 322 0.38 967 0.31 | 275 0.45 893 0.36 | 237 0.52 829 0.42 | 206 0.60 774 0.48 |  | 0.967 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.846 |
| 2 ½" x ³/16" | 17.00 | 105 | U Du C Dc | 4,796 0.03 4,796 0.03 | 3,069 0.05 3,837 0.04 | 2,132 1,566 0.07 0.10 3,197 2,741 0.06 0.08 |  | 1,199 0.13 2,398 0.11 | 947 0.17 2,132 0.13 | 767 0.21 1,918 0.17 | 634 0.25 1,744 0.20 | 533 0.30 1,599 0.24 | 454 0.35 1,476 0.28 | 392 0.41 1,370 0.32 | 341 0.47 1,279 0.37 | 300 0.53 1,199 0.42 | 1.599 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.799 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

19W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 1-3/8 | 2-9/16 | 3-3/4 | 4-15/16 | 6-1/8 | 7-5/16 | 8-1/2 | 9-11/16 | 10-7/8 | 12-1/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 13-1/4 | 14-7/16 | 15-5/8 | 16-13/16 | 18 | 19-3/16 | 20-3/8 | 21-9/16 | 22-3/4 | 23-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 25-1/8 | 26-5/16 | 27-1/2 | 28-11/16 | 29-7/8 | 31-1/16 | 32-1/4 | 33-7/16 | 34-5/8 | 35-13/16 |

Deduct 1/16” for 1/8” bearing bars

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              29

LOAD TABLES | STANDARD DUTY, IMPERIAL

LOAD TABLES - CLOSE-MESH                                                                                                  LOAD TABLES - CLOSE-MESH

| Grating Type: 15W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ⅛" | 6.15 | 55 | U Du C Dc | 800 0.07 800 0.06 | 512 0.12 640 0.09 | 356 0.17 533 0.13 | 261 0.23 457 0.18 | 200 0.30 400 0.24 | 158 0.38 356 0.30 | 128 0.47 320 0.37 |  |  |  |  |  |  | 0.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.133 |
| 1" x ³/16" | 8.87 | 60 | U Du C Dc | 1,200 0.07 1,200 0.06 | 768 0.12 960 0.09 | 533 0.17 800 0.13 | 392 0.23 686 0.18 | 300 0.30 600 0.24 | 237 0.38 533 0.30 | 192 0.47 480 0.37 |  |  |  |  |  |  | 0.400 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.200 |
| 1 ¼" x ⅛" | 7.51 | 65 | U Du C Dc | 1,250 0.06 1,250 0.05 | 800 0.09 1,000 0.07 | 556 0.13 833 0.11 | 408 0.18 714 0.15 | 313 0.24 625 0.19 | 247 200 0.30 0.37 556 500 0.24 0.30 |  | 165 0.45 455 0.36 |  |  |  |  |  | 0.417 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| 1 ¼" x ³/16" | 10.91 | 71 | U Du C Dc | 1,875 0.06 1,875 0.05 | 1,200 0.09 1,500 0.07 | 833 0.13 1,250 0.11 | 612 0.18 1,071 0.15 | 469 0.24 938 0.19 | 370 300 0.30 0.37 833 750 0.24 0.30 |  | 248 0.45 682 0.36 | 208 0.54 625 0.43 |  |  |  |  | 0.625 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.391 |
| 1 ½" x ⅛" | 8.87 | 74 | U Du C Dc | 1,800 0.05 1,800 0.04 | 1,152 0.08 1,440 0.06 | 800 0.11 1,200 0.09 | 588 0.15 1,029 0.12 | 450 0.20 900 0.16 | 356 288 0.25 0.31 800 720 0.20 0.25 |  | 238 0.38 655 0.30 | 200 0.45 600 0.36 | 170 0.52 554 0.42 |  |  |  | 0.600 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.450 |
| 1 ½" x ³/16" | 12.95 | 82 | U Du C Dc | 2,700 0.05 2,700 0.04 | 1,728 0.08 2,160 0.06 | 1,200 0.11 1,800 0.09 | 882 0.15 1,543 0.12 | 675 0.20 1,350 0.16 | 533 432 0.25 0.31 1,200 1,080 0.20 0.25 |  | 357 0.38 982 0.30 | 300 0.45 900 0.36 | 256 0.52 831 0.42 | 220 0.61 771 0.49 |  |  | 0.900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.675 |
| 1 ¾" x ³/16" | 15.18 | 92 | U Du C Dc | 3,675 0.04 3,675 0.03 | 2,352 0.07 2,940 0.05 | 1,633 0.10 2,450 0.08 | 1,200 0.13 2,100 0.10 | 919 0.17 1,838 0.14 | 726 588 0.22 0.27 1,633 1,470 0.17 0.21 |  | 486 0.32 1,336 0.26 | 408 0.38 1,225 0.31 | 348 0.45 1,131 0.36 | 300 0.52 1,050 0.42 | 261 0.60 980 0.48 | 230 0.68 919 0.54 | 1.225 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.072 |
| 2" x ³/16" | 17.22 | 102 | U Du C Dc | 4,800 0.04 4,800 0.03 | 3,072 0.06 3,840 0.05 | 2,133 0.08 3,200 0.07 | 1,567 0.11 2,743 0.09 | 1,200 0.15 2,400 0.12 | 948 768 0.19 0.23 2,133 1,920 0.15 0.19 |  | 635 0.28 1,745 0.23 | 533 0.34 1,600 0.27 | 454 0.39 1,477 0.31 | 392 0.46 1,371 0.36 | 341 0.52 1,280 0.42 | 300 0.60 1,200 0.48 | 1.600 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.600 |
| 2 ½" x ³/16" | 21.30 | 120 | U Du C Dc | 7,500 0.03 7,500 0.02 | 4,800 0.05 6,000 0.04 | 3,333 0.07 5,000 0.05 | 2,449 0.09 4,286 0.07 | 1,875 0.12 3,750 0.10 | 1,481 1,200 0.15 0.19 3,333 3,000 0.12 0.15 |  | 992 0.23 2,727 0.18 | 833 0.27 2,500 0.21 | 710 0.31 2,308 0.25 | 612 0.36 2,143 0.29 | 533 0.42 2,000 0.34 | 469 0.48 1,875 0.38 | 2.500 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.125 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 15W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 1-1/8 | 2-1/16 | 3 | 3-15/16 | 4-7/8 | 5-13/16 | 6-3/4 | 7-11/16 | 8-5/8 | 9-9/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 10-1/2 | 11-7/16 | 12-3/8 | 13-5/16 | 14-1/4 | 15-3/16 | 16-1/8 | 17-1/16 | 18 | 18-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 19-7/8 | 20-13/16 | 21-3/4 | 22-11/16 | 23-5/8 | 24-9/16 | 25-1/2 | 26-7/16 | 27-3/8 | 28-5/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 29-1/4 | 30-3/16 | 31-1/8 | 32-1/16 | 33 | 33-15/16 | 34-7/8 | 35-13/16 | 36-3/4 | 37-11/16 |

Deduct 1/16” for 1/8” bearing bars

LOAD TABLES - SD IMPERIAL

| Grating Type: 15W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ⅛" | 6.15 | 44 | U Du C Dc | 450 0.10 450 0.08 | 288 0.16 360 0.12 | 200 0.22 300 0.18 | 147 0.30 257 0.24 | 113 0.40 225 0.32 |  |  |  |  |  |  |  |  | 0.150 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.056 |
| 1" x ³/16" | 8.87 | 49 | U Du C Dc | 675 0.10 675 0.08 | 432 0.16 540 0.12 | 300 0.22 450 0.18 | 220 169 0.30 0.40 386 338 0.24 0.32 |  | 133 0.50 300 0.40 |  |  |  |  |  |  |  | 0.225 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.084 |
| 1 ¼" x ⅛" | 7.51 | 55 | U Du C Dc | 800 0.07 800 0.06 | 512 0.12 640 0.09 | 356 0.17 533 0.13 | 261 200 0.23 0.30 457 400 0.18 0.24 |  | 158 0.38 356 0.30 | 128 0.47 320 0.37 |  |  |  |  |  |  | 0.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.133 |
| 1 ¼" x ³/16" | 10.91 | 60 | U Du C Dc | 1,200 0.07 1,200 0.06 | 768 0.12 960 0.09 | 533 0.17 800 0.13 | 392 300 0.23 0.30 686 600 0.18 0.24 |  | 237 0.38 533 0.30 | 192 0.47 480 0.37 |  |  |  |  |  |  | 0.400 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.200 |
| 1 ½" x ⅛" | 8.87 | 65 | U Du C Dc | 1,250 0.06 1,250 0.05 | 800 0.09 1,000 0.07 | 556 0.13 833 0.11 | 408 313 0.18 0.24 714 625 0.15 0.19 |  | 247 0.30 556 0.24 | 200 0.37 500 0.30 | 165 0.45 455 0.36 |  |  |  |  |  | 0.417 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| 1 ½" x ³/16" | 12.95 | 71 | U Du C Dc | 1,875 0.06 1,875 0.05 | 1,200 0.09 1,500 0.07 | 833 0.13 1,250 0.11 | 612 469 0.18 0.24 1,071 938 0.15 0.19 |  | 370 0.30 833 0.24 | 300 0.37 750 0.30 | 248 0.45 682 0.36 | 208 0.54 625 0.43 |  |  |  |  | 0.625 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.391 |
| 1 ¾" x ³/16" | 14.99 | 82 | U Du C Dc | 2,700 0.05 2,700 0.04 | 1,728 0.08 2,160 0.06 | 1,200 0.11 1,800 0.09 | 882 675 0.15 0.20 1,543 1,350 0.12 0.16 |  | 533 0.25 1,200 0.20 | 432 0.31 1,080 0.25 | 357 0.38 982 0.30 | 300 0.45 900 0.36 | 256 0.52 831 0.42 | 220 0.61 771 0.49 |  |  | 0.900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.675 |
| 2" x ³/16" | 17.22 | 92 | U Du C Dc | 3,675 0.04 3,675 0.03 | 2,352 0.07 2,940 0.05 | 1,633 0.10 2,450 0.08 | 1,200 919 0.13 0.17 2,100 1,838 0.10 0.14 |  | 726 0.22 1,633 0.17 | 588 0.27 1,470 0.21 | 486 0.32 1,336 0.26 | 408 0.38 1,225 0.31 | 348 0.45 1,131 0.36 | 300 0.52 1,050 0.42 | 261 0.60 980 0.48 | 230 0.68 919 0.54 | 1.225 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.072 |
| 2 ½" x ³/16" | 21.30 | 111 | U Du C Dc | 6,075 0.03 6,075 0.03 | 3,888 0.05 4,860 0.04 | 2,700 0.07 4,050 0.06 | 1,984 1,519 0.10 0.13 3,471 3,038 0.08 0.11 |  | 1,200 0.17 2,700 0.13 | 972 0.21 2,430 0.17 | 803 0.25 2,209 0.20 | 675 0.30 2,025 0.24 | 575 0.35 1,869 0.28 | 496 0.41 1,736 0.32 | 432 0.47 1,620 0.37 | 380 0.53 1,519 0.42 | 2.025 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.278 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

15W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 1-1/8 | 2-1/16 | 3 | 3-15/16 | 4-7/8 | 5-13/16 | 6-3/4 | 7-11/16 | 8-5/8 | 9-9/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 10-1/2 | 11-7/16 | 12-3/8 | 13-5/16 | 14-1/4 | 15-3/16 | 16-1/8 | 17-1/16 | 18 | 18-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 19-7/8 | 20-13/16 | 21-3/4 | 22-11/16 | 23-5/8 | 24-9/16 | 25-1/2 | 26-7/16 | 27-3/8 | 28-5/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 29-1/4 | 30-3/16 | 31-1/8 | 32-1/16 | 33 | 33-15/16 | 34-7/8 | 35-13/16 | 36-3/4 | 37-11/16 |

Deduct 1/16” for 1/8” bearing bars

30                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               31

LOAD TABLES | STANDARD DUTY, IMPERIAL

LOAD TABLES - ADA                                                                                                                 LOAD TABLES - ADA

| Grating Type: 11W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ³/16" | 11.84 | 65 | U Du C Dc | 1,636 0.07 1,636 0.06 | 1,047 0.12 1,309 0.09 | 727 0.17 1,091 0.13 | 534 0.23 935 0.18 | 409 0.30 818 0.24 | 323 0.38 727 0.30 | 262 0.47 655 0.37 | 216 0.56 595 0.45 |  |  |  |  |  | 0.545 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.273 |
| 1 ¼" x ³/16" | 14.62 | 77 | U Du C Dc | 2,557 0.06 2,557 0.05 | 1,636 0.09 2,045 0.07 | 1,136 0.13 1,705 0.11 | 835 0.18 1,461 0.15 | 639 0.24 1,278 0.19 | 505 0.30 1,136 0.24 | 409 338 0.37 0.45 1,023 930 0.30 0.36 |  | 284 0.54 852 0.43 | 242 0.63 787 0.50 |  |  |  | 0.852 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.533 |
| 1 ½" x ³/16" | 17.40 | 89 | U Du C Dc | 3,682 0.05 3,682 0.04 | 2,356 0.08 2,945 0.06 | 1,636 0.11 2,455 0.09 | 1,202 0.15 2,104 0.12 | 920 0.20 1,841 0.16 | 727 0.25 1,636 0.20 | 589 487 0.31 0.38 1,473 1,339 0.25 0.30 |  | 409 0.45 1,227 0.36 | 349 0.52 1,133 0.42 | 301 0.61 1,052 0.49 | 262 0.70 982 0.56 |  | 1.227 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.920 |
| 1 ¾" x ³/16" | 20.37 | 99 | U Du C Dc | 5,011 0.04 5,011 0.03 | 3,207 0.07 4,009 0.05 | 2,227 0.10 3,341 0.08 | 1,636 0.13 2,864 0.10 | 1,253 0.17 2,506 0.14 | 990 0.22 2,227 0.17 | 802 663 0.27 0.32 2,005 1,822 0.21 0.26 |  | 557 0.38 1,670 0.31 | 474 0.45 1,542 0.36 | 409 0.52 1,432 0.42 | 356 0.60 1,336 0.48 | 313 0.68 1,253 0.54 | 1.670 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.462 |
| 2" x ³/16" | 23.15 | 110 | U Du C Dc | 6,545 0.04 6,545 0.03 | 4,189 0.06 5,236 0.05 | 2,909 0.08 4,364 0.07 | 2,137 0.11 3,740 0.09 | 1,636 0.15 3,273 0.12 | 1,293 0.19 2,909 0.15 | 1,047 866 0.23 0.28 2,618 2,380 0.19 0.23 |  | 727 0.34 2,182 0.27 | 620 0.39 2,014 0.31 | 534 0.46 1,870 0.36 | 465 0.52 1,745 0.42 | 409 0.60 1,636 0.48 | 2.182 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.182 |
| 2 ½" x ³/16" | 28.71 | 130 | U Du C Dc | 10,227 0.03 10,227 0.02 | 6,545 0.05 8,182 0.04 | 4,545 0.07 6,818 0.05 | 3,340 0.09 5,844 0.07 | 2,557 0.12 5,114 0.10 | 2,020 0.15 4,545 0.12 | 1,636 1,352 0.19 0.23 4,091 3,719 0.15 0.18 |  | 1,136 0.27 3,409 0.21 | 968 0.31 3,147 0.25 | 835 0.36 2,922 0.29 | 727 0.42 2,727 0.34 | 639 0.48 2,557 0.38 | 3.409 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.261 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 11W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 7/8 | 1-9/16 | 2-1/4 | 2-15/16 | 3-5/8 | 4-5/16 | 5 | 5-11/16 | 6-3/8 | 7-1/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 7-3/4 | 8-7/16 | 9-1/8 | 9-13/16 | 10-1/2 | 11-3/16 | 11-7/8 | 12-9/16 | 13-1/4 | 13-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 14-5/8 | 15-5/16 | 16 | 16-11/16 | 17-3/8 | 18-1/16 | 18-3/4 | 19-7/16 | 20-1/8 | 20-13/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 21-1/2 | 22-3/16 | 22-7/8 | 23-9/16 | 24-1/4 | 24-15/16 | 24-15/16 | 26-5/16 | 27 | 27-11/16 |

LOAD TABLES - SD IMPERIAL

| Grating Type: 11W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ³/16" | 11.84 | 53 | U Du C Dc | 920 0.10 920 0.08 | 589 0.16 736 0.12 | 409 0.22 614 0.18 | 301 0.30 526 0.24 | 230 0.40 460 0.32 | 182 0.50 409 0.40 |  |  |  |  |  |  |  | 0.307 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.115 |
| 1 ¼" x ³/16" | 14.62 | 65 | U Du C Dc | 1,636 0.07 1,636 0.06 | 1,047 0.12 1,309 0.09 | 727 0.17 1,091 0.13 | 534 0.23 935 0.18 | 409 323 0.30 0.38 818 727 0.24 0.30 |  | 262 0.47 655 0.37 | 216 0.56 595 0.45 |  |  |  |  |  | 0.545 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.273 |
| 1 ½" x ³/16" | 17.40 | 77 | U Du C Dc | 2,557 0.06 2,557 0.05 | 1,636 0.09 2,045 0.07 | 1,136 0.13 1,705 0.11 | 835 0.18 1,461 0.15 | 639 505 0.24 0.30 1,278 1,136 0.19 0.24 |  | 409 0.37 1,023 0.30 | 338 0.45 930 0.36 | 284 0.54 852 0.43 | 242 0.63 787 0.50 |  |  |  | 0.852 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.533 |
| 1 ¾" x ³/16" | 20.18 | 89 | U Du C Dc | 3,682 0.05 3,682 0.04 | 2,356 0.08 2,945 0.06 | 1,636 0.11 2,455 0.09 | 1,202 0.15 2,104 0.12 | 920 727 0.20 0.25 1,841 1,636 0.16 0.20 |  | 589 0.31 1,473 0.25 | 487 0.38 1,339 0.30 | 409 0.45 1,227 0.36 | 349 0.52 1,133 0.42 | 301 0.61 1,052 0.49 | 262 0.70 982 0.56 |  | 1.227 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.920 |
| 2" x ³/16" | 23.15 | 99 | U Du C Dc | 5,011 0.04 5,011 0.03 | 3,207 0.07 4,009 0.05 | 2,227 0.10 3,341 0.08 | 1,636 0.13 2,864 0.10 | 1,253 990 0.17 0.22 2,506 2,227 0.14 0.17 |  | 802 0.27 2,005 0.21 | 663 0.32 1,822 0.26 | 557 0.38 1,670 0.31 | 474 0.45 1,542 0.36 | 409 0.52 1,432 0.42 | 356 0.60 1,336 0.48 | 313 0.68 1,253 0.54 | 1.670 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.462 |
| 2 ½" x ³/16" | 28.71 | 120 | U Du C Dc | 8,284 0.03 8,284 0.03 | 5,302 0.05 6,627 0.04 | 3,682 0.07 5,523 0.06 | 2,705 0.10 4,734 0.08 | 2,071 1,636 0.13 0.17 4,142 3,682 0.11 0.13 |  | 1,325 0.21 3,314 0.17 | 1,095 0.25 3,012 0.20 | 920 0.30 2,761 0.24 | 784 0.35 2,549 0.28 | 676 0.41 2,367 0.32 | 589 0.47 2,209 0.37 | 518 0.53 2,071 0.42 | 2.761 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.107 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

11W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 7/8 | 1-9/16 | 2-1/4 | 2-15/16 | 3-5/8 | 4-5/16 | 5 | 5-11/16 | 6-3/8 | 7-1/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 7-3/4 | 8-7/16 | 9-1/8 | 9-13/16 | 10-1/2 | 11-3/16 | 11-7/8 | 12-9/16 | 13-1/4 | 13-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 14-5/8 | 15-5/16 | 16 | 16-11/16 | 17-3/8 | 18-1/16 | 18-3/4 | 19-7/16 | 20-1/8 | 20-13/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 21-1/2 | 22-3/16 | 22-7/8 | 23-9/16 | 24-1/4 | 24-15/16 | 24-15/16 | 26-5/16 | 27 | 27-11/16 |

32                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               33

LOAD TABLES | STANDARD DUTY, IMPERIAL

LOAD TABLES - HEELS FRIENDLY                                                                                            LOAD TABLES - HEELS FRIENDLY

| Grating Type: 7W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ³/16" | 18.19 | 73 | U Du C Dc | 2,571 0.07 2,571 0.06 | 1,646 0.12 2,057 0.09 | 1,143 0.17 1,714 0.13 | 840 0.23 1,469 0.18 | 643 0.30 1,286 0.24 | 508 0.38 1,143 0.30 | 411 0.47 1,029 0.37 | 340 0.56 935 0.45 | 286 0.67 857 0.54 | 243 0.79 791 0.63 |  |  |  | 0.857 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.429 |
| 1 ¼" x ³/16" | 22.57 | 86 | U Du C Dc | 4,018 0.06 4,018 0.05 | 2,571 0.09 3,214 0.07 | 1,786 0.13 2,679 0.11 | 1,312 0.18 2,296 0.15 | 1,004 0.24 2,009 0.19 | ### 794 0.30 1,786 0.24 | 643 0.37 1,607 0.30 | 531 0.45 1,461 0.36 | 446 380 0.54 0.63 1,339 1,236 0.43 0.50 |  | 328 0.73 1,148 0.58 | 286 0.84 1,071 0.67 |  | 1.339 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.837 |
| 1 ½" x ³/16" | 26.94 | 99 | U Du C Dc | 5,786 0.05 5,786 0.04 | 3,703 0.08 4,629 0.06 | 2,571 0.11 3,857 0.09 | 1,889 0.15 3,306 0.12 | 1,446 0.20 2,893 0.16 | 1,143 0.25 2,571 0.20 | 926 0.31 2,314 0.25 | 765 0.38 2,104 0.30 | 643 548 0.45 0.52 1,929 1,780 0.36 0.42 |  | 472 0.61 1,653 0.49 | 411 0.70 1,543 0.56 | 362 0.79 1,446 0.64 | 1.929 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.446 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

7W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 5/8 | 1-1/16 | 1-1/2 | 1-15/16 | 2-3/8 | 2-13/16 | 3-1/4 | 3-11/16 | 4-1/8 | 4-9/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 5 | 5-7/16 | 5-7/8 | 6-5/16 | 6-3/4 | 7-3/16 | 7-5/8 | 8-1/16 | 8-1/2 | 8-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 9-3/8 | 9-13/16 | 10-1/4 | 10-11/16 | 11-1/8 | 11-9/16 | 12 | 12-7/16 | 12-7/8 | 13-5/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 13-3/4 | 14-3/16 | 14-5/8 | 15-1/16 | 15-1/2 | 15-15/16 | 16-3/8 | 16-13/16 | 17-1/4 | 17-11/16 |
| # of Bars | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| 3/16” Bars | 18-1/8 | 18-9/16 | 19 | 19-7/16 | 19-7/8 | 20-5/16 | 20-3/4 | 21-3/16 | 21-5/8 | 22-1/16 |
| # of Bars | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 |
| 3/16” Bars | 22-1/2 | 22-15/16 | 23-3/8 | 23-13/16 | 24-1/4 | 24-11/16 | 25-1/8 | 25-9/16 | 26 | 26-7/16 |
| # of Bars | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| 3/16” Bars | 26-7/8 | 27-5/16 | 27-3/4 | 28-3/16 | 28-5/8 | 29-1/16 | 29-1/2 | 29-15/16 | 30-3/8 | 30-13/16 |

| Grating Type: 7W4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 18 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ³/16" | 18.19 | 59 | U Du C Dc | 1,446 0.10 1,446 0.08 | 926 0.16 1,157 0.12 | 643 0.22 964 0.18 | 472 0.30 827 0.24 | 362 0.40 723 0.32 | 286 0.50 643 0.40 | 231 0.62 579 0.50 |  |  |  |  |  |  | 0.482 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.181 |
| 1 ¼" x ³/16" | 22.57 | 73 | U Du C Dc | 2,571 0.07 2,571 0.06 | 1,646 0.12 2,057 0.09 | 1,143 0.17 1,714 0.13 | 840 0.23 1,469 0.18 | 643 0.30 1,286 0.24 | 508 411 0.38 0.47 1,143 1,029 0.30 0.37 |  | 340 0.56 935 0.45 | 286 0.67 857 0.54 | 243 0.79 791 0.63 |  |  |  | 0.857 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.429 |
| 1 ½" x ³/16" | 26.94 | 86 | U Du C Dc | 4,018 0.06 4,018 0.05 | 2,571 0.09 3,214 0.07 | 1,786 0.13 2,679 0.11 | 1,312 0.18 2,296 0.15 | 1,004 0.24 2,009 0.19 | 794 643 0.30 0.37 1,786 1,607 0.24 0.30 |  | 531 0.45 1,461 0.36 | 446 0.54 1,339 0.43 | 380 0.63 1,236 0.50 | 328 0.73 1,148 0.58 | 286 0.84 1,071 0.67 |  | 1.339 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.837 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

LOAD TABLES - SD IMPERIAL

7W4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/16” Bars | 5/8 | 1-1/16 | 1-1/2 | 1-15/16 | 2-3/8 | 2-13/16 | 3-1/4 | 3-11/16 | 4-1/8 | 4-9/16 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 3/16” Bars | 5 | 5-7/16 | 5-7/8 | 6-5/16 | 6-3/4 | 7-3/16 | 7-5/8 | 8-1/16 | 8-1/2 | 8-15/16 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 3/16” Bars | 9-3/8 | 9-13/16 | 10-1/4 | 10-11/16 | 11-1/8 | 11-9/16 | 12 | 12-7/16 | 12-7/8 | 13-5/16 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 3/16” Bars | 13-3/4 | 14-3/16 | 14-5/8 | 15-1/16 | 15-1/2 | 15-15/16 | 16-3/8 | 16-13/16 | 17-1/4 | 17-11/16 |
| # of Bars | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| 3/16” Bars | 18-1/8 | 18-9/16 | 19 | 19-7/16 | 19-7/8 | 20-5/16 | 20-3/4 | 21-3/16 | 21-5/8 | 22-1/16 |
| # of Bars | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 |
| 3/16” Bars | 22-1/2 | 22-15/16 | 23-3/8 | 23-13/16 | 24-1/4 | 24-11/16 | 25-1/8 | 25-9/16 | 26 | 26-7/16 |
| # of Bars | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| 3/16” Bars | 26-7/8 | 27-5/16 | 27-3/4 | 28-3/16 | 28-5/8 | 29-1/16 | 29-1/2 | 29-15/16 | 30-3/8 | 30-13/16 |

34                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              35

## LOAD TABLES | STANDARD DUTY, METRIC

OVERVIEW

Standard

Standard  Duty Grating  is the  most common type of grating  used  in  the  industrial  flooring  market. The open grid construction provides for maximum passage for light, air circulation and drainage.

Close-Mesh

When a certain bar depth must be held but standard duty is not sufficient, Close Mesh moves the bars closer to gain more strength and stiffness. This may also be warranted if the bar gap on Standard Duty is wider than desired.

ADA-Compliant

When the Grating needs to adhere to the guidelines of  the  Americans-with-Disabilities  Act  (ADA),  an even-smaller bar gap is required.  The ADA requires no more than a 13mm opening as part of Chapter 3, Section 302.3 of the ADA Guide published by the Unites States Access Board.

Heels-Friendly

Also ADA-Compliant, Heel-Friendly grating provides an  even tighter  bar  spacing  to  ensure that  high heels, canes, and other narrow items will be difficult to get wedged in the gaps.

DESIGN CRITERIA

LOAD TABLES - SD METRIC

|  | Uniform Load | Concentrated Load |
| --- | --- | --- |
| Determine M: | M = | M = |
| Determine U or C: | 8M U= L2 | 4M C= L |
| Check D*: | 5UL*L3 D = 384 EI | C*L3 D = 48 EI |

*Deflection should be limited to 7mm under .45kN uniform load to afford pedestrian comfort.

U =   Uniform Load - kPa

C =   Concentrated Load - kN/m of grating width S =   Section Modulus - mm3/m of grating width	I =   Moment of Inertia - mm4/m of grating width L =   Simple Clear Span - m

D =   Deflection - mm

E =   Modulus of Elasticity - kPa

tip

F =   Allowable Bending Stress - kPa M =   Bending Moment - kN-m

Vulcraft Online Bar Grating Design Tools

If you are uncertain about which type of grating to use, try our on-line calculator. Enter your project specs

and our calculator will recommend the appropriate grating. This powerful on-line tool allows you to compare alternatives so you can optimize weight, cost, or both. It saves you valuable time by eliminating manual

calculations and the risk of errors. Visit www.vulcraft.com/design-tools/bar-grating-design-aid

36                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               37

LOAD TABLES | STANDARD DUTY, METRIC

LOAD TABLES - SD

| Grating Type: 30W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 3 | 24.1 | 1,312.00 | U Du C Dc | 30.6 1.9 9.3 1.5 | 19.6 3.0 7.5 2.4 | 13.6 4.3 6.2 3.4 | 10.0 5.8 5.3 4.6 | 7.7 7.6 4.7 6.1 | 6.0 9.6 4.2 7.7 |  |  |  |  |  |  |  | 11,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 145.66E+3 |
| 25 x 5 | 34.7 | 1,452.00 | U Du C Dc | 45.9 1.9 14.0 1.5 | 29.4 3.0 11.2 2.4 | 20.4 4.3 9.3 3.4 | 15.0 5.8 8.0 4.6 | 11.5 9.1 7.6 9.6 7.0 6.2 6.1 7.7 |  | 7.4 11.8 5.6 9.5 |  |  |  |  |  |  | 17,200 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 218.49E+3 |
| 32 x 3 | 29.5 | 1,552.00 | U Du C Dc | 48.0 1.5 14.6 1.2 | 30.7 2.4 11.7 1.9 | 21.3 3.4 9.8 2.7 | 15.7 4.6 8.4 3.7 | 12.0 9.5 6.0 7.7 7.3 6.5 4.8 6.1 |  | 7.7 9.4 5.9 7.6 | 6.3 11.4 5.3 9.1 |  |  |  |  |  | 17,980 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 285.84E+3 |
| 32 x 5 | 42.8 | 1,718.00 | U Du C Dc | 72.0 1.5 21.9 1.2 | 46.1 2.4 17.6 1.9 | 32.0 3.4 14.6 2.7 | 23.5 4.6 12.5 3.7 | 18.0 14.2 6.0 7.7 11.0 9.8 4.8 6.1 |  | 11.5 9.4 8.8 7.6 | 9.5 11.4 8.0 9.1 | 8.0 13.6 7.3 10.9 |  |  |  |  | 26,970 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 428.77E+3 |
| 38 x 3 | 34.3 | 1,778.00 | U Du C Dc | 68.9 1.3 21.0 1.0 | 44.1 2.0 16.8 1.6 | 30.6 2.8 14.0 2.3 | 22.5 3.9 12.0 3.1 | 17.2 13.6 5.0 6.4 10.5 9.3 4.0 5.1 |  | 11.0 7.9 8.4 6.3 | 9.1 9.5 7.6 7.6 | 7.7 11.4 7.0 9.1 |  |  |  |  | 25,810 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 491.61E+3 |
| 38 x 5 | 51.5 | 1,967.00 | U Du C Dc | 103.3 1.3 31.5 1.0 | 66.1 2.0 25.2 1.6 | 45.9 2.8 21.0 2.3 | 33.7 3.9 18.0 3.1 | 25.8 20.4 5.0 6.4 15.8 14.0 4.0 5.1 |  | 16.5 7.9 12.6 6.3 | 13.7 9.5 11.5 7.6 | 11.5 11.4 10.5 9.1 | 9.8 13.3 9.7 10.7 |  |  |  | 38,710 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 737.42E+3 |
| 45 x 5 | 59.5 | 2,210.00 | U Du C Dc | 140.9 1.1 43.0 0.9 | 90.2 1.7 34.4 1.4 | 62.7 2.4 28.7 1.9 | 46.0 3.3 24.6 2.6 | 35.2 27.9 4.3 5.5 21.5 19.1 3.5 4.4 |  | 22.6 6.8 17.2 5.4 | 18.6 8.2 15.6 6.5 | 15.7 9.7 14.3 7.8 | 13.4 11.4 13.2 9.1 | 11.5 13.2 12.3 10.6 | 10.0 15.2 11.5 12.1 |  | 52,810 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.17E+6 |
| 51 x 5 | 67.4 | 2,441.00 | U Du C Dc | 183.6 0.9 56.0 0.8 | 117.6 1.5 44.8 1.2 | 81.6 2.1 37.3 1.7 | 60.0 2.9 32.0 2.3 | 45.9 36.3 3.8 4.8 28.0 24.9 3.0 3.8 |  | 29.4 5.9 22.4 4.7 | 24.3 7.2 20.4 5.7 | 20.4 8.5 18.7 6.8 | 17.4 10.0 17.2 8.0 | 15.0 11.6 16.0 9.3 | 13.1 13.3 14.9 10.6 | 11.5 15.1 14.0 12.1 | 68,820 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.75E+6 |
| 64 x 5 | 83.4 | 2,886.00 | U Du C Dc | 286.9 0.8 87.5 0.6 | 183.7 1.2 70.0 0.9 | 127.6 1.7 58.4 1.4 | 93.7 2.3 50.0 1.9 | 71.8 56.7 3.0 3.8 43.8 38.9 2.4 3.1 |  | 45.9 4.7 35.0 3.8 | 38.0 5.7 31.8 4.6 | 31.9 6.8 29.2 5.4 | 27.2 8.0 26.9 6.4 | 23.4 9.3 25.0 7.4 | 20.4 10.6 23.3 8.5 | 18.0 12.1 21.9 9.7 | 107,530 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.41E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 30W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 35 | 65 | 95 | 125 | 155 | 185 | 215 | 245 | 275 | 305 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 335 | 365 | 395 | 425 | 455 | 485 | 515 | 545 | 575 | 605 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 635 | 665 | 695 | 725 | 755 | 785 | 815 | 845 | 875 | 905 |

Deduct 1.5mm for 3mm bearing bars

LOAD TABLES - SD METRIC

LOAD TABLES - SD

| Grating Type: 30W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 3 | 24.1 | 1,072.00 | U Du C Dc | 17.9 2.5 5.4 2.0 | 11.4 3.9 4.4 3.1 | 7.9 5.6 3.6 4.5 | 5.8 7.6 3.1 6.1 | 4.5 9.9 2.7 7.9 |  |  |  |  |  |  |  |  | 6,690 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 64.90E+3 |
| 25 x 5 | 34.7 | 1,186.00 | U Du C Dc | 26.8 2.5 8.2 2.0 | 17.1 3.9 6.5 3.1 | 11.9 5.6 5.4 4.5 | 8.7 7.6 4.7 6.1 | 6.7 9.9 4.1 7.9 |  |  |  |  |  |  |  |  | 10,040 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 97.35E+3 |
| 32 x 3 | 29.5 | 1,327.00 | U Du C Dc | 31.6 1.9 9.6 1.5 | 20.2 2.9 7.7 2.3 | 14.0 4.2 6.4 3.4 | 10.3 7.9 5.7 7.5 5.5 4.8 4.6 6.0 |  | 6.2 9.4 4.3 7.5 |  |  |  |  |  |  |  | 11,830 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 152.65E+3 |
| 32 x 5 | 42.8 | 1,469.00 | U Du C Dc | 47.4 1.9 14.4 1.5 | 30.3 2.9 11.6 2.3 | 21.1 4.2 9.6 3.4 | 15.5 11.8 5.7 7.5 8.3 7.2 4.6 6.0 |  | 9.4 9.4 6.4 7.5 | 7.6 11.6 5.8 9.3 |  |  |  |  |  |  | 17,750 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 228.98E+3 |
| 38 x 3 | 34.7 | 1,563.00 | U Du C Dc | 48.9 1.5 14.9 1.2 | 31.3 2.3 11.9 1.9 | 21.7 3.4 9.9 2.7 | 16.0 12.2 4.6 6.0 8.5 7.5 3.7 4.8 |  | 9.7 7.6 6.6 6.1 | 7.8 9.4 6.0 7.5 | 6.5 11.3 5.4 9.1 |  |  |  |  |  | 18,320 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 294.01E+3 |
| 38 x 5 | 50.7 | 1,730.00 | U Du C Dc | 73.3 1.5 22.4 1.2 | 46.9 2.3 17.9 1.9 | 32.6 3.4 14.9 2.7 | 24.0 18.3 4.6 6.0 12.8 11.2 3.7 4.8 |  | 14.5 7.6 9.9 6.1 | 11.7 9.4 8.9 7.5 | 9.7 11.3 8.1 9.1 | 8.2 13.5 7.5 10.8 |  |  |  |  | 27,480 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 441.02E+3 |
| 45 x 5 | 59.5 | 1,983.00 | U Du C Dc | 105.5 1.2 32.2 1.0 | 67.5 2.0 25.7 1.6 | 46.9 2.8 21.5 2.2 | 34.5 26.4 3.8 5.0 18.4 16.1 3.1 4.0 |  | 20.8 6.3 14.3 5.1 | 16.9 7.8 12.9 6.2 | 14.0 9.4 11.7 7.6 | 11.7 11.2 10.7 9.0 | 10.0 13.2 9.9 10.5 | 8.6 15.3 9.2 12.2 |  |  | 39,530 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 760.89E+3 |
| 51 x 5 | 67.4 | 2,222.00 | U Du C Dc | 142.8 1.1 43.6 0.9 | 91.4 1.7 34.9 1.3 | 63.5 2.4 29.0 1.9 | 46.7 35.7 3.3 4.3 24.9 21.8 2.6 3.4 |  | 28.2 5.4 19.4 4.3 | 22.9 6.7 17.4 5.4 | 18.9 8.1 15.8 6.5 | 15.9 9.7 14.5 7.7 | 13.5 11.3 13.4 9.1 | 11.7 13.1 12.5 10.5 | 10.2 15.1 11.6 12.1 |  | 53,520 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.20E+6 |
| 64 x 5 | 83.4 | 2,679.00 | U Du C Dc | 235.3 0.8 71.8 0.7 | 150.6 1.3 57.4 1.0 | 104.6 1.9 47.8 1.5 | 76.9 58.9 2.6 3.3 41.0 35.9 2.0 2.7 |  | 46.5 4.2 31.9 3.4 | 37.7 5.2 28.7 4.2 | 31.1 6.3 26.1 5.1 | 26.2 7.5 23.9 6.0 | 22.3 8.8 22.1 7.1 | 19.2 10.2 20.5 8.2 | 16.7 11.8 19.1 9.4 | 14.7 13.4 18.0 10.7 | 88,170 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.53E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

30W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 35 | 65 | 95 | 125 | 155 | 185 | 215 | 245 | 275 | 305 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 335 | 365 | 395 | 425 | 455 | 485 | 515 | 545 | 575 | 605 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 635 | 665 | 695 | 725 | 755 | 785 | 815 | 845 | 875 | 905 |

Deduct 1.5mm for 3mm bearing bars

38                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              39

LOAD TABLES | STANDARD DUTY, METRIC

LOAD TABLES - CLOSE-MESH                                                                                                  LOAD TABLES - CLOSE-MESH

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 3 | 29.4 | 1,387.00 | U Du C Dc | 38.3 1.9 11.7 1.5 | 24.5 3.0 9.3 2.4 | 17.0 4.3 7.8 3.4 | 12.5 5.8 6.7 4.6 | 9.6 7.6 5.8 6.1 | 7.6 9.6 5.2 7.7 | 6.1 11.8 4.7 9.5 |  |  |  |  |  |  | 14,340 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 182.08E+3 |
| 25 x 5 | 42.7 | 1,535.00 | U Du C Dc | 57.4 1.9 17.5 1.5 | 36.7 3.0 14.0 2.4 | 25.5 4.3 11.7 3.4 | 18.7 5.8 10.0 4.6 | 14.4 7.6 8.8 6.1 | 11.3 9.2 9.6 11.8 7.8 7.0 7.7 9.5 |  | 7.6 14.3 6.4 11.4 |  |  |  |  |  | 21,510 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 273.12E+3 |
| 32 x 3 | 36.1 | 1,641.00 | U Du C Dc | 60.0 1.5 18.3 1.2 | 38.4 2.4 14.6 1.9 | 26.7 3.4 12.2 2.7 | 19.6 4.6 10.5 3.7 | 15.0 6.0 9.1 4.8 | 11.9 9.6 7.7 9.4 8.1 7.3 6.1 7.6 |  | 7.9 11.4 6.7 9.1 |  |  |  |  |  | 22,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 357.31E+3 |
| 32 x 5 | 52.8 | 1,817.00 | U Du C Dc | 89.9 1.5 27.4 1.2 | 57.6 2.4 21.9 1.9 | 40.0 3.4 18.3 2.7 | 29.4 4.6 15.7 3.7 | 22.5 6.0 13.7 4.8 | 17.8 14.4 7.7 9.4 12.2 11.0 6.1 7.6 |  | 11.9 11.4 10.0 9.1 | 10.0 13.6 9.1 10.9 |  |  |  |  | 33,710 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 535.96E+3 |
| 38 x 3 | 42.3 | 1,880.00 | U Du C Dc | 86.1 1.3 26.3 1.0 | 55.1 2.0 21.0 1.6 | 38.3 2.8 17.5 2.3 | 28.1 3.9 15.0 3.1 | 21.5 5.0 13.1 4.0 | 17.0 13.8 6.4 7.9 11.7 10.5 5.1 6.3 |  | 11.4 9.5 9.6 7.6 | 9.6 11.4 8.8 9.1 | 8.2 13.3 8.1 10.7 |  |  |  | 32,260 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 614.52E+3 |
| 38 x 5 | 63.4 | 2,080.00 | U Du C Dc | 129.1 1.3 39.4 1.0 | 82.7 2.0 31.5 1.6 | 57.4 2.8 26.3 2.3 | 42.2 3.9 22.5 3.1 | 32.3 5.0 19.7 4.0 | 25.5 20.7 6.4 7.9 17.5 15.8 5.1 6.3 |  | 17.1 9.5 14.3 7.6 | 14.4 11.4 13.1 9.1 | 12.2 13.3 12.1 10.7 | 10.5 15.5 11.3 12.4 |  |  | 48,390 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 921.77E+3 |
| 45 x 5 | 73.5 | 2,337.00 | U Du C Dc | 176.1 1.1 53.7 0.9 | 112.8 1.7 43.0 1.4 | 78.3 2.4 35.8 1.9 | 57.5 3.3 30.7 2.6 | 44.1 4.3 26.9 3.5 | 34.8 28.2 5.5 6.8 23.9 21.5 4.4 5.4 |  | 23.3 8.2 19.5 6.5 | 19.6 9.7 17.9 7.8 | 16.7 11.4 16.5 9.1 | 14.4 13.2 15.4 10.6 | 12.5 15.2 14.3 12.1 | 11.0 17.3 13.4 13.8 | 66,010 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.47E+6 |
| 51 x 5 | 83.4 | 2,581.00 | U Du C Dc | 229.5 0.9 70.0 0.8 | 146.9 1.5 56.0 1.2 | 102.1 2.1 46.7 1.7 | 75.0 2.9 40.0 2.3 | 57.4 3.8 35.0 3.0 | 45.4 36.8 4.8 5.9 31.1 28.0 3.8 4.7 |  | 30.4 7.2 25.5 5.7 | 25.5 8.5 23.3 6.8 | 21.8 10.0 21.6 8.0 | 18.8 11.6 20.0 9.3 | 16.3 13.3 18.7 10.6 | 14.4 15.1 17.5 12.1 | 86,020 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.18E+6 |
| 64 x 5 | 103.3 | 3,052.00 | U Du C Dc | 358.6 0.8 109.4 0.6 | 229.6 1.2 87.5 0.9 | 159.5 1.7 72.9 1.4 | 117.2 2.3 62.5 1.9 | 89.7 3.0 54.7 2.4 | 70.9 57.4 3.8 4.7 48.6 43.8 3.1 3.8 |  | 47.5 5.7 39.8 4.6 | 39.9 6.8 36.5 5.4 | 34.0 8.0 33.7 6.4 | 29.3 9.3 31.3 7.4 | 25.5 10.6 29.2 8.5 | 22.5 12.1 27.4 9.7 | 134,410 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.27E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 24W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 29 | 53 | 77 | 101 | 125 | 149 | 173 | 197 | 221 | 245 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 269 | 293 | 317 | 341 | 365 | 389 | 413 | 437 | 461 | 485 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 509 | 533 | 557 | 581 | 605 | 629 | 653 | 677 | 701 | 725 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 749 | 773 | 797 | 821 | 845 | 869 | 869 | 917 | 941 | 965 |

Deduct 1.5mm for 3mm bearing bars

LOAD TABLES - SD METRIC

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 3 | 29.4 | 1,133.00 | U Du C Dc | 22.3 2.5 6.8 2.0 | 14.3 3.9 5.4 3.1 | 9.9 5.6 4.5 4.5 | 7.3 7.6 3.9 6.1 | 5.6 9.9 3.4 7.9 |  |  |  |  |  |  |  |  | 8,360 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 81.13E+3 |
| 25 x 5 | 42.7 | 1,254.00 | U Du C Dc | 33.5 2.5 10.2 2.0 | 21.4 3.9 8.2 3.1 | 14.9 5.6 6.8 4.5 | 10.9 8.4 7.6 9.9 5.8 5.1 6.1 7.9 |  | 6.6 12.5 4.5 10.0 |  |  |  |  |  |  |  | 12,550 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 121.69E+3 |
| 32 x 3 | 36.1 | 1,403.00 | U Du C Dc | 39.5 1.9 12.0 1.5 | 25.3 2.9 9.6 2.3 | 17.5 4.2 8.0 3.4 | 12.9 9.9 5.7 7.5 6.9 6.0 4.6 6.0 |  | 7.8 9.4 5.4 7.5 | 6.3 11.6 4.8 9.3 |  |  |  |  |  |  | 14,790 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 190.82E+3 |
| 32 x 5 | 52.8 | 1,553.00 | U Du C Dc | 59.2 1.9 18.1 1.5 | 37.9 2.9 14.4 2.3 | 26.3 4.2 12.0 3.4 | 19.3 14.8 5.7 7.5 10.3 9.0 4.6 6.0 |  | 11.7 9.4 8.0 7.5 | 9.5 11.6 7.2 9.3 | 7.8 14.1 6.6 11.3 |  |  |  |  |  | 22,190 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 286.23E+3 |
| 38 x 3 | 42.7 | 1,653.00 | U Du C Dc | 61.1 1.5 18.6 1.2 | 39.1 2.3 14.9 1.9 | 27.2 3.4 12.4 2.7 | 20.0 15.3 4.6 6.0 10.7 9.3 3.7 4.8 |  | 12.1 7.6 8.3 6.1 | 9.8 9.4 7.5 7.5 | 8.1 11.3 6.8 9.1 |  |  |  |  |  | 22,900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 367.51E+3 |
| 38 x 5 | 62.7 | 1,829.00 | U Du C Dc | 91.6 1.5 28.0 1.2 | 58.7 2.3 22.4 1.9 | 40.8 3.4 18.6 2.7 | 29.9 22.9 4.6 6.0 16.0 14.0 3.7 4.8 |  | 18.1 7.6 12.4 6.1 | 14.7 9.4 11.2 7.5 | 12.1 11.3 10.2 9.1 | 10.2 13.5 9.3 10.8 |  |  |  |  | 34,350 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 551.27E+3 |
| 45 x 5 | 73.5 | 2,097.00 | U Du C Dc | 131.8 1.2 40.2 1.0 | 84.4 2.0 32.2 1.6 | 58.6 2.8 26.8 2.2 | 43.1 33.0 3.8 5.0 23.0 20.1 3.1 4.0 |  | 26.1 6.3 17.9 5.1 | 21.1 7.8 16.1 6.2 | 17.4 9.4 14.6 7.6 | 14.7 11.2 13.4 9.0 | 12.5 13.2 12.4 10.5 | 10.8 15.3 11.5 12.2 |  |  | 49,410 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 951.11E+3 |
| 51 x 5 | 83.4 | 2,349.00 | U Du C Dc | 178.5 1.1 54.4 0.9 | 114.3 1.7 43.6 1.3 | 79.4 2.4 36.3 1.9 | 58.3 44.7 3.3 4.3 31.1 27.2 2.6 3.4 |  | 35.3 5.4 24.2 4.3 | 28.6 6.7 21.8 5.4 | 23.6 8.1 19.8 6.5 | 19.9 9.7 18.2 7.7 | 16.9 11.3 16.8 9.1 | 14.6 13.1 15.6 10.5 | 12.7 15.1 14.5 12.1 | 11.2 17.2 13.6 13.7 | 66,900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.50E+6 |
| 64 x 5 | 103.3 | 2,833.00 | U Du C Dc | 294.1 0.8 89.7 0.7 | 188.3 1.3 71.8 1.0 | 130.8 1.9 59.8 1.5 | 96.1 73.6 2.6 3.3 51.3 44.9 2.0 2.7 |  | 58.1 4.2 39.9 3.4 | 47.1 5.2 35.9 4.2 | 38.9 6.3 32.6 5.1 | 32.7 7.5 29.9 6.0 | 27.9 8.8 27.6 7.1 | 24.0 10.2 25.6 8.2 | 20.9 11.8 23.9 9.4 | 18.4 13.4 22.4 10.7 | 110,210 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.17E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

24W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 29 | 53 | 77 | 101 | 125 | 149 | 173 | 197 | 221 | 245 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 269 | 293 | 317 | 341 | 365 | 389 | 413 | 437 | 461 | 485 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 509 | 533 | 557 | 581 | 605 | 629 | 653 | 677 | 701 | 725 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 749 | 773 | 797 | 821 | 845 | 869 | 869 | 917 | 941 | 965 |

Deduct 1.5mm for 3mm bearing bars

40                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               41

LOAD TABLES | STANDARD DUTY, METRIC

LOAD TABLES - ADA                                                                                                                 LOAD TABLES - ADA

| Grating Type: 17W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 5 | 59.1 | 1,673.00 | U Du C Dc | 81.0 1.9 24.7 1.5 | 51.9 3.0 19.8 2.4 | 36.0 4.3 16.5 3.4 | 26.5 5.8 14.1 4.6 | 20.3 7.6 12.4 6.1 | 16.0 9.6 11.0 7.7 | 13.0 11.8 9.9 9.5 | 10.7 14.3 9.0 11.4 |  |  |  |  |  | 30,360 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 385.58E+3 |
| 32 x 5 | 73.3 | 1,980.00 | U Du C Dc | 127.0 1.5 38.7 1.2 | 81.3 2.4 31.0 1.9 | 56.5 3.4 25.8 2.7 | 41.5 4.6 22.1 3.7 | 31.8 6.0 19.4 4.8 | 25.1 7.7 17.2 6.1 | 20.3 16.8 9.4 11.4 15.5 14.1 7.6 9.1 |  | 14.1 13.6 12.9 10.9 | 12.0 16.0 11.9 12.8 |  |  |  | 47,590 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 756.65E+3 |
| 38 x 5 | 88.0 | 2,268.00 | U Du C Dc | 182.3 1.3 55.6 1.0 | 116.7 2.0 44.5 1.6 | 81.0 2.8 37.1 2.3 | 59.6 3.9 31.8 3.1 | 45.6 5.0 27.8 4.0 | 36.0 6.4 24.7 5.1 | 29.2 24.1 7.9 9.5 22.2 20.2 6.3 7.6 |  | 20.3 11.4 18.5 9.1 | 17.3 13.3 17.1 10.7 | 14.9 15.5 15.9 12.4 | 13.0 17.7 14.8 14.2 |  | 68,310 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.30E+6 |
| 45 x 5 | 102.2 | 2,548.00 | U Du C Dc | 248.7 1.1 75.8 0.9 | 159.2 1.7 60.7 1.4 | 110.6 2.4 50.6 1.9 | 81.2 3.3 43.3 2.6 | 62.2 4.3 37.9 3.5 | 49.2 5.5 33.7 4.4 | 39.8 32.9 6.8 8.2 30.3 27.6 5.4 6.5 |  | 27.7 9.7 25.3 7.8 | 23.6 11.4 23.3 9.1 | 20.3 13.2 21.7 10.6 | 17.7 15.2 20.2 12.1 | 15.6 17.3 19.0 13.8 | 93,190 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.07E+6 |
| 51 x 5 | 116.2 | 2,814.00 | U Du C Dc | 324.0 0.9 98.8 0.8 | 207.4 1.5 79.1 1.2 | 144.1 2.1 65.9 1.7 | 105.9 2.9 56.5 2.3 | 81.1 3.8 49.4 3.0 | 64.1 4.8 43.9 3.8 | 51.9 42.9 5.9 7.2 39.5 36.0 4.7 5.7 |  | 36.0 8.5 33.0 6.8 | 30.7 10.0 30.4 8.0 | 26.5 11.6 28.3 9.3 | 23.1 13.3 26.4 10.6 | 20.3 15.1 24.7 12.1 | 121,440 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.08E+6 |
| 64 x 5 | 144.3 | 3,326.00 | U Du C Dc | 506.3 0.8 154.4 0.6 | 324.1 1.2 123.6 0.9 | 225.1 1.7 103.0 1.4 | 165.4 2.3 88.3 1.9 | 126.7 3.0 77.2 2.4 | 100.1 3.8 68.7 3.1 | 81.1 67.0 4.7 5.7 61.8 56.2 3.8 4.6 |  | 56.3 6.8 51.5 5.4 | 48.0 8.0 47.5 6.4 | 41.4 9.3 44.1 7.4 | 36.0 10.6 41.2 8.5 | 31.7 12.1 38.6 9.7 | 189,750 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.02E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 17W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 22 | 39 | 56 | 73 | 90 | 107 | 124 | 141 | 158 | 175 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 192 | 209 | 226 | 243 | 260 | 277 | 294 | 311 | 328 | 345 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 362 | 379 | 396 | 413 | 430 | 447 | 464 | 481 | 498 | 515 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 532 | 549 | 566 | 583 | 600 | 617 | 617 | 651 | 668 | 685 |

LOAD TABLES - SD METRIC

| Grating Type: 17W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 5 | 59.1 | 1,367.00 | U Du C Dc | 47.3 2.5 14.4 2.0 | 30.3 3.9 11.5 3.1 | 21.0 5.6 9.6 4.5 | 15.4 7.6 8.2 6.1 | 11.8 9.9 7.2 7.9 | 9.3 12.5 6.4 10.0 |  |  |  |  |  |  |  | 17,710 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 171.80E+3 |
| 32 x 5 | 73.3 | 1,693.00 | U Du C Dc | 83.6 1.9 25.5 1.5 | 53.5 2.9 20.4 2.3 | 37.2 4.2 17.0 3.4 | 27.3 5.7 14.6 4.6 | 20.9 16.5 7.5 9.4 12.8 11.3 6.0 7.5 |  | 13.4 11.6 10.2 9.3 | 11.1 14.1 9.3 11.3 | 9.3 16.8 8.5 13.4 |  |  |  |  | 31,320 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 404.08E+3 |
| 38 x 5 | 87.3 | 1,994.00 | U Du C Dc | 129.4 1.5 39.5 1.2 | 82.8 2.3 31.6 1.9 | 57.5 3.4 26.3 2.7 | 42.3 4.6 22.6 3.7 | 32.4 25.6 6.0 7.6 19.7 17.5 4.8 6.1 |  | 20.7 9.4 15.8 7.5 | 17.1 11.3 14.4 9.1 | 14.4 13.5 13.2 10.8 | 12.3 15.8 12.1 12.7 | 10.6 18.3 11.3 14.7 |  |  | 48,490 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 778.26E+3 |
| 45 x 5 | 102.2 | 2,285.00 | U Du C Dc | 186.1 1.2 56.8 1.0 | 119.1 2.0 45.4 1.6 | 82.8 2.8 37.9 2.2 | 60.8 3.8 32.4 3.1 | 46.6 36.8 5.0 6.3 28.4 25.2 4.0 5.1 |  | 29.8 7.8 22.7 6.2 | 24.6 9.4 20.7 7.6 | 20.7 11.2 18.9 9.0 | 17.6 13.2 17.5 10.5 | 15.2 15.3 16.2 12.2 | 13.2 17.6 15.1 14.0 |  | 69,750 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.34E+6 |
| 51 x 5 | 116.2 | 2,561.00 | U Du C Dc | 252.0 1.1 76.9 0.9 | 161.3 1.7 61.5 1.3 | 112.1 2.4 51.3 1.9 | 82.3 3.3 43.9 2.6 | 63.0 49.8 4.3 5.4 38.4 34.2 3.4 4.3 |  | 40.4 6.7 30.8 5.4 | 33.4 8.1 28.0 6.5 | 28.0 9.7 25.6 7.7 | 23.9 11.3 23.7 9.1 | 20.6 13.1 22.0 10.5 | 17.9 15.1 20.5 12.1 | 15.8 17.2 19.2 13.7 | 94,450 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.12E+6 |
| 64 x 5 | 144.3 | 3,088.00 | U Du C Dc | 415.2 0.8 126.6 0.7 | 265.8 1.3 101.3 1.0 | 184.6 1.9 84.4 1.5 | 135.6 2.6 72.4 2.0 | 103.9 82.1 3.3 4.2 63.3 56.3 2.7 3.4 |  | 66.5 5.2 50.7 4.2 | 54.9 6.3 46.1 5.1 | 46.2 7.5 42.2 6.0 | 39.3 8.8 39.0 7.1 | 33.9 10.2 36.2 8.2 | 29.6 11.8 33.8 9.4 | 26.0 13.4 31.7 10.7 | 155,590 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.47E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

17W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 22 | 39 | 56 | 73 | 90 | 107 | 124 | 141 | 158 | 175 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 192 | 209 | 226 | 243 | 260 | 277 | 294 | 311 | 328 | 345 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 362 | 379 | 396 | 413 | 430 | 447 | 464 | 481 | 498 | 515 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 532 | 549 | 566 | 583 | 600 | 617 | 617 | 651 | 668 | 685 |

42                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               43

LOAD TABLES | STANDARD DUTY, METRIC

LOAD TABLES - HEELS FRIENDLY                                                                                            LOAD TABLES - HEELS FRIENDLY

| Grating Type: 11W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 5 | 89.8 | 1,865.00 | U Du C Dc | 125.2 1.9 38.2 1.5 | 80.1 3.0 30.6 2.4 | 55.7 4.3 25.5 3.4 | 40.9 5.8 21.8 4.6 | 31.3 7.6 19.1 6.1 | 24.7 9.6 17.0 7.7 | 20.0 11.8 15.3 9.5 | 16.6 14.3 13.9 11.4 | 13.9 17.0 12.7 13.6 | 11.9 20.0 11.8 16.0 |  |  |  | 46,920 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 595.89E+3 |
| 32 x 5 | 111.8 | 2,208.00 | U Du C Dc | 196.2 1.5 59.9 1.2 | 125.6 2.4 47.9 1.9 | 87.3 3.4 39.9 2.7 | 64.1 4.6 34.2 3.7 | 49.1 6.0 29.9 4.8 | 38.8 7.7 26.6 6.1 | 31.4 9.4 24.0 7.6 | 26.0 11.4 21.8 9.1 | 21.8 18.6 13.6 16.0 20.0 18.4 10.9 12.8 |  | 16.0 18.5 17.1 14.8 | 14.0 21.3 16.0 17.0 |  | 73,540 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.17E+6 |
| 38 x 5 | 134.1 | 2,528.00 | U Du C Dc | 281.7 1.3 85.9 1.0 | 180.3 2.0 68.7 1.6 | 125.3 2.8 57.3 2.3 | 92.0 3.9 49.1 3.1 | 70.5 5.0 43.0 4.0 | 55.7 6.4 38.2 5.1 | 45.1 7.9 34.4 6.3 | 37.3 9.5 31.3 7.6 | 31.3 26.7 11.4 13.3 28.7 26.4 9.1 10.7 |  | 23.0 15.5 24.6 12.4 | 20.1 17.7 22.9 14.2 | 17.6 20.2 21.5 16.1 | 105,570 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.01E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 11W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 16 | 27 | 38 | 49 | 60 | 71 | 82 | 93 | 104 | 115 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 126 | 137 | 148 | 159 | 170 | 181 | 192 | 203 | 214 | 225 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 236 | 247 | 258 | 269 | 280 | 291 | 302 | 313 | 324 | 335 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 346 | 357 | 368 | 379 | 390 | 401 | 401 | 423 | 434 | 445 |
| # of Bars | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| 5mm Bars | 456 | 467 | 478 | 489 | 500 | 511 | 522 | 533 | 544 | 555 |
| # of Bars | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 |
| 5mm Bars | 566 | 577 | 588 | 599 | 610 | 621 | 632 | 643 | 654 | 665 |
| # of Bars | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| 5mm Bars | 676 | 676 | 698 | 709 | 720 | 731 | 742 | 753 | 764 | 775 |

Deduct 1.5mm for 3mm bearing bars

| Grating Type: 11W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Type B Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 124.11 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 610 | 762 | 915 | 1067 | 1220 | 1372 | 1524 | 1677 | 1829 | 1982 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 5 | 89.8 | 1,524.00 | U Du C Dc | 73.0 2.5 22.3 2.0 | 46.8 3.9 17.8 3.1 | 32.5 5.6 14.9 4.5 | 23.9 7.6 12.7 6.1 | 18.3 9.9 11.1 7.9 | 14.4 12.5 9.9 10.0 | 11.7 15.5 8.9 12.4 |  |  |  |  |  |  | 27,370 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 265.51E+3 |
| 32 x 5 | 111.8 | 1,887.00 | U Du C Dc | 129.2 1.9 39.4 1.5 | 82.7 2.9 31.5 2.3 | 57.4 4.2 26.3 3.4 | 42.2 5.7 22.5 4.6 | 32.3 7.5 19.7 6.0 | 25.5 20.7 9.4 11.6 17.5 15.8 7.5 9.3 |  | 17.1 14.1 14.3 11.3 | 14.4 16.8 13.1 13.4 | 12.2 19.7 12.1 15.7 |  |  |  | 48,410 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 624.49E+3 |
| 38 x 5 | 133.4 | 2,223.00 | U Du C Dc | 200.0 1.5 61.0 1.2 | 128.0 2.3 48.8 1.9 | 88.9 3.4 40.7 2.7 | 65.3 4.6 34.9 3.7 | 50.0 6.0 30.5 4.8 | 39.5 32.0 7.6 9.4 27.1 24.4 6.1 7.5 |  | 26.5 11.3 22.2 9.1 | 22.2 13.5 20.3 10.8 | 18.9 15.8 18.8 12.7 | 16.3 18.3 17.4 14.7 | 14.2 21.1 16.3 16.8 |  | 74,940 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.20E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

LOAD TABLES - SD METRIC

11W102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5mm Bars | 16 | 27 | 38 | 49 | 60 | 71 | 82 | 93 | 104 | 115 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 5mm Bars | 126 | 137 | 148 | 159 | 170 | 181 | 192 | 203 | 214 | 225 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 5mm Bars | 236 | 247 | 258 | 269 | 280 | 291 | 302 | 313 | 324 | 335 |
| # of Bars | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| 5mm Bars | 346 | 357 | 368 | 379 | 390 | 401 | 401 | 423 | 434 | 445 |
| # of Bars | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| 5mm Bars | 456 | 467 | 478 | 489 | 500 | 511 | 522 | 533 | 544 | 555 |
| # of Bars | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 |
| 5mm Bars | 566 | 577 | 588 | 599 | 610 | 621 | 632 | 643 | 654 | 665 |
| # of Bars | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| 5mm Bars | 676 | 676 | 698 | 709 | 720 | 731 | 742 | 753 | 764 | 775 |

Deduct 1.5mm for 3mm bearing bars

44                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              45

# LOAD TABLES | HEAVY DUTY, IMPERIAL

DESIGN CRITERIA

OVERVIEW

Heavy-Duty Welded Grating has the strength for heavy- duty load areas such as airfields, industrial plants,

truck and bus terminals, parking lots and railroad

yards. Common uses are flooring, driveways, subway and tunnel ventilation grilles, curb inlet grates, ramps, docks, etc.

It is a sturdy grating to carry loads and maintain the

same level over many years of use. Whenever rolling wheel loads are to be used over the grating, we

recommend that the grating be load banded to add

lateral strength. Serrations are available on bars up to 3/8” thick to provide  additional traction for rolling loads.

Standard heavy-duty bar grating is resistance-welded

for durability, strength and safety using an automated

electric/hydraulic welding process. High temperatures, combined with high pressure fuse the bearing bars and cross bars together to form a permanent joint.

Heavy-Duty

HD Close-Mesh

Wide-Gap

Extra-Wide-Gap

| The load and deflection tables on the following pages have been prepared to provide the designer with a convenient reference for the load carrying capabilities of typical heavy duty grating. Static Loads Uniform loads and concentrated loads per foot of | grating width are given on six inch increments for simple spans ranging from one foot to eight feet. Metric tables provide loads per meter of grating width in 152.4 millimeters increments for spans from 304.8 millimeters to 2438.4 millimeters. The values in these load tables are based on allowable stresses for static loads. |
| --- | --- |

| Determine M: | FS M= 12 |  |  |  |
| --- | --- | --- | --- | --- |
| Substituting for M, solve for L: | (i) a M = | > L PL2 | (ii) a > L M = P(.25L - .125a) b |  |
|  |  | 8ab |  |  |
| Check D*: | D= | P1 [(2 | L3) - (a2 L) + (a3/4)] 96EI |  |

*Deflection should be limited to 1/400 span.

M =   Bending Moment

S =   Section Modulus - in3/ft of grating width I =   Moment of Inertia - in4/ft of grating width

E =   Modulus of Elasticity

F =   Allowable Bending Stress L =   Simple Clear Span - inches D =   Deflection - inches

a =   Partial Load Contact Parallel to Span - inches

LOAD TABLES - HD IMPERIAL

s =   Center-to-Center Spacing Between Bars - inches

b =   Partial Load Contact Dimension at 90° to span - inches b =   a + (2s)

P =   Total Wheel of Partial Load Including Impact - lbs.

P1 =   P per bearing bar P1 =   P x (s/b)

46

VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               47

# LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - HD IMPERIAL

### VEHICULAR LOADS

Note: All loads based on Smooth surface

Vehicular load tables are designed in accordance with the 16th Edition of the American Association of State

Highway and Transportation Officials (AASHTO) for

H-10 through H-25 loads with deflection limited to the

lesser of .125 inches (3.175 mm) or L/400 to a maximum simple span of 8’- 0” (2,438mm). Automobile and forklift

loads are similarly evaluated with loads calculated

and distributed in accordance with the criteria shown

below. If the load conditions of your application are not adequately addressed in the criteria presented, please contact Vulcraft for assistance in determining the

proper grating for your application.

| 19HW4 |  |  |  | Maximum Clear Span Between Supports (in) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx in³/ft. | Ix in⁴/ft. | Unit Wt. lb/ft² | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 1 x ¼ | 0.421 | 0.211 | 9.92 | 15 | 13 | 11 | 8 | 13 | 8 | 7 | 7 |
| 1-¼ x ¼ | 0.658 | 0.411 | 12.06 | 17 | 15 | 12 | 10 | 17 | 10 | 8 | 10 |
| 1-½ x ¼ | 0.947 | 0.711 | 14.21 | 19 | 17 | 15 | 13 | 23 | 12 | 10 | 14 |
| 1-½ x ⅜ | 1.421 | 1.066 | 20.83 | 22 | 20 | 18 | 16 | 32* | 15 | 14 | 20 |
| 2 x ¼ | 1.684 | 1.684 | 18.51 | 24 | 22 | 20 | 18 | 38* | 17 | 16 | 23 |
| 2-½ x ¼ | 2.632 | 3.289 | 22.80 | 31 | 29 | 27 | 26 | 47* | 23 | 22 | 35 |
| 3 x ¼ | 3.789 | 5.684 | 27.10 | 39 | 37 | 36 | 35 | 56* | 31 | 31 | 48* |
| 3 x ⅜ | 5.684 | 8.526 | 40.70 | 47* | 47* | 47* | 48* | 64* | 43 | 44 | 55* |
| 4 x ¼ | 6.737 | 13.474 | 35.69 | 55* | 54* | 55* | 55* | 75* | 50 | 52 | 64* |
| 4 x ⅜ | 10.105 | 20.211 | 53.58 | 62* | 62* | 62* | 63* | 85* | 60* | 61* | 73* |
| 5 x ⅜ | 15.789 | 39.474 | 66.47 | 77* | 77* | 78* | 79* | 107* | 74* | 76* | 92* |
| 6 x ⅜ | 22.737 | 68.211 | 79.35 | 92* | 92* | 93* | 95* | 128* | 89* | 92* | 110* |

Indicates that value was controlled by L/400  ≤ ⅛ " deflection limit.

Note: All loads based on Smooth surface

| 15HW4 |  |  |  | Maximum Clear Span Between Supports (in) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx in³/ft. | Ix in⁴/ft. | Unit Wt. lb/ft² | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 1 x ¼ | 0.533 | 0.267 | 12.21 | 16 | 14 | 11 | 9 | 15 | 9 | 7 | 8 |
| 1-¼ x ¼ | 0.833 | 0.521 | 14.93 | 18 | 16 | 14 | 11 | 20 | 11 | 9 | 12 |
| 1-½ x ¼ | 1.200 | 0.900 | 17.65 | 21 | 18 | 16 | 14 | 27 | 13 | 12 | 16 |
| 1-½ x ⅜ | 1.800 | 1.350 | 25.98 | 25 | 23 | 20 | 19 | 38 | 17 | 16 | 23 |
| 2 x ¼ | 2.133 | 2.133 | 23.09 | 27 | 25 | 23 | 21 | 45 | 19 | 18 | 26 |
| 2-½ x ¼ | 3.333 | 4.167 | 28.53 | 35 | 33 | 32 | 30 | 67 | 27 | 26 | 40 |
| 3 x ¼ | 4.800 | 7.200 | 33.97 | 46 | 44 | 42 | 42 | 84* | 36 | 36 | 57 |
| 3 x ⅜ | 7.200 | 10.800 | 51.01 | 62 | 60 | 59 | 60 | 96* | 51 | 52 | 81* |
| 4 x ¼ | 8.533 | 17.067 | 44.85 | 71 | 70 | 69 | 70 | 112* | 60 | 61 | 95* |
| 4 x ⅜ | 12.800 | 25.600 | 67.33 | 93* | 93* | 93* | 95* | 128* | 87 | 90 | 108* |
| 5 x ⅜ | 20.000 | 50.000 | 83.65 | 115* | 116* | 117* | 118* | 160* | 112* | 114* | 135* |
| 6 x ⅜ | 28.800 | 86.400 | 99.97 | 138* | 139* | 140* | 142* | 192* | 134* | 137* | 163* |

VEHICULAR LOADS

| Vehicular Load Table Criteria | H-255 | H-20/ HL-936 | H-15 | H-102 | Passenger Vehicles | 5 Ton Forklifts3 | 3 Ton Forklifts3 | 1 Ton Forklifts3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Truck/ Vehicle Weight (lbs) |  |  |  |  | 6,322 | 14,400 | 9,800 | 4,200 |
| Load Capacity (lbs) |  |  |  |  | 3,578 | 10,000 | 6,000 | 2,000 |
| Axle Load (lbs) | 40,000 | 32,000 | 24,000 | 16,000 |  |  |  |  |
| Impact Factor | 30% | 30% | 30% | 30% | 30% | 30% | 30% | 30% |
| Total Load (lbs) | 52,000 | 41,600 | 31,200 | 20,800 | 4,651 | 13,000 | 7,800 | 2,600 |
| % of Load on Drive Axel |  |  |  |  | 60% | 85% | 85% | 85% |
| Wheel Load (lbs) | 26,000 | 20,800 | 15,600 | 10,400 | 2,326 | 6,500 | 3,900 | 1,300 |
| A-Length of distribution perpendicular to axle or parallel to main bars (in) | 25 | 20 | 15 | 10 | 9 | 11 | 7 | 4 |
| C-Width of distribution parallel to axle or perpendicular to main bars (in) | 25 | 20 | 15 | 10 | 9 | 11 | 7 | 4 |

Notes:

1.  For continuous spans, use continuity factor = .80.

Indicates that value was controlled by L/400  ≤ ⅛ " deflection limit.

Note: All loads based on Smooth surface

2. This distribution results in larger grating sizes for lighter trucks on shorter spans.

3. The fork lift wheel loads and load distribution patterns depicted above, generally, and only partially, represent the broad range of rubber-tired lift trucks available. For those applications falling outside of these examples, please contact Vulcraft.

4. Wheeled vehicles with urethane tires should NEVER be used in conjunction with open grid bar grating.

5. HS20 is the same as H20 and HS15 is the same as H15. The “S” stands for semi-trailer.

6. The “HL-93” notation shown with “H-20” represents AASHTO’s truck loading standard post-1993. Since, 1993, H-10, H-20, etc. have been retired in lieu of the “HL-93” loading which represents all trucks.

| 30HW4 |  |  |  | Maximum Clear Span Between Supports (in) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx in³/ft. | Ix in⁴/ft. | Unit Wt. lb/ft² | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 1 x ¼ | 0.267 | 0.133 | 6.77 | 14 | 12 | 10 | 7 | 10 | 7 | 6 | 6 |
| 1-¼ x ¼ | 0.417 | 0.260 | 8.13 | 16 | 13 | 11 | 9 | 14 | 9 | 7 | 8 |
| 1-½ x ¼ | 0.600 | 0.450 | 9.49 | 17 | 15 | 12 | 10 | 18 | 10 | 8 | 11 |
| 1-½ x ⅜ | 0.900 | 0.675 | 13.74 | 19 | 17 | 15 | 13 | 24 | 12 | 11 | 16 |
| 2 x ¼ | 1.067 | 1.067 | 12.21 | 20 | 18 | 16 | 14 | 28 | 13 | 12 | 18 |
| 2-½ x ¼ | 1.667 | 2.083 | 14.93 | 25 | 23 | 21 | 20 | 41 | 18 | 17 | 27 |
| 3 x ¼ | 2.400 | 3.600 | 17.65 | 30 | 28 | 27 | 26 | 50* | 23 | 23 | 38 |
| 3 x ⅜ | 3.600 | 5.400 | 26.53 | 39 | 37 | 36 | 37 | 57* | 32 | 33 | 50* |
| 4 x ¼ | 4.267 | 8.533 | 23.09 | 44 | 42 | 42 | 43 | 67* | 37 | 39 | 59* |
| 4 x ⅜ | 6.400 | 12.800 | 34.69 | 55* | 55* | 57* | 56* | 76* | 52 | 55* | 67* |
| 5 x ⅜ | 10.000 | 25.000 | 42.85 | 68* | 68* | 69* | 70* | 95* | 66* | 69* | 84* |
| 6 x ⅜ | 14.400 | 43.200 | 51.01 | 81* | 81* | 82* | 84* | 114* | 79* | 82* | 101* |

Indicates that value was controlled by L/400  ≤ ⅛ " deflection limit.

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              49

48                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES | HEAVY DUTY, IMPERIAL

VEHICULAR LOADS

Note: All loads based on Smooth surface

| 38HW4 |  |  |  | Maximum Clear Span Between Supports (in) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx in³/ft. | Ix in⁴/ft. | Unit Wt. lb/ft² | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 1 x ¼ | 0.211 | 0.105 | 5.62 | 14 | 12 | 9 | 7 | 9 | 7 | 5 | 6 |
| 1-¼ x ¼ | 0.329 | 0.206 | 6.70 | 15 | 13 | 10 | 8 | 12 | 8 | 6 | 8 |
| 1-½ x ¼ | 0.474 | 0.355 | 7.77 | 16 | 14 | 11 | 9 | 16 | 9 | 8 | 10 |
| 1-½ x ⅜ | 0.711 | 0.533 | 11.17 | 18 | 16 | 13 | 12 | 21 | 11 | 10 | 14 |
| 2 x ¼ | 0.842 | 0.842 | 9.92 | 19 | 17 | 15 | 13 | 24 | 12 | 11 | 16 |
| 2-½ x ¼ | 1.316 | 1.645 | 12.06 | 23 | 20 | 19 | 17 | 36 | 16 | 15 | 24 |
| 3 x ¼ | 1.895 | 2.842 | 14.21 | 27 | 25 | 23 | 23 | 48* | 20 | 21 | 34 |
| 3 x ⅜ | 2.842 | 4.263 | 21.37 | 34 | 33 | 31 | 32 | 54* | 28 | 29 | 48* |
| 4 x ¼ | 3.368 | 6.737 | 18.51 | 38 | 37 | 36 | 37 | 63* | 32 | 34 | 57* |
| 4 x ⅜ | 5.053 | 10.105 | 27.82 | 51 | 50 | 50 | 53 | 72* | 45 | 49 | 65* |
| 5 x ⅜ | 7.895 | 19.737 | 34.26 | 63* | 64* | 64* | 67* | 90* | 63* | 65* | 81* |
| 6 x ⅜ | 11.368 | 34.105 | 40.70 | 75* | 76* | 77* | 80* | 108* | 75* | 78* | 97* |

LOAD TABLES - HD IMPERIAL

Grating Frames

Vulcraft’s structural fabrication services can be

leveraged to further aid you in getting a superior

solution for covering your concrete opening by also obtaining an Embed Frame with your grating. A steel embed frame can improve the quality and lifespan of your project by:

•  Shielding the concrete at the opening edges from cracking and chipping,

•  Providing an edge for the opening when forming the concrete pour,

•  Providing uniform elevation for the opening to minimize potential for uneven surfaces,

•  And providing a smooth an uniform bearing surface for the grating, allowing for easier attachment and better performance over it’s lifetime.

Frames are available in normal rectangular

configurations only and will be supplied as a fully-

assembled, four-sided unit in sizes up to those that can safely be transported via normal flatbed carriers. Sizes or configurations other than this should be discussed

with Vulcraft. Embed frames can be supplied mill

finished, painted, or hot-dipped galvanized. To order, please include a detail similar to the following with the Clear Opening Width and Span clearly defined as well as the desired quantities and finish.

* Indicates that value was controlled by L/400  ≤ ⅛ " deflection limit.

50                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               51

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - HD

| Grating Type: 19HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 9.92 | 61 | U Du C Dc | 5,614 0.02 2,807 0.02 | 2,495 0.05 1,871 0.04 | 1,404 0.08 1,404 0.07 | 898 0.13 1,123 0.10 | 624 0.19 936 0.15 | 458 0.25 802 0.20 | 351 0.33 702 0.26 | 277 0.42 624 0.34 | 225 0.52 561 0.41 | 186 0.63 510 0.50 |  |  |  |  |  | 0.421 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.211 |
| 1 ¼" x ¼" | 12.06 | 72 | U Du C Dc | 8,772 0.02 4,386 0.01 | 3,899 0.04 2,924 0.03 | 2,193 0.07 2,193 0.05 | 1,404 0.10 1,754 0.08 | 975 0.15 1,462 0.12 | 716 0.20 1,253 0.16 | 548 0.26 1,096 0.21 | 433 0.34 975 0.27 | 351 290 0.41 0.50 877 797 0.33 0.40 |  | 244 0.60 731 0.48 |  |  |  |  | 0.658 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.411 |
| 1 ½" x ¼" | 14.21 | 83 | U Du C Dc | 12,632 0.01 6,316 0.01 | 5,614 0.03 4,211 0.02 | 3,158 0.06 3,158 0.04 | 2,021 0.09 2,526 0.07 | 1,404 0.12 2,105 0.10 | 1,031 0.17 1,805 0.14 | 789 0.22 1,579 0.18 | 624 0.28 1,404 0.22 | 505 418 0.34 0.42 1,263 1,148 0.28 0.33 |  | 351 0.50 1,053 0.40 | 299 0.58 972 0.47 | 258 0.68 902 0.54 |  |  | 0.947 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.711 |
| 1 ½" x ⅜" | 20.83 | 92 | U Du C Dc | 18,947 0.01 9,474 0.01 | 8,421 0.03 6,316 0.02 | 4,737 0.06 4,737 0.04 | 3,032 0.09 3,789 0.07 | 2,105 0.12 3,158 0.10 | 1,547 0.17 2,707 0.14 | 1,184 0.22 2,368 0.18 | 936 0.28 2,105 0.22 | 758 626 0.34 0.42 1,895 1,722 0.28 0.33 |  | 526 0.50 1,579 0.40 | 448 0.58 1,457 0.47 | 387 0.68 1,353 0.54 | 337 0.78 1,263 0.62 | 296 0.88 1,184 0.71 | 1.421 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.066 |
| 2" x ¼" | 18.51 | 103 | U Du C Dc | 22,456 0.01 11,228 0.01 | 9,981 0.02 7,485 0.02 | 5,614 0.04 5,614 0.03 | 3,593 0.06 4,491 0.05 | 2,495 0.09 3,743 0.07 | 1,833 0.13 3,208 0.10 | 1,404 0.17 2,807 0.13 | 1,109 0.21 2,495 0.17 | 898 742 0.26 0.31 2,246 2,041 0.21 0.25 |  | 624 0.37 1,871 0.30 | 532 0.44 1,727 0.35 | 458 0.51 1,604 0.41 | 399 0.58 1,497 0.47 | 351 0.66 1,404 0.53 | 1.684 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.684 |
| 2 ½" x ¼" | 22.80 | 122 | U Du C Dc | 35,088 0.01 17,544 0.01 | 15,595 0.02 11,696 0.01 | 8,772 0.03 8,772 0.03 | 5,614 0.05 7,018 0.04 | 3,899 0.07 5,848 0.06 | 2,864 0.10 5,013 0.08 | 2,193 0.13 4,386 0.11 | 1,733 0.17 3,899 0.13 | 1,404 1,160 0.21 0.25 3,509 3,190 0.17 0.20 |  | 975 0.30 2,924 0.24 | 830 0.35 2,699 0.28 | 716 0.41 2,506 0.32 | 624 0.47 2,339 0.37 | 548 0.53 2,193 0.42 | 2.632 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.289 |
| 3" x ¼" | 27.10 | 140 | U Du C Dc | 50,526 0.01 25,263 0.01 | 22,456 0.02 16,842 0.01 | 12,632 0.03 12,632 0.02 | 8,084 0.04 10,105 0.03 | 5,614 0.06 8,421 0.05 | 4,125 0.08 7,218 0.07 | 3,158 0.11 6,316 0.09 | 2,495 0.14 5,614 0.11 | 2,021 1,670 0.17 0.21 5,053 4,593 0.14 0.17 |  | 1,404 0.25 4,211 0.20 | 1,196 0.29 3,887 0.23 | 1,031 0.34 3,609 0.27 | 898 0.39 3,368 0.31 | 789 0.44 3,158 0.35 | 3.789 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.684 |
| 3" x ⅜" | 40.70 | 154 | U Du C Dc | 75,789 0.01 37,895 0.01 | 33,684 0.02 25,263 0.01 | 18,947 0.03 18,947 0.02 | 12,126 0.04 15,158 0.03 | 8,421 0.06 12,632 0.05 | 6,187 0.08 10,827 0.07 | 4,737 0.11 9,474 0.09 | 3,743 0.14 8,421 0.11 | 3,032 2,505 0.17 0.21 7,579 6,890 0.14 0.17 |  | 2,105 0.25 6,316 0.20 | 1,794 0.29 5,830 0.23 | 1,547 0.34 5,414 0.27 | 1,347 0.39 5,053 0.31 | 1,184 0.44 4,737 0.35 | 5.684 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.526 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

19HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-7/16 | 2-5/8 | 3-13/16 | 5 | 6-3/16 | 7-3/8 | 8-9/16 | 9-3/4 | 10-15/16 | 12-1/8 |
| 3/8” Bars | 1-9/16 | 2-3/4 | 3-15/16 | 5-1/8 | 6-5/16 | 7-1/2 | 8-11/16 | 9-7/8 | 11-1/16 | 12-1/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 13-5/16 | 14-1/2 | 15-11/16 | 16-7/8 | 18-1/16 | 19-1/4 | 20-7/16 | 21-5/8 | 22-13/16 | 24 |
| 3/8” Bars | 13-7/16 | 14-5/8 | 15-13/16 | 17 | 18-3/16 | 19-3/8 | 20-9/16 | 21-3/4 | 22-15/16 | 24-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 25-3/16 | 26-3/8 | 27-9/16 | 28-3/4 | 29-15/16 | 31-1/8 | 32-5/16 | 33-1/2 | 34-11/16 | 35-7/8 |
| 3/8” Bars | 25-5/16 | 26-1/2 | 27-11/16 | 28-7/8 | 30-1/16 | 31-1/4 | 32-7/16 | 33-5/8 | 34-13/16 | 36 |

52                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - HD

| Grating Type: 19HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 31.39 | 157 | U Du C Dc | 68,772 0.01 34,386 0.00 | 30,565 0.01 22,924 0.01 | 17,193 0.02 17,193 0.02 | 11,004 0.04 13,754 0.03 | 7,641 0.05 11,462 0.04 | 5,614 0.07 9,825 0.06 | 4,298 0.09 8,596 0.08 | 3,396 0.12 7,641 0.10 | 2,751 0.15 6,877 0.12 | 2,273 0.18 6,252 0.14 | 1,910 0.21 5,731 0.17 | 1,628 0.25 5,290 0.20 | 1,404 0.29 4,912 0.23 | 1,223 0.33 4,585 0.27 | 1,075 0.38 4,298 0.30 | 5.158 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.026 |
| 3 ½" x ⅜" | 47.14 | 173 | U Du C Dc | 103,158 0.01 51,579 0.00 | 45,848 0.01 34,386 0.01 | 25,789 0.02 25,789 0.02 | 16,505 0.04 20,632 0.03 | 11,462 0.05 17,193 0.04 | 8,421 0.07 14,737 0.06 | 6,447 0.09 12,895 0.08 | 5,094 0.12 11,462 0.10 | 4,126 0.15 10,316 0.12 | 3,410 0.18 9,378 0.14 | 2,865 0.21 8,596 0.17 | 2,442 0.25 7,935 0.20 | 2,105 0.29 7,368 0.23 | 1,834 0.33 6,877 0.27 | 1,612 0.38 6,447 0.30 | 7.737 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.539 |
| 4" x¼" | 35.69 | 173 | U Du C Dc | 89,825 0.01 44,912 0.00 | 39,922 0.01 29,942 0.01 | 22,456 0.02 22,456 0.02 | 14,372 0.03 17,965 0.03 | 9,981 0.05 14,971 0.04 | 7,333 0.06 12,832 0.05 | 5,614 0.08 11,228 0.07 | 4,436 0.10 9,981 0.08 | 3,593 0.13 8,982 0.10 | 2,969 0.16 8,166 0.13 | 2,495 0.19 7,485 0.15 | 2,126 0.22 6,910 0.17 | 1,833 0.25 6,416 0.20 | 1,597 0.29 5,988 0.23 | 1,404 0.33 5,614 0.26 | 6.737 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.474 |
| 4" x ⅜" | 53.58 | 192 | U Du C Dc | 134,737 0.01 67,368 0.00 | 59,883 0.01 44,912 0.01 | 33,684 0.02 33,684 0.02 | 21,558 0.03 26,947 0.03 | 14,971 0.05 22,456 0.04 | 10,999 0.06 19,248 0.05 | 8,421 0.08 16,842 0.07 | 6,654 0.10 14,971 0.08 | 5,389 0.13 13,474 0.10 | 4,454 0.16 12,249 0.13 | 3,743 0.19 11,228 0.15 | 3,189 0.22 10,364 0.17 | 2,750 0.25 9,624 0.20 | 2,395 0.29 8,982 0.23 | 2,105 0.33 8,421 0.26 | 10.105 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20.211 |
| 5" x ⅜" | 66.47 | 227 | U Du C Dc | 210,526 0.00 105,263 0.00 | 93,567 0.01 70,175 0.01 | 52,632 0.02 52,632 0.01 | 33,684 0.03 42,105 0.02 | 23,392 0.04 35,088 0.03 | 17,186 0.05 30,075 0.04 | 13,158 0.07 26,316 0.05 | 10,396 0.08 23,392 0.07 | 8,421 0.10 21,053 0.08 | 6,960 0.13 19,139 0.10 | 5,848 0.15 17,544 0.12 | 4,983 0.17 16,194 0.14 | 4,296 0.20 15,038 0.16 | 3,743 0.23 14,035 0.19 | 3,289 0.26 13,158 0.21 | 15.789 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39.474 |
| 6" x ⅜" | 79.35 | 260 | U Du C Dc | 303,158 0.00 151,579 0.00 | 134,737 0.01 101,053 0.01 | 75,789 0.01 75,789 0.01 | 48,505 0.02 60,632 0.02 | 33,684 0.03 50,526 0.02 | 24,748 0.04 43,308 0.03 | 18,947 0.06 37,895 0.04 | 14,971 0.07 33,684 0.06 | 12,126 0.09 30,316 0.07 | 10,022 0.10 27,560 0.08 | 8,421 0.12 25,263 0.10 | 7,175 0.15 23,320 0.12 | 6,187 0.17 21,654 0.14 | 5,389 0.19 20,211 0.16 | 4,737 0.22 18,947 0.18 | 22.737 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 68.211 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 19HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-7/16 | 2-5/8 | 3-13/16 | 5 | 6-3/16 | 7-3/8 | 8-9/16 | 9-3/4 | 10-15/16 | 12-1/8 |
| 3/8” Bars | 1-9/16 | 2-3/4 | 3-15/16 | 5-1/8 | 6-5/16 | 7-1/2 | 8-11/16 | 9-7/8 | 11-1/16 | 12-1/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 13-5/16 | 14-1/2 | 15-11/16 | 16-7/8 | 18-1/16 | 19-1/4 | 20-7/16 | 21-5/8 | 22-13/16 | 24 |
| 3/8” Bars | 13-7/16 | 14-5/8 | 15-13/16 | 17 | 18-3/16 | 19-3/8 | 20-9/16 | 21-3/4 | 22-15/16 | 24-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 25-3/16 | 26-3/8 | 27-9/16 | 28-3/4 | 29-15/16 | 31-1/8 | 32-5/16 | 33-1/2 | 34-11/16 | 35-7/8 |
| 3/8” Bars | 25-5/16 | 26-1/2 | 27-11/16 | 28-7/8 | 30-1/16 | 31-1/4 | 32-7/16 | 33-5/8 | 34-13/16 | 36 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               53

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - HD

| Grating Type: 19HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 9.92 | 49 | U Du C Dc | 3,158 0.03 1,579 0.02 | 1,404 0.06 1,053 0.05 | 789 0.11 789 0.09 | 505 0.17 632 0.14 | 351 0.25 526 0.20 | 258 0.34 451 0.27 | 197 0.44 395 0.35 | 156 0.56 351 0.45 |  |  |  |  |  |  |  | 0.237 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.089 |
| 1 ¼" x ¼" | 12.06 | 61 | U Du C Dc | 5,614 0.02 2,807 0.02 | 2,495 0.05 1,871 0.04 | 1,404 0.08 1,404 0.07 | 898 0.13 1,123 0.10 | 624 0.19 936 0.15 | 458 0.25 802 0.20 | 351 277 0.33 0.42 702 624 0.26 0.34 |  | 225 0.52 561 0.41 | 186 0.63 510 0.50 |  |  |  |  |  | 0.421 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.211 |
| 1 ½" x ¼" | 14.21 | 72 | U Du C Dc | 8,772 0.02 4,386 0.01 | 3,899 0.04 2,924 0.03 | 2,193 0.07 2,193 0.05 | 1,404 0.10 1,754 0.08 | 975 0.15 1,462 0.12 | 716 0.20 1,253 0.16 | 548 433 0.26 0.34 1,096 975 0.21 0.27 |  | 351 0.41 877 0.33 | 290 0.50 797 0.40 | 244 0.60 731 0.48 |  |  |  |  | 0.658 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.411 |
| 1 ½" x ⅜" | 20.83 | 80 | U Du C Dc | 13,158 0.02 6,579 0.01 | 5,848 0.04 4,386 0.03 | 3,289 0.07 3,289 0.05 | 2,105 0.10 2,632 0.08 | 1,462 0.15 2,193 0.12 | 1,074 0.20 1,880 0.16 | 822 650 0.26 0.34 1,645 1,462 0.21 0.27 |  | 526 0.41 1,316 0.33 | 435 0.50 1,196 0.40 | 365 0.60 1,096 0.48 | 311 0.70 1,012 0.56 | 269 0.81 940 0.65 |  |  | 0.987 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.617 |
| 2" x ¼" | 18.51 | 93 | U Du C Dc | 17,193 0.01 8,596 0.01 | 7,641 0.03 5,731 0.02 | 4,298 0.05 4,298 0.04 | 2,751 0.07 3,439 0.06 | 1,910 0.11 2,865 0.09 | 1,404 0.14 2,456 0.12 | 1,075 849 0.19 0.24 2,149 1,910 0.15 0.19 |  | 688 0.30 1,719 0.24 | 568 0.36 1,563 0.29 | 478 0.43 1,433 0.34 | 407 0.50 1,323 0.40 | 351 0.58 1,228 0.46 | 306 0.67 1,146 0.53 | 269 0.76 1,075 0.61 | 1.289 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.128 |
| 2 ½" x ¼" | 22.80 | 113 | U Du C Dc | 28,421 0.01 14,211 0.01 | 12,632 0.02 9,474 0.02 | 7,105 0.04 7,105 0.03 | 4,547 0.06 5,684 0.05 | 3,158 0.08 4,737 0.07 | 2,320 0.11 4,060 0.09 | 1,776 1,404 0.15 0.19 3,553 3,158 0.12 0.15 |  | 1,137 0.23 2,842 0.18 | 940 0.28 2,584 0.22 | 789 0.33 2,368 0.26 | 673 0.39 2,186 0.31 | 580 0.45 2,030 0.36 | 505 0.52 1,895 0.41 | 444 0.59 1,776 0.47 | 2.132 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.398 |
| 3" x ¼" | 27.10 | 131 | U Du C Dc | 42,456 0.01 21,228 0.01 | 18,869 0.02 14,152 0.01 | 10,614 0.03 10,614 0.02 | 6,793 0.05 8,491 0.04 | 4,717 0.07 7,076 0.05 | 3,466 0.09 6,065 0.07 | 2,654 2,097 0.12 0.15 5,307 4,717 0.10 0.12 |  | 1,698 0.19 4,246 0.15 | 1,404 0.23 3,860 0.18 | 1,179 0.27 3,538 0.22 | 1,005 0.32 3,266 0.25 | 866 0.37 3,033 0.29 | 755 0.42 2,830 0.34 | 663 0.48 2,654 0.39 | 3.184 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.378 |
| 3" x ⅜" | 40.16 | 145 | U Du C Dc | 63,684 0.01 31,842 0.01 | 28,304 0.02 21,228 0.01 | 15,921 0.03 15,921 0.02 | 10,189 0.05 12,737 0.04 | 7,076 0.07 10,614 0.05 | 5,199 0.09 9,098 0.07 | 3,980 3,145 0.12 0.15 7,961 7,076 0.10 0.12 |  | 2,547 0.19 6,368 0.15 | 2,105 0.23 5,789 0.18 | 1,769 0.27 5,307 0.22 | 1,507 0.32 4,899 0.25 | 1,300 0.37 4,549 0.29 | 1,132 0.42 4,246 0.34 | 995 0.48 3,980 0.39 | 4.776 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.567 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

19HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-7/16 | 2-5/8 | 3-13/16 | 5 | 6-3/16 | 7-3/8 | 8-9/16 | 9-3/4 | 10-15/16 | 12-1/8 |
| 3/8” Bars | 1-9/16 | 2-3/4 | 3-15/16 | 5-1/8 | 6-5/16 | 7-1/2 | 8-11/16 | 9-7/8 | 11-1/16 | 12-1/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 13-5/16 | 14-1/2 | 15-11/16 | 16-7/8 | 18-1/16 | 19-1/4 | 20-7/16 | 21-5/8 | 22-13/16 | 24 |
| 3/8” Bars | 13-7/16 | 14-5/8 | 15-13/16 | 17 | 18-3/16 | 19-3/8 | 20-9/16 | 21-3/4 | 22-15/16 | 24-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 25-3/16 | 26-3/8 | 27-9/16 | 28-3/4 | 29-15/16 | 31-1/8 | 32-5/16 | 33-1/2 | 34-11/16 | 35-7/8 |
| 3/8” Bars | 25-5/16 | 26-1/2 | 27-11/16 | 28-7/8 | 30-1/16 | 31-1/4 | 32-7/16 | 33-5/8 | 34-13/16 | 36 |

54                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - HD

| Grating Type: 19HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 31.39 | 148 | U Du C Dc | 59,298 0.01 29,649 0.01 | 26,355 0.01 19,766 0.01 | 14,825 0.03 14,825 0.02 | 9,488 0.04 11,860 0.03 | 6,589 0.06 9,883 0.05 | 4,841 0.08 8,471 0.06 | 3,706 0.10 7,412 0.08 | 2,928 0.13 6,589 0.10 | 2,372 0.16 5,930 0.13 | 1,960 0.19 5,391 0.15 | 1,647 0.23 4,942 0.18 | 1,404 0.27 4,561 0.22 | 1,210 0.31 4,236 0.25 | 1,054 0.36 3,953 0.29 | 927 0.41 3,706 0.33 | 4.447 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.227 |
| 3 ½" x ⅜" | 47.14 | 164 | U Du C Dc | 88,947 0.01 44,474 0.01 | 39,532 0.01 29,649 0.01 | 22,237 0.03 22,237 0.02 | 14,232 0.04 17,789 0.03 | 9,883 0.06 14,825 0.05 | 7,261 0.08 12,707 0.06 | 5,559 0.10 11,118 0.08 | 4,392 0.13 9,883 0.10 | 3,558 0.16 8,895 0.13 | 2,940 0.19 8,086 0.15 | 2,471 0.23 7,412 0.18 | 2,105 0.27 6,842 0.22 | 1,815 0.31 6,353 0.25 | 1,581 0.36 5,930 0.29 | 1,390 0.41 5,559 0.33 | 6.671 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.840 |
| 4" x¼" | 35.69 | 165 | U Du C Dc | 78,947 0.01 39,474 0.00 | 35,088 0.01 26,316 0.01 | 19,737 0.02 19,737 0.02 | 12,632 0.03 15,789 0.03 | 8,772 0.05 13,158 0.04 | 6,445 0.07 11,278 0.05 | 4,934 0.09 9,868 0.07 | 3,899 0.11 8,772 0.09 | 3,158 0.14 7,895 0.11 | 2,610 0.17 7,177 0.13 | 2,193 0.20 6,579 0.16 | 1,869 0.23 6,073 0.19 | 1,611 0.27 5,639 0.22 | 1,404 0.31 5,263 0.25 | 1,234 0.35 4,934 0.28 | 5.921 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.102 |
| 4" x ⅜" | 53.58 | 183 | U Du C Dc | 118,421 0.01 59,211 0.00 | 52,632 0.01 39,474 0.01 | 29,605 0.02 29,605 0.02 | 18,947 0.03 23,684 0.03 | 13,158 0.05 19,737 0.04 | 9,667 0.07 16,917 0.05 | 7,401 0.09 14,803 0.07 | 5,848 0.11 13,158 0.09 | 4,737 0.14 11,842 0.11 | 3,915 0.17 10,766 0.13 | 3,289 0.20 9,868 0.16 | 2,803 0.23 9,109 0.19 | 2,417 0.27 8,459 0.22 | 2,105 0.31 7,895 0.25 | 1,850 0.35 7,401 0.28 | 8.882 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16.653 |
| 5" x ⅜" | 66.47 | 218 | U Du C Dc | 190,000 0.00 95,000 0.00 | 84,444 0.01 63,333 0.01 | 47,500 0.02 47,500 0.01 | 30,400 0.03 38,000 0.02 | 21,111 0.04 31,667 0.03 | 15,510 0.05 27,143 0.04 | 11,875 0.07 23,750 0.06 | 9,383 0.09 21,111 0.07 | 7,600 0.11 19,000 0.09 | 6,281 0.13 17,273 0.11 | 5,278 0.16 15,833 0.13 | 4,497 0.18 14,615 0.15 | 3,878 0.21 13,571 0.17 | 3,378 0.25 12,667 0.20 | 2,969 0.28 11,875 0.22 | 14.250 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 33.844 |
| 6" x ⅜" | 79.35 | 252 | U Du C Dc | 278,421 0.00 139,211 0.00 | 123,743 0.01 92,807 0.01 | 69,605 0.01 69,605 0.01 | 44,547 0.02 55,684 0.02 | 30,936 0.03 46,404 0.03 | 22,728 0.04 39,774 0.04 | 17,401 0.06 34,803 0.05 | 13,749 0.07 30,936 0.06 | 11,137 0.09 27,842 0.07 | 9,204 0.11 25,311 0.09 | 7,734 0.13 23,202 0.10 | 6,590 0.15 21,417 0.12 | 5,682 0.18 19,887 0.14 | 4,950 0.20 18,561 0.16 | 4,350 0.23 17,401 0.18 | 20.882 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60.035 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

19HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-7/16 | 2-5/8 | 3-13/16 | 5 | 6-3/16 | 7-3/8 | 8-9/16 | 9-3/4 | 10-15/16 | 12-1/8 |
| 3/8” Bars | 1-9/16 | 2-3/4 | 3-15/16 | 5-1/8 | 6-5/16 | 7-1/2 | 8-11/16 | 9-7/8 | 11-1/16 | 12-1/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 13-5/16 | 14-1/2 | 15-11/16 | 16-7/8 | 18-1/16 | 19-1/4 | 20-7/16 | 21-5/8 | 22-13/16 | 24 |
| 3/8” Bars | 13-7/16 | 14-5/8 | 15-13/16 | 17 | 18-3/16 | 19-3/8 | 20-9/16 | 21-3/4 | 22-15/16 | 24-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 25-3/16 | 26-3/8 | 27-9/16 | 28-3/4 | 29-15/16 | 31-1/8 | 32-5/16 | 33-1/2 | 34-11/16 | 35-7/8 |
| 3/8” Bars | 25-5/16 | 26-1/2 | 27-11/16 | 28-7/8 | 30-1/16 | 31-1/4 | 32-7/16 | 33-5/8 | 34-13/16 | 36 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              55

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 15HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 12.21 | 65 | U Du C Dc | 7,111 0.02 3,556 0.02 | 3,160 0.05 2,370 0.04 | 1,778 0.08 1,778 0.07 | 1,138 0.13 1,422 0.10 | 790 0.19 1,185 0.15 | 580 0.25 1,016 0.20 | 444 0.33 889 0.26 | 351 0.42 790 0.34 | 284 0.52 711 0.41 | 235 0.63 646 0.50 |  |  |  |  |  | 0.533 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.267 |
| 1 ¼" x ¼" | 14.93 | 77 | U Du C Dc | 11,111 0.02 5,556 0.01 | 4,938 0.04 3,704 0.03 | 2,778 0.07 2,778 0.05 | 1,778 0.10 2,222 0.08 | 1,235 0.15 1,852 0.12 | 907 0.20 1,587 0.16 | 694 0.26 1,389 0.21 | 549 0.34 1,235 0.27 | 444 367 0.41 0.50 1,111 1,010 0.33 0.40 |  | 309 0.60 926 0.48 | 263 0.70 855 0.56 |  |  |  | 0.833 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.521 |
| 1 ½" x ¼" | 17.65 | 88 | U Du C Dc | 16,000 0.01 8,000 0.01 | 7,111 0.03 5,333 0.02 | 4,000 0.06 4,000 0.04 | 2,560 0.09 3,200 0.07 | 1,778 0.12 2,667 0.10 | 1,306 0.17 2,286 0.14 | 1,000 0.22 2,000 0.18 | 790 0.28 1,778 0.22 | 640 529 0.34 0.42 1,600 1,455 0.28 0.33 |  | 444 0.50 1,333 0.40 | 379 0.58 1,231 0.47 | 327 0.68 1,143 0.54 | 284 0.78 1,067 0.62 |  | 1.200 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.900 |
| 1 ½" x ⅜" | 25.98 | 97 | U Du C Dc | 24,000 0.01 12,000 0.01 | 10,667 0.03 8,000 0.02 | 6,000 0.06 6,000 0.04 | 3,840 0.09 4,800 0.07 | 2,667 0.12 4,000 0.10 | 1,959 0.17 3,429 0.14 | 1,500 0.22 3,000 0.18 | 1,185 0.28 2,667 0.22 | 960 793 0.34 0.42 2,400 2,182 0.28 0.33 |  | 667 0.50 2,000 0.40 | 568 0.58 1,846 0.47 | 490 0.68 1,714 0.54 | 427 0.78 1,600 0.62 | 375 0.88 1,500 0.71 | 1.800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.350 |
| 2" x ¼" | 23.09 | 109 | U Du C Dc | 28,444 0.01 14,222 0.01 | 12,642 0.02 9,481 0.02 | 7,111 0.04 7,111 0.03 | 4,551 0.06 5,689 0.05 | 3,160 0.09 4,741 0.07 | 2,322 0.13 4,063 0.10 | 1,778 0.17 3,556 0.13 | 1,405 0.21 3,160 0.17 | 1,138 940 0.26 0.31 2,844 2,586 0.21 0.25 |  | 790 0.37 2,370 0.30 | 673 0.44 2,188 0.35 | 580 0.51 2,032 0.41 | 506 0.58 1,896 0.47 | 444 0.66 1,778 0.53 | 2.133 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.133 |
| 2 ½" x ¼" | 28.53 | 129 | U Du C Dc | 44,444 0.01 22,222 0.01 | 19,753 0.02 14,815 0.01 | 11,111 0.03 11,111 0.03 | 7,111 0.05 8,889 0.04 | 4,938 0.07 7,407 0.06 | 3,628 0.10 6,349 0.08 | 2,778 0.13 5,556 0.11 | 2,195 0.17 4,938 0.13 | 1,778 1,469 0.21 0.25 4,444 4,040 0.17 0.20 |  | 1,235 0.30 3,704 0.24 | 1,052 0.35 3,419 0.28 | 907 0.41 3,175 0.32 | 790 0.47 2,963 0.37 | 694 0.53 2,778 0.42 | 3.333 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.167 |
| 3" x ¼" | 33.97 | 148 | U Du C Dc | 64,000 0.01 32,000 0.01 | 28,444 0.02 21,333 0.01 | 16,000 0.03 16,000 0.02 | 10,240 0.04 12,800 0.03 | 7,111 0.06 10,667 0.05 | 5,224 0.08 9,143 0.07 | 4,000 0.11 8,000 0.09 | 3,160 0.14 7,111 0.11 | 2,560 2,116 0.17 0.21 6,400 5,818 0.14 0.17 |  | 1,778 0.25 5,333 0.20 | 1,515 0.29 4,923 0.23 | 1,306 0.34 4,571 0.27 | 1,138 0.39 4,267 0.31 | 1,000 0.44 4,000 0.35 | 4.800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.200 |
| 3" x ⅜" | 51.01 | 164 | U Du C Dc | 96,000 0.01 48,000 0.01 | 42,667 0.02 32,000 0.01 | 24,000 0.03 24,000 0.02 | 15,360 0.04 19,200 0.03 | 10,667 0.06 16,000 0.05 | 7,837 0.08 13,714 0.07 | 6,000 0.11 12,000 0.09 | 4,741 0.14 10,667 0.11 | 3,840 3,174 0.17 0.21 9,600 8,727 0.14 0.17 |  | 2,667 0.25 8,000 0.20 | 2,272 0.29 7,385 0.23 | 1,959 0.34 6,857 0.27 | 1,707 0.39 6,400 0.31 | 1,500 0.44 6,000 0.35 | 7.200 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.800 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 15HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-3/16 | 2-1/8 | 3-1/16 | 4 | 4-15/16 | 5-7/8 | 6-13/16 | 7-3/4 | 8-11/16 | 9-5/8 |
| 3/8” Bars | 1-5/16 | 2-1/4 | 3-3/16 | 4-1/8 | 5-1/16 | 6 | 6-15/16 | 7-7/8 | 8-13/16 | 9-3/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 10-9/16 | 11-1/2 | 12-7/16 | 13-3/8 | 14-5/16 | 15-1/4 | 16-3/16 | 17-1/8 | 18-1/16 | 19 |
| 3/8” Bars | 10-11/16 | 11-5/8 | 12-9/16 | 13-1/2 | 14-7/16 | 15-3/8 | 16-5/16 | 17-1/4 | 18-3/16 | 19-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 19-15/16 | 20-7/8 | 21-13/16 | 22-3/4 | 23-11/16 | 24-5/8 | 25-9/16 | 26-1/2 | 27-7/16 | 28-3/8 |
| 3/8” Bars | 20-1/16 | 21 | 21-15/16 | 22-7/8 | 23-13/16 | 24-3/4 | 25-11/16 | 26-5/8 | 27-9/16 | 28-1/2 |

56                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 15HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 39.41 | 166 | U Du C Dc | 87,111 0.01 43,556 0.00 | 38,716 0.01 29,037 0.01 | 21,778 0.02 21,778 0.02 | 13,938 0.04 17,422 0.03 | 9,679 0.05 14,519 0.04 | 7,111 0.07 12,444 0.06 | 5,444 0.09 10,889 0.08 | 4,302 0.12 9,679 0.10 | 3,484 0.15 8,711 0.12 | 2,880 0.18 7,919 0.14 | 2,420 0.21 7,259 0.17 | 2,062 0.25 6,701 0.20 | 1,778 0.29 6,222 0.23 | 1,549 0.33 5,807 0.27 | 1,361 0.38 5,444 0.30 | 6.533 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.433 |
| 3 ½" x ⅜" | 59.17 | 184 | U Du C Dc | 130,667 0.01 65,333 0.00 | 58,074 0.01 43,556 0.01 | 32,667 0.02 32,667 0.02 | 20,907 0.04 26,133 0.03 | 14,519 0.05 21,778 0.04 | 10,667 0.07 18,667 0.06 | 8,167 0.09 16,333 0.08 | 6,453 0.12 14,519 0.10 | 5,227 0.15 13,067 0.12 | 4,320 0.18 11,879 0.14 | 3,630 0.21 10,889 0.17 | 3,093 0.25 10,051 0.20 | 2,667 0.29 9,333 0.23 | 2,323 0.33 8,711 0.27 | 2,042 0.38 8,167 0.30 | 9.800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17.150 |
| 4" x¼" | 44.85 | 184 | U Du C Dc | 113,778 0.01 56,889 0.00 | 50,568 0.01 37,926 0.01 | 28,444 0.02 28,444 0.02 | 18,204 0.03 22,756 0.03 | 12,642 0.05 18,963 0.04 | 9,288 0.06 16,254 0.05 | 7,111 0.08 14,222 0.07 | 5,619 0.10 12,642 0.08 | 4,551 0.13 11,378 0.10 | 3,761 0.16 10,343 0.13 | 3,160 0.19 9,481 0.15 | 2,693 0.22 8,752 0.17 | 2,322 0.25 8,127 0.20 | 2,023 0.29 7,585 0.23 | 1,778 0.33 7,111 0.26 | 8.533 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17.067 |
| 4" x ⅜" | 67.33 | 203 | U Du C Dc | 170,667 0.01 85,333 0.00 | 75,852 0.01 56,889 0.01 | 42,667 0.02 42,667 0.02 | 27,307 0.03 34,133 0.03 | 18,963 0.05 28,444 0.04 | 13,932 0.06 24,381 0.05 | 10,667 0.08 21,333 0.07 | 8,428 0.10 18,963 0.08 | 6,827 0.13 17,067 0.10 | 5,642 0.16 15,515 0.13 | 4,741 0.19 14,222 0.15 | 4,039 0.22 13,128 0.17 | 3,483 0.25 12,190 0.20 | 3,034 0.29 11,378 0.23 | 2,667 0.33 10,667 0.26 | 12.800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25.600 |
| 5" x ⅜" | 83.65 | 240 | U Du C Dc | 266,667 0.00 133,333 0.00 | 118,519 0.01 88,889 0.01 | 66,667 0.02 66,667 0.01 | 42,667 0.03 53,333 0.02 | 29,630 0.04 44,444 0.03 | 21,769 0.05 38,095 0.04 | 16,667 0.07 33,333 0.05 | 13,169 0.08 29,630 0.07 | 10,667 0.10 26,667 0.08 | 8,815 0.13 24,242 0.10 | 7,407 0.15 22,222 0.12 | 6,312 0.17 20,513 0.14 | 5,442 0.20 19,048 0.16 | 4,741 0.23 17,778 0.19 | 4,167 0.26 16,667 0.21 | 20.000 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50.000 |
| 6" x ⅜" | 99.97 | 276 | U Du C Dc | 384,000 0.00 192,000 0.00 | 170,667 0.01 128,000 0.01 | 96,000 0.01 96,000 0.01 | 61,440 0.02 76,800 0.02 | 42,667 0.03 64,000 0.02 | 31,347 0.04 54,857 0.03 | 24,000 0.06 48,000 0.04 | 18,963 0.07 42,667 0.06 | 15,360 0.09 38,400 0.07 | 12,694 0.10 34,909 0.08 | 10,667 0.12 32,000 0.10 | 9,089 0.15 29,538 0.12 | 7,837 0.17 27,429 0.14 | 6,827 0.19 25,600 0.16 | 6,000 0.22 24,000 0.18 | 28.800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 86.400 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

15HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-3/16 | 2-1/8 | 3-1/16 | 4 | 4-15/16 | 5-7/8 | 6-13/16 | 7-3/4 | 8-11/16 | 9-5/8 |
| 3/8” Bars | 1-5/16 | 2-1/4 | 3-3/16 | 4-1/8 | 5-1/16 | 6 | 6-15/16 | 7-7/8 | 8-13/16 | 9-3/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 10-9/16 | 11-1/2 | 12-7/16 | 13-3/8 | 14-5/16 | 15-1/4 | 16-3/16 | 17-1/8 | 18-1/16 | 19 |
| 3/8” Bars | 10-11/16 | 11-5/8 | 12-9/16 | 13-1/2 | 14-7/16 | 15-3/8 | 16-5/16 | 17-1/4 | 18-3/16 | 19-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 19-15/16 | 20-7/8 | 21-13/16 | 22-3/4 | 23-11/16 | 24-5/8 | 25-9/16 | 26-1/2 | 27-7/16 | 28-3/8 |
| 3/8” Bars | 20-1/16 | 21 | 21-15/16 | 22-7/8 | 23-13/16 | 24-3/4 | 25-11/16 | 26-5/8 | 27-9/16 | 28-1/2 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               57

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 15HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 12.21 | 52 | U Du C Dc | 4,000 0.03 2,000 0.02 | 1,778 0.06 1,333 0.05 | 1,000 0.11 1,000 0.09 | 640 0.17 800 0.14 | 444 0.25 667 0.20 | 327 0.34 571 0.27 | 250 0.44 500 0.35 | 198 0.56 444 0.45 |  |  |  |  |  |  |  | 0.300 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.113 |
| 1 ¼" x ¼" | 14.93 | 65 | U Du C Dc | 7,111 0.02 3,556 0.02 | 3,160 0.05 2,370 0.04 | 1,778 0.08 1,778 0.07 | 1,138 0.13 1,422 0.10 | 790 0.19 1,185 0.15 | 580 0.25 1,016 0.20 | 444 351 0.33 0.42 889 790 0.26 0.34 |  | 284 0.52 711 0.41 | 235 0.63 646 0.50 |  |  |  |  |  | 0.533 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.267 |
| 1 ½" x ¼" | 17.65 | 77 | U Du C Dc | 11,111 0.02 5,556 0.01 | 4,938 0.04 3,704 0.03 | 2,778 0.07 2,778 0.05 | 1,778 0.10 2,222 0.08 | 1,235 0.15 1,852 0.12 | 907 0.20 1,587 0.16 | 694 549 0.26 0.34 1,389 1,235 0.21 0.27 |  | 444 0.41 1,111 0.33 | 367 0.50 1,010 0.40 | 309 0.60 926 0.48 | 263 0.70 855 0.56 |  |  |  | 0.833 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.521 |
| 1 ½" x ⅜" | 25.98 | 85 | U Du C Dc | 16,667 0.02 8,333 0.01 | 7,407 0.04 5,556 0.03 | 4,167 0.07 4,167 0.05 | 2,667 0.10 3,333 0.08 | 1,852 0.15 2,778 0.12 | 1,361 0.20 2,381 0.16 | 1,042 823 0.26 0.34 2,083 1,852 0.21 0.27 |  | 667 0.41 1,667 0.33 | 551 0.50 1,515 0.40 | 463 0.60 1,389 0.48 | 394 0.70 1,282 0.56 | 340 0.81 1,190 0.65 | 296 0.93 1,111 0.74 |  | 1.250 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.781 |
| 2" x ¼" | 23.09 | 99 | U Du C Dc | 21,778 0.01 10,889 0.01 | 9,679 0.03 7,259 0.02 | 5,444 0.05 5,444 0.04 | 3,484 0.07 4,356 0.06 | 2,420 0.11 3,630 0.09 | 1,778 0.14 3,111 0.12 | 1,361 1,075 0.19 0.24 2,722 2,420 0.15 0.19 |  | 871 0.30 2,178 0.24 | 720 0.36 1,980 0.29 | 605 0.43 1,815 0.34 | 515 0.50 1,675 0.40 | 444 0.58 1,556 0.46 | 387 0.67 1,452 0.53 | 340 0.76 1,361 0.61 | 1.633 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.429 |
| 2 ½" x ¼" | 28.53 | 119 | U Du C Dc | 36,000 0.01 18,000 0.01 | 16,000 0.02 12,000 0.02 | 9,000 0.04 9,000 0.03 | 5,760 0.06 7,200 0.05 | 4,000 0.08 6,000 0.07 | 2,939 0.11 5,143 0.09 | 2,250 1,778 0.15 0.19 4,500 4,000 0.12 0.15 |  | 1,440 0.23 3,600 0.18 | 1,190 0.28 3,273 0.22 | 1,000 0.33 3,000 0.26 | 852 0.39 2,769 0.31 | 735 0.45 2,571 0.36 | 640 0.52 2,400 0.41 | 563 0.59 2,250 0.47 | 2.700 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.038 |
| 3" x ¼" | 33.97 | 139 | U Du C Dc | 53,778 0.01 26,889 0.01 | 23,901 0.02 17,926 0.01 | 13,444 0.03 13,444 0.02 | 8,604 0.05 10,756 0.04 | 5,975 0.07 8,963 0.05 | 4,390 0.09 7,683 0.07 | 3,361 2,656 0.12 0.15 6,722 5,975 0.10 0.12 |  | 2,151 0.19 5,378 0.15 | 1,778 0.23 4,889 0.18 | 1,494 0.27 4,481 0.22 | 1,273 0.32 4,137 0.25 | 1,098 0.37 3,841 0.29 | 956 0.42 3,585 0.34 | 840 0.48 3,361 0.39 | 4.033 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.546 |
| 3" x ⅜" | 50.46 | 154 | U Du C Dc | 80,667 0.01 40,333 0.01 | 35,852 0.02 26,889 0.01 | 20,167 0.03 20,167 0.02 | 12,907 0.05 16,133 0.04 | 8,963 0.07 13,444 0.05 | 6,585 0.09 11,524 0.07 | 5,042 3,984 0.12 0.15 10,083 8,963 0.10 0.12 |  | 3,227 0.19 8,067 0.15 | 2,667 0.23 7,333 0.18 | 2,241 0.27 6,722 0.22 | 1,909 0.32 6,205 0.25 | 1,646 0.37 5,762 0.29 | 1,434 0.42 5,378 0.34 | 1,260 0.48 5,042 0.39 | 6.050 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.319 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

15HW4 (in)

# of Bars           2                     3                   4                   5                   6                   7                     8                    9                    10                    11

1/4” Bars

3/8” Bars

| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 10-9/16 | 11-1/2 | 12-7/16 | 13-3/8 | 14-5/16 | 15-1/4 | 16-3/16 | 17-1/8 | 18-1/16 | 19 |
| 3/8” Bars | 10-11/16 | 11-5/8 | 12-9/16 | 13-1/2 | 14-7/16 | 15-3/8 | 16-5/16 | 17-1/4 | 18-3/16 | 19-1/8 |

6-13/16

6-15/16

8-11/16

8-13/16

4-15/16

5-1/16

3-1/16

3-3/16

1-3/16

1-5/16

9-5/8

9-3/4

5-7/8

6

7-3/4

7-7/8

2-1/8

2-1/4

4

4-1/8

1/4” Bars       19-15/16

23-11/16 0.05

23-13/16

20-7/8

25-9/16 0.13

25-11/16

21-13/16

26-1/2 0.18

26-5/8

281-83/0850

28-1/2

22-3/4

24-5/8 0.09

24-3/4

27-7/16 0.25

27-9/16

D

0.28

0.00

0.01          0.02

0.03          0.04

0.07

0.11

0.21

0.16

# of Bars          22                 23                 24                 25                 26                 27                 28                 29                 30                  31

LOAD TABLES - HD IMPERIAL

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 15HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 39.41 | 157 | U Du C Dc | 75,111 0.01 37,556 0.01 | 33,383 0.01 25,037 0.01 | 18,778 0.03 18,778 0.02 | 12,018 0.04 15,022 0.03 | 8,346 0.06 12,519 0.05 | 6,132 0.08 10,730 0.06 | 4,694 0.10 9,389 0.08 | 3,709 0.13 8,346 0.10 | 3,004 0.16 7,511 0.13 | 2,483 0.19 6,828 0.15 | 2,086 0.23 6,259 0.18 | 1,778 0.27 5,778 0.22 | 1,533 0.31 5,365 0.25 | 1,335 0.36 5,007 0.29 | 1,174 0.41 4,694 0.33 | 5.633 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.154 |
| 3 ½" x ⅜" | 59.17 | 174 | U Du C Dc | 112,667 0.01 56,333 0.01 | 50,074 0.01 37,556 0.01 | 28,167 0.03 28,167 0.02 | 18,027 0.04 22,533 0.03 | 12,519 0.06 18,778 0.05 | 9,197 0.08 16,095 0.06 | 7,042 0.10 14,083 0.08 | 5,564 0.13 12,519 0.10 | 4,507 0.16 11,267 0.13 | 3,725 0.19 10,242 0.15 | 3,130 0.23 9,389 0.18 | 2,667 0.27 8,667 0.22 | 2,299 0.31 8,048 0.25 | 2,003 0.36 7,511 0.29 | 1,760 0.41 7,042 0.33 | 8.450 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.731 |
| 4" x¼" | 44.85 | 175 | U Du C Dc | 100,000 0.01 50,000 0.00 | 44,444 0.01 33,333 0.01 | 25,000 0.02 25,000 0.02 | 16,000 0.03 20,000 0.03 | 11,111 0.05 16,667 0.04 | 8,163 0.07 14,286 0.05 | 6,250 0.09 12,500 0.07 | 4,938 0.11 11,111 0.09 | 4,000 0.14 10,000 0.11 | 3,306 0.17 9,091 0.13 | 2,778 0.20 8,333 0.16 | 2,367 0.23 7,692 0.19 | 2,041 0.27 7,143 0.22 | 1,778 0.31 6,667 0.25 | 1,563 0.35 6,250 0.28 | 7.500 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.063 |
| 4" x ⅜" | 67.33 | 194 | U Du C Dc | 150,000 0.01 75,000 0.00 | 66,667 0.01 50,000 0.01 | 37,500 0.02 37,500 0.02 | 24,000 0.03 30,000 0.03 | 16,667 0.05 25,000 0.04 | 12,245 0.07 21,429 0.05 | 9,375 0.09 18,750 0.07 | 7,407 0.11 16,667 0.09 | 6,000 0.14 15,000 0.11 | 4,959 0.17 13,636 0.13 | 4,167 0.20 12,500 0.16 | 3,550 0.23 11,538 0.19 | 3,061 0.27 10,714 0.22 | 2,667 0.31 10,000 0.25 | 2,344 0.35 9,375 0.28 | 11.250 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21.094 |
| 5" x ⅜" | 83.65 | 231 | U Du C Dc | 240,667 0.00 120,333 0.00 | 106,963 0.01 80,222 0.01 | 60,167 0.02 60,167 0.01 | 38,507 0.03 48,133 0.02 | 26,741 0.04 40,111 0.03 | 19,646 0.05 34,381 0.04 | 15,042 0.07 30,083 0.06 | 11,885 0.09 26,741 0.07 | 9,627 0.11 24,067 0.09 | 7,956 0.13 21,879 0.11 | 6,685 0.16 20,056 0.13 | 5,696 0.18 18,513 0.15 | 4,912 0.21 17,190 0.17 | 4,279 0.25 16,044 0.20 | 3,760 0.28 15,042 0.22 | 18.050 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 42.869 |
| 6" x ⅜" | 99.97 | 267 | U Du C Dc | 352,667 0.00 176,333 0.00 | 156,741 0.01 117,556 0.01 | 88,167 0.01 88,167 0.01 | 56,427 0.02 70,533 0.02 | 39,185 0.03 58,778 0.03 | 28,789 0.04 50,381 0.04 | 22,042 0.06 44,083 0.05 | 17,416 0.07 39,185 0.06 | 14,107 0.09 35,267 0.07 | 11,658 0.11 32,061 0.09 | 9,796 0.13 29,389 0.10 | 8,347 0.15 27,128 0.12 | 7,197 0.18 25,190 0.14 | 6,270 0.20 23,511 0.16 | 5,510 0.23 22,042 0.18 | 26.450 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 76.044 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

15HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 1-3/16 | 2-1/8 | 3-1/16 | 4 | 4-15/16 | 5-7/8 | 6-13/16 | 7-3/4 | 8-11/16 | 9-5/8 |
| 3/8” Bars | 1-5/16 | 2-1/4 | 3-3/16 | 4-1/8 | 5-1/16 | 6 | 6-15/16 | 7-7/8 | 8-13/16 | 9-3/4 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 10-9/16 | 11-1/2 | 12-7/16 | 13-3/8 | 14-5/16 | 15-1/4 | 16-3/16 | 17-1/8 | 18-1/16 | 19 |
| 3/8” Bars | 10-11/16 | 11-5/8 | 12-9/16 | 13-1/2 | 14-7/16 | 15-3/8 | 16-5/16 | 17-1/4 | 18-3/16 | 19-1/8 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 1/4” Bars | 19-15/16 | 20-7/8 | 21-13/16 | 22-3/4 | 23-11/16 | 24-5/8 | 25-9/16 | 26-1/2 | 27-7/16 | 28-3/8 |
| 3/8” Bars | 20-1/16 | 21 | 21-15/16 | 22-7/8 | 23-13/16 | 24-3/4 | 25-11/16 | 26-5/8 | 27-9/16 | 28-1/2 |

18.050

42.869

26.450

76.044

D                   0.00          0.01          0.02          0.03          0.04          0.05          0.07          0.09          0.11          0.13          0.16          0.18         0.21          0.25          0.28

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.06          0.07          0.09         0.11          0.13          0.15          0.17          0.20          0.22

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.06          0.07          0.09          0.11          0.13          0.15         0.18          0.20          0.23

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.05          0.06          0.07         0.09          0.10          0.12          0.14          0.16          0.18

5" x ⅜"          83.65          231

5" x ⅜"          83.65          231

22-7/8

21

21-15/16

3/8” Bars        20-1/16

42.869

26.450

76.044

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.06          0.07          0.09         0.11          0.13          0.15          0.17          0.20          0.22

6" x ⅜"          99.97          267

6" x ⅜"          99.97          267

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.06          0.07          0.09          0.11          0.13          0.15         0.18          0.20          0.23

D                   0.00          0.01          0.01          0.02          0.03          0.04          0.05          0.06          0.07         0.09          0.10          0.12          0.14          0.16          0.18

58                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              59

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - WIDE-GAP

| Grating Type: 30HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 6.77 | 55 | U Du C Dc | 3,556 0.02 1,778 0.02 | 1,580 0.05 1,185 0.04 | 889 0.08 889 0.07 | 569 0.13 711 0.10 | 395 0.19 593 0.15 | 290 0.25 508 0.20 | 222 0.33 444 0.26 | 176 0.42 395 0.34 | 142 0.52 356 0.41 |  |  |  |  |  |  | 0.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.133 |
| 1 ¼" x ¼" | 8.13 | 65 | U Du C Dc | 5,556 0.02 2,778 0.01 | 2,469 0.04 1,852 0.03 | 1,389 0.07 1,389 0.05 | 889 0.10 1,111 0.08 | 617 0.15 926 0.12 | 454 0.20 794 0.16 | 347 0.26 694 0.21 | 274 222 0.34 0.41 617 556 0.27 0.33 |  | 184 0.50 505 0.40 |  |  |  |  |  | 0.417 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| 1 ½" x ¼" | 9.49 | 74 | U Du C Dc | 8,000 0.01 4,000 0.01 | 3,556 0.03 2,667 0.02 | 2,000 0.06 2,000 0.04 | 1,280 0.09 1,600 0.07 | 889 0.12 1,333 0.10 | 653 0.17 1,143 0.14 | 500 0.22 1,000 0.18 | 395 320 0.28 0.34 889 800 0.22 0.28 |  | 264 0.42 727 0.33 | 222 0.50 667 0.40 | 189 0.58 615 0.47 |  |  |  | 0.600 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.450 |
| 1 ½" x ⅜" | 13.74 | 82 | U Du C Dc | 12,000 0.01 6,000 0.01 | 5,333 0.03 4,000 0.02 | 3,000 0.06 3,000 0.04 | 1,920 0.09 2,400 0.07 | 1,333 0.12 2,000 0.10 | 980 0.17 1,714 0.14 | 750 0.22 1,500 0.18 | 593 480 0.28 0.34 1,333 1,200 0.22 0.28 |  | 397 0.42 1,091 0.33 | 333 0.50 1,000 0.40 | 284 0.58 923 0.47 | 245 0.68 857 0.54 |  |  | 0.900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.675 |
| 2" x ¼" | 12.21 | 92 | U Du C Dc | 14,222 0.01 7,111 0.01 | 6,321 0.02 4,741 0.02 | 3,556 0.04 3,556 0.03 | 2,276 0.06 2,844 0.05 | 1,580 0.09 2,370 0.07 | 1,161 0.13 2,032 0.10 | 889 0.17 1,778 0.13 | 702 569 0.21 0.26 1,580 1,422 0.17 0.21 |  | 470 0.31 1,293 0.25 | 395 0.37 1,185 0.30 | 337 0.44 1,094 0.35 | 290 0.51 1,016 0.41 | 253 0.58 948 0.47 | 222 0.66 889 0.53 | 1.067 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.067 |
| 2 ½" x ¼" | 14.93 | 109 | U Du C Dc | 22,222 0.01 11,111 0.01 | 9,877 0.02 7,407 0.01 | 5,556 0.03 5,556 0.03 | 3,556 0.05 4,444 0.04 | 2,469 0.07 3,704 0.06 | 1,814 0.10 3,175 0.08 | 1,389 0.13 2,778 0.11 | 1,097 889 0.17 0.21 2,469 2,222 0.13 0.17 |  | 735 0.25 2,020 0.20 | 617 0.30 1,852 0.24 | 526 0.35 1,709 0.28 | 454 0.41 1,587 0.32 | 395 0.47 1,481 0.37 | 347 0.53 1,389 0.42 | 1.667 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.083 |
| 3" x ¼" | 17.65 | 125 | U Du C Dc | 32,000 0.01 16,000 0.01 | 14,222 0.02 10,667 0.01 | 8,000 0.03 8,000 0.02 | 5,120 0.04 6,400 0.03 | 3,556 0.06 5,333 0.05 | 2,612 0.08 4,571 0.07 | 2,000 0.11 4,000 0.09 | 1,580 1,280 0.14 0.17 3,556 3,200 0.11 0.14 |  | 1,058 0.21 2,909 0.17 | 889 0.25 2,667 0.20 | 757 0.29 2,462 0.23 | 653 0.34 2,286 0.27 | 569 0.39 2,133 0.31 | 500 0.44 2,000 0.35 | 2.400 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.600 |
| 3" x ⅜" | 26.53 | 138 | U Du C Dc | 48,000 0.01 24,000 0.01 | 21,333 0.02 16,000 0.01 | 12,000 0.03 12,000 0.02 | 7,680 0.04 9,600 0.03 | 5,333 0.06 8,000 0.05 | 3,918 0.08 6,857 0.07 | 3,000 0.11 6,000 0.09 | 2,370 1,920 0.14 0.17 5,333 4,800 0.11 0.14 |  | 1,587 0.21 4,364 0.17 | 1,333 0.25 4,000 0.20 | 1,136 0.29 3,692 0.23 | 980 0.34 3,429 0.27 | 853 0.39 3,200 0.31 | 750 0.44 3,000 0.35 | 3.600 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.400 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 30HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-1/8 | 4 | 5-7/8 | 7-3/4 | 9-5/8 | 11-1/2 | 13-3/8 | 15-1/4 | 17-1/8 | 19 |
| 3/8” Bars | 2-1/4 | 4-1/8 | 6 | 7-7/8 | 9-3/4 | 11-5/8 | 13-1/2 | 15-3/8 | 17-1/4 | 19-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 20-7/8 | 22-3/4 | 24-5/8 | 26-1/2 | 28-3/8 | 30-1/4 | 32-1/8 | 34 | 35-7/8 | 37-3/4 |
| 3/8” Bars | 21 | 22-7/8 | 24-3/4 | 26-5/8 | 28-1/2 | 30-3/8 | 32-1/4 | 34-1/8 | 36 | 37-7/8 |

60                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - WIDE-GAP

| Grating Type: 30HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 20.37 | 140 | U Du C Dc | 43,556 0.01 21,778 0.00 | 19,358 0.01 14,519 0.01 | 10,889 0.02 10,889 0.02 | 6,969 0.04 8,711 0.03 | 4,840 0.05 7,259 0.04 | 3,556 0.07 6,222 0.06 | 2,722 0.09 5,444 0.08 | 2,151 0.12 4,840 0.10 | 1,742 0.15 4,356 0.12 | 1,440 0.18 3,960 0.14 | 1,210 0.21 3,630 0.17 | 1,031 0.25 3,350 0.20 | 889 0.29 3,111 0.23 | 774 0.33 2,904 0.27 | 681 0.38 2,722 0.30 | 3.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.717 |
| 3 ½" x ⅜" | 30.61 | 155 | U Du C Dc | 65,333 0.01 32,667 0.00 | 29,037 0.01 21,778 0.01 | 16,333 0.02 16,333 0.02 | 10,453 0.04 13,067 0.03 | 7,259 0.05 10,889 0.04 | 5,333 0.07 9,333 0.06 | 4,083 0.09 8,167 0.08 | 3,226 0.12 7,259 0.10 | 2,613 0.15 6,533 0.12 | 2,160 0.18 5,939 0.14 | 1,815 0.21 5,444 0.17 | 1,546 0.25 5,026 0.20 | 1,333 0.29 4,667 0.23 | 1,161 0.33 4,356 0.27 | 1,021 0.38 4,083 0.30 | 4.900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.575 |
| 4" x¼" | 23.09 | 155 | U Du C Dc | 56,889 0.01 28,444 0.00 | 25,284 0.01 18,963 0.01 | 14,222 0.02 14,222 0.02 | 9,102 0.03 11,378 0.03 | 6,321 0.05 9,481 0.04 | 4,644 0.06 8,127 0.05 | 3,556 0.08 7,111 0.07 | 2,809 0.10 6,321 0.08 | 2,276 0.13 5,689 0.10 | 1,881 0.16 5,172 0.13 | 1,580 0.19 4,741 0.15 | 1,346 0.22 4,376 0.17 | 1,161 0.25 4,063 0.20 | 1,011 0.29 3,793 0.23 | 889 0.33 3,556 0.26 | 4.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.533 |
| 4" x ⅜" | 34.69 | 171 | U Du C Dc | 85,333 0.01 42,667 0.00 | 37,926 0.01 28,444 0.01 | 21,333 0.02 21,333 0.02 | 13,653 0.03 17,067 0.03 | 9,481 0.05 14,222 0.04 | 6,966 0.06 12,190 0.05 | 5,333 0.08 10,667 0.07 | 4,214 0.10 9,481 0.08 | 3,413 0.13 8,533 0.10 | 2,821 0.16 7,758 0.13 | 2,370 0.19 7,111 0.15 | 2,020 0.22 6,564 0.17 | 1,741 0.25 6,095 0.20 | 1,517 0.29 5,689 0.23 | 1,333 0.33 5,333 0.26 | 6.400 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.800 |
| 5" x ⅜" | 42.85 | 202 | U Du C Dc | 133,333 0.00 66,667 0.00 | 59,259 0.01 44,444 0.01 | 33,333 0.02 33,333 0.01 | 21,333 0.03 26,667 0.02 | 14,815 0.04 22,222 0.03 | 10,884 0.05 19,048 0.04 | 8,333 0.07 16,667 0.05 | 6,584 0.08 14,815 0.07 | 5,333 0.10 13,333 0.08 | 4,408 0.13 12,121 0.10 | 3,704 0.15 11,111 0.12 | 3,156 0.17 10,256 0.14 | 2,721 0.20 9,524 0.16 | 2,370 0.23 8,889 0.19 | 2,083 0.26 8,333 0.21 | 10.000 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25.000 |
| 6" x ⅜" | 51.01 | 232 | U Du C Dc | 192,000 0.00 96,000 0.00 | 85,333 0.01 64,000 0.01 | 48,000 0.01 48,000 0.01 | 30,720 0.02 38,400 0.02 | 21,333 0.03 32,000 0.02 | 15,673 0.04 27,429 0.03 | 12,000 0.06 24,000 0.04 | 9,481 0.07 21,333 0.06 | 7,680 0.09 19,200 0.07 | 6,347 0.10 17,455 0.08 | 5,333 0.12 16,000 0.10 | 4,544 0.15 14,769 0.12 | 3,918 0.17 13,714 0.14 | 3,413 0.19 12,800 0.16 | 3,000 0.22 12,000 0.18 | 14.400 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 43.200 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

30HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-1/8 | 4 | 5-7/8 | 7-3/4 | 9-5/8 | 11-1/2 | 13-3/8 | 15-1/4 | 17-1/8 | 19 |
| 3/8” Bars | 2-1/4 | 4-1/8 | 6 | 7-7/8 | 9-3/4 | 11-5/8 | 13-1/2 | 15-3/8 | 17-1/4 | 19-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 20-7/8 | 22-3/4 | 24-5/8 | 26-1/2 | 28-3/8 | 30-1/4 | 32-1/8 | 34 | 35-7/8 | 37-3/4 |
| 3/8” Bars | 21 | 22-7/8 | 24-3/4 | 26-5/8 | 28-1/2 | 30-3/8 | 32-1/4 | 34-1/8 | 36 | 37-7/8 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               61

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - WIDE-GAP

| Grating Type: 30HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 6.77 | 44 | U Du C Dc | 2,000 0.03 1,000 0.02 | 889 0.06 667 0.05 | 500 0.11 500 0.09 | 320 0.17 400 0.14 | 222 0.25 333 0.20 | 163 0.34 286 0.27 | 125 0.44 250 0.35 |  |  |  |  |  |  |  |  | 0.150 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.056 |
| 1 ¼" x ¼" | 8.13 | 55 | U Du C Dc | 3,556 0.02 1,778 0.02 | 1,580 0.05 1,185 0.04 | 889 0.08 889 0.07 | 569 0.13 711 0.10 | 395 0.19 593 0.15 | 290 222 0.25 0.33 508 444 0.20 0.26 |  | 176 0.42 395 0.34 | 142 0.52 356 0.41 |  |  |  |  |  |  | 0.267 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.133 |
| 1 ½" x ¼" | 9.49 | 65 | U Du C Dc | 5,556 0.02 2,778 0.01 | 2,469 0.04 1,852 0.03 | 1,389 0.07 1,389 0.05 | 889 0.10 1,111 0.08 | 617 0.15 926 0.12 | 454 347 0.20 0.26 794 694 0.16 0.21 |  | 274 0.34 617 0.27 | 222 0.41 556 0.33 | 184 0.50 505 0.40 |  |  |  |  |  | 0.417 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.260 |
| 1 ½" x ⅜" | 13.74 | 71 | U Du C Dc | 8,333 0.02 4,167 0.01 | 3,704 0.04 2,778 0.03 | 2,083 0.07 2,083 0.05 | 1,333 0.10 1,667 0.08 | 926 0.15 1,389 0.12 | 680 521 0.20 0.26 1,190 1,042 0.16 0.21 |  | 412 0.34 926 0.27 | 333 0.41 833 0.33 | 275 0.50 758 0.40 | 231 0.60 694 0.48 |  |  |  |  | 0.625 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.391 |
| 2" x ¼" | 12.21 | 83 | U Du C Dc | 10,889 0.01 5,444 0.01 | 4,840 0.03 3,630 0.02 | 2,722 0.05 2,722 0.04 | 1,742 0.07 2,178 0.06 | 1,210 0.11 1,815 0.09 | 889 681 0.14 0.19 1,556 1,361 0.12 0.15 |  | 538 0.24 1,210 0.19 | 436 0.30 1,089 0.24 | 360 0.36 990 0.29 | 302 0.43 907 0.34 | 258 0.50 838 0.40 | 222 0.58 778 0.46 |  |  | 0.817 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.715 |
| 2 ½" x ¼" | 14.93 | 100 | U Du C Dc | 18,000 0.01 9,000 0.01 | 8,000 0.02 6,000 0.02 | 4,500 0.04 4,500 0.03 | 2,880 0.06 3,600 0.05 | 2,000 0.08 3,000 0.07 | 1,469 1,125 0.11 0.15 2,571 2,250 0.09 0.12 |  | 889 0.19 2,000 0.15 | 720 0.23 1,800 0.18 | 595 0.28 1,636 0.22 | 500 0.33 1,500 0.26 | 426 0.39 1,385 0.31 | 367 0.45 1,286 0.36 | 320 0.52 1,200 0.41 | 281 0.59 1,125 0.47 | 1.350 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.519 |
| 3" x ¼" | 17.65 | 117 | U Du C Dc | 26,889 0.01 13,444 0.01 | 11,951 0.02 8,963 0.01 | 6,722 0.03 6,722 0.02 | 4,302 0.05 5,378 0.04 | 2,988 0.07 4,481 0.05 | 2,195 1,681 0.09 0.12 3,841 3,361 0.07 0.10 |  | 1,328 0.15 2,988 0.12 | 1,076 0.19 2,689 0.15 | 889 0.23 2,444 0.18 | 747 0.27 2,241 0.22 | 636 0.32 2,068 0.25 | 549 0.37 1,921 0.29 | 478 0.42 1,793 0.34 | 420 0.48 1,681 0.39 | 2.017 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.773 |
| 3" x ⅜" | 25.98 | 129 | U Du C Dc | 40,333 0.01 20,167 0.01 | 17,926 0.02 13,444 0.01 | 10,083 0.03 10,083 0.02 | 6,453 0.05 8,067 0.04 | 4,481 0.07 6,722 0.05 | 3,293 2,521 0.09 0.12 5,762 5,042 0.07 0.10 |  | 1,992 0.15 4,481 0.12 | 1,613 0.19 4,033 0.15 | 1,333 0.23 3,667 0.18 | 1,120 0.27 3,361 0.22 | 955 0.32 3,103 0.25 | 823 0.37 2,881 0.29 | 717 0.42 2,689 0.34 | 630 0.48 2,521 0.39 | 3.025 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.159 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

30HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-1/8 | 4 | 5-7/8 | 7-3/4 | 9-5/8 | 11-1/2 | 13-3/8 | 15-1/4 | 17-1/8 | 19 |
| 3/8” Bars | 2-1/4 | 4-1/8 | 6 | 7-7/8 | 9-3/4 | 11-5/8 | 13-1/2 | 15-3/8 | 17-1/4 | 19-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 20-7/8 | 22-3/4 | 24-5/8 | 26-1/2 | 28-3/8 | 30-1/4 | 32-1/8 | 34 | 35-7/8 | 37-3/4 |
| 3/8” Bars | 21 | 22-7/8 | 24-3/4 | 26-5/8 | 28-1/2 | 30-3/8 | 32-1/4 | 34-1/8 | 36 | 37-7/8 |

62                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - WIDE-GAP

| Grating Type: 30HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 20.37 | 132 | U Du C Dc | 37,556 0.01 18,778 0.01 | 16,691 0.01 12,519 0.01 | 9,389 0.03 9,389 0.02 | 6,009 0.04 7,511 0.03 | 4,173 0.06 6,259 0.05 | 3,066 0.08 5,365 0.06 | 2,347 0.10 4,694 0.08 | 1,855 0.13 4,173 0.10 | 1,502 0.16 3,756 0.13 | 1,242 0.19 3,414 0.15 | 1,043 0.23 3,130 0.18 | 889 0.27 2,889 0.22 | 766 0.31 2,683 0.25 | 668 0.36 2,504 0.29 | 587 0.41 2,347 0.33 | 2.817 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.577 |
| 3 ½" x ⅜" | 30.61 | 146 | U Du C Dc | 56,333 0.01 28,167 0.01 | 25,037 0.01 18,778 0.01 | 14,083 0.03 14,083 0.02 | 9,013 0.04 11,267 0.03 | 6,259 0.06 9,389 0.05 | 4,599 0.08 8,048 0.06 | 3,521 0.10 7,042 0.08 | 2,782 0.13 6,259 0.10 | 2,253 0.16 5,633 0.13 | 1,862 0.19 5,121 0.15 | 1,565 0.23 4,694 0.18 | 1,333 0.27 4,333 0.22 | 1,150 0.31 4,024 0.25 | 1,001 0.36 3,756 0.29 | 880 0.41 3,521 0.33 | 4.225 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.866 |
| 4" x¼" | 23.09 | 147 | U Du C Dc | 50,000 0.01 25,000 0.00 | 22,222 0.01 16,667 0.01 | 12,500 0.02 12,500 0.02 | 8,000 0.03 10,000 0.03 | 5,556 0.05 8,333 0.04 | 4,082 0.07 7,143 0.05 | 3,125 0.09 6,250 0.07 | 2,469 0.11 5,556 0.09 | 2,000 0.14 5,000 0.11 | 1,653 0.17 4,545 0.13 | 1,389 0.20 4,167 0.16 | 1,183 0.23 3,846 0.19 | 1,020 0.27 3,571 0.22 | 889 0.31 3,333 0.25 | 781 0.35 3,125 0.28 | 3.750 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.031 |
| 4" x ⅜" | 34.69 | 163 | U Du C Dc | 75,000 0.01 37,500 0.00 | 33,333 0.01 25,000 0.01 | 18,750 0.02 18,750 0.02 | 12,000 0.03 15,000 0.03 | 8,333 0.05 12,500 0.04 | 6,122 0.07 10,714 0.05 | 4,688 0.09 9,375 0.07 | 3,704 0.11 8,333 0.09 | 3,000 0.14 7,500 0.11 | 2,479 0.17 6,818 0.13 | 2,083 0.20 6,250 0.16 | 1,775 0.23 5,769 0.19 | 1,531 0.27 5,357 0.22 | 1,333 0.31 5,000 0.25 | 1,172 0.35 4,688 0.28 | 5.625 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.547 |
| 5" x ⅜" | 42.85 | 195 | U Du C Dc | 120,333 0.00 60,167 0.00 | 53,481 0.01 40,111 0.01 | 30,083 0.02 30,083 0.01 | 19,253 0.03 24,067 0.02 | 13,370 0.04 20,056 0.03 | 9,823 0.05 17,190 0.04 | 7,521 0.07 15,042 0.06 | 5,942 0.09 13,370 0.07 | 4,813 0.11 12,033 0.09 | 3,978 0.13 10,939 0.11 | 3,343 0.16 10,028 0.13 | 2,848 0.18 9,256 0.15 | 2,456 0.21 8,595 0.17 | 2,139 0.25 8,022 0.20 | 1,880 0.28 7,521 0.22 | 9.025 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21.434 |
| 6" x ⅜" | 51.01 | 225 | U Du C Dc | 176,333 0.00 88,167 0.00 | 78,370 0.01 58,778 0.01 | 44,083 0.01 44,083 0.01 | 28,213 0.02 35,267 0.02 | 19,593 0.03 29,389 0.03 | 14,395 0.04 25,190 0.04 | 11,021 0.06 22,042 0.05 | 8,708 0.07 19,593 0.06 | 7,053 0.09 17,633 0.07 | 5,829 0.11 16,030 0.09 | 4,898 0.13 14,694 0.10 | 4,174 0.15 13,564 0.12 | 3,599 0.18 12,595 0.14 | 3,135 0.20 11,756 0.16 | 2,755 0.23 11,021 0.18 | 13.225 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 38.022 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

30HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-1/8 | 4 | 5-7/8 | 7-3/4 | 9-5/8 | 11-1/2 | 13-3/8 | 15-1/4 | 17-1/8 | 19 |
| 3/8” Bars | 2-1/4 | 4-1/8 | 6 | 7-7/8 | 9-3/4 | 11-5/8 | 13-1/2 | 15-3/8 | 17-1/4 | 19-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 20-7/8 | 22-3/4 | 24-5/8 | 26-1/2 | 28-3/8 | 30-1/4 | 32-1/8 | 34 | 35-7/8 | 37-3/4 |
| 3/8” Bars | 21 | 22-7/8 | 24-3/4 | 26-5/8 | 28-1/2 | 30-3/8 | 32-1/4 | 34-1/8 | 36 | 37-7/8 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              63

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 38HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 5.62 | 51 | U Du C Dc | 2,807 0.02 1,404 0.02 | 1,248 0.05 936 0.04 | 702 0.08 702 0.07 | 449 0.13 561 0.10 | 312 0.19 468 0.15 | 229 0.25 401 0.20 | 175 0.33 351 0.26 | 139 0.42 312 0.34 |  |  |  |  |  |  |  | 0.211 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.105 |
| 1 ¼" x ¼" | 6.70 | 61 | U Du C Dc | 4,386 0.02 2,193 0.01 | 1,949 0.04 1,462 0.03 | 1,096 0.07 1,096 0.05 | 702 0.10 877 0.08 | 487 0.15 731 0.12 | 358 0.20 627 0.16 | 274 217 0.26 0.34 548 487 0.21 0.27 |  | 175 0.41 439 0.33 | 145 0.50 399 0.40 |  |  |  |  |  | 0.329 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.206 |
| 1 ½" x ¼" | 7.77 | 70 | U Du C Dc | 6,316 0.01 3,158 0.01 | 2,807 0.03 2,105 0.02 | 1,579 0.06 1,579 0.04 | 1,011 0.09 1,263 0.07 | 702 0.12 1,053 0.10 | 516 0.17 902 0.14 | 395 312 0.22 0.28 789 702 0.18 0.22 |  | 253 0.34 632 0.28 | 209 0.42 574 0.33 | 175 0.50 526 0.40 |  |  |  |  | 0.474 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.355 |
| 1 ½" x ⅜" | 11.17 | 77 | U Du C Dc | 9,474 0.01 4,737 0.01 | 4,211 0.03 3,158 0.02 | 2,368 0.06 2,368 0.04 | 1,516 0.09 1,895 0.07 | 1,053 0.12 1,579 0.10 | 773 0.17 1,353 0.14 | 592 468 0.22 0.28 1,184 1,053 0.18 0.22 |  | 379 0.34 947 0.28 | 313 0.42 861 0.33 | 263 0.50 789 0.40 | 224 0.58 729 0.47 |  |  |  | 0.711 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.533 |
| 2" x ¼" | 9.92 | 87 | U Du C Dc | 11,228 0.01 5,614 0.01 | 4,990 0.02 3,743 0.02 | 2,807 0.04 2,807 0.03 | 1,796 0.06 2,246 0.05 | 1,248 0.09 1,871 0.07 | 917 0.13 1,604 0.10 | 702 554 0.17 0.21 1,404 1,248 0.13 0.17 |  | 449 0.26 1,123 0.21 | 371 0.31 1,021 0.25 | 312 0.37 936 0.30 | 266 0.44 864 0.35 | 229 0.51 802 0.41 | 200 0.58 749 0.47 |  | 0.842 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.842 |
| 2 ½" x ¼" | 12.06 | 102 | U Du C Dc | 17,544 0.01 8,772 0.01 | 7,797 0.02 5,848 0.01 | 4,386 0.03 4,386 0.03 | 2,807 0.05 3,509 0.04 | 1,949 0.07 2,924 0.06 | 1,432 0.10 2,506 0.08 | 1,096 866 0.13 0.17 2,193 1,949 0.11 0.13 |  | 702 0.21 1,754 0.17 | 580 0.25 1,595 0.20 | 487 0.30 1,462 0.24 | 415 0.35 1,350 0.28 | 358 0.41 1,253 0.32 | 312 0.47 1,170 0.37 | 274 0.53 1,096 0.42 | 1.316 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.645 |
| 3" x ¼" | 14.21 | 117 | U Du C Dc | 25,263 0.01 12,632 0.01 | 11,228 0.02 8,421 0.01 | 6,316 0.03 6,316 0.02 | 4,042 0.04 5,053 0.03 | 2,807 0.06 4,211 0.05 | 2,062 0.08 3,609 0.07 | 1,579 1,248 0.11 0.14 3,158 2,807 0.09 0.11 |  | 1,011 0.17 2,526 0.14 | 835 0.21 2,297 0.17 | 702 0.25 2,105 0.20 | 598 0.29 1,943 0.23 | 516 0.34 1,805 0.27 | 449 0.39 1,684 0.31 | 395 0.44 1,579 0.35 | 1.895 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.842 |
| 3" x ⅜" | 21.37 | 130 | U Du C Dc | 37,895 0.01 18,947 0.01 | 16,842 0.02 12,632 0.01 | 9,474 0.03 9,474 0.02 | 6,063 0.04 7,579 0.03 | 4,211 0.06 6,316 0.05 | 3,093 0.08 5,414 0.07 | 2,368 1,871 0.11 0.14 4,737 4,211 0.09 0.11 |  | 1,516 0.17 3,789 0.14 | 1,253 0.21 3,445 0.17 | 1,053 0.25 3,158 0.20 | 897 0.29 2,915 0.23 | 773 0.34 2,707 0.27 | 674 0.39 2,526 0.31 | 592 0.44 2,368 0.35 | 2.842 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.263 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 38HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-5/8 | 5 | 7-3/8 | 9-3/4 | 12-1/8 | 14-1/2 | 16-7/8 | 19-1/4 | 21-5/8 | 24 |
| 3/8” Bars | 2-3/4 | 5-1/8 | 7-1/2 | 9-7/8 | 12-1/4 | 14-5/8 | 17 | 19-3/8 | 21-3/4 | 24-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 26-3/8 | 28-3/4 | 31-1/8 | 33-1/2 | 35-7/8 | 38-1/4 | 40-5/8 | 43 | 45-3/8 | 47-3/4 |
| 3/8” Bars | 26-1/2 | 28-7/8 | 31-1/4 | 33-5/8 | 36 | 38-3/8 | 40-3/4 | 43-1/8 | 45-1/2 | 47-7/8 |

64                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 38HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Smooth |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 16.36 | 132 | U Du C Dc | 34,386 0.01 17,193 0.00 | 15,283 0.01 11,462 0.01 | 8,596 0.02 8,596 0.02 | 5,502 0.04 6,877 0.03 | 3,821 0.05 5,731 0.04 | 2,807 0.07 4,912 0.06 | 2,149 0.09 4,298 0.08 | 1,698 0.12 3,821 0.10 | 1,375 0.15 3,439 0.12 | 1,137 0.18 3,126 0.14 | 955 0.21 2,865 0.17 | 814 0.25 2,645 0.20 | 702 0.29 2,456 0.23 | 611 0.33 2,292 0.27 | 537 0.38 2,149 0.30 | 2.579 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.513 |
| 3 ½" x ⅜" | 24.60 | 146 | U Du C Dc | 51,579 0.01 25,789 0.00 | 22,924 0.01 17,193 0.01 | 12,895 0.02 12,895 0.02 | 8,253 0.04 10,316 0.03 | 5,731 0.05 8,596 0.04 | 4,211 0.07 7,368 0.06 | 3,224 0.09 6,447 0.08 | 2,547 0.12 5,731 0.10 | 2,063 0.15 5,158 0.12 | 1,705 0.18 4,689 0.14 | 1,433 0.21 4,298 0.17 | 1,221 0.25 3,968 0.20 | 1,053 0.29 3,684 0.23 | 917 0.33 3,439 0.27 | 806 0.38 3,224 0.30 | 3.868 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.770 |
| 4" x¼" | 18.51 | 146 | U Du C Dc | 44,912 0.01 22,456 0.00 | 19,961 0.01 14,971 0.01 | 11,228 0.02 11,228 0.02 | 7,186 0.03 8,982 0.03 | 4,990 0.05 7,485 0.04 | 3,666 0.06 6,416 0.05 | 2,807 0.08 5,614 0.07 | 2,218 0.10 4,990 0.08 | 1,796 0.13 4,491 0.10 | 1,485 0.16 4,083 0.13 | 1,248 0.19 3,743 0.15 | 1,063 0.22 3,455 0.17 | 917 0.25 3,208 0.20 | 798 0.29 2,994 0.23 | 702 0.33 2,807 0.26 | 3.368 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.737 |
| 4" x ⅜" | 27.82 | 161 | U Du C Dc | 67,368 0.01 33,684 0.00 | 29,942 0.01 22,456 0.01 | 16,842 0.02 16,842 0.02 | 10,779 0.03 13,474 0.03 | 7,485 0.05 11,228 0.04 | 5,499 0.06 9,624 0.05 | 4,211 0.08 8,421 0.07 | 3,327 0.10 7,485 0.08 | 2,695 0.13 6,737 0.10 | 2,227 0.16 6,124 0.13 | 1,871 0.19 5,614 0.15 | 1,595 0.22 5,182 0.17 | 1,375 0.25 4,812 0.20 | 1,198 0.29 4,491 0.23 | 1,053 0.33 4,211 0.26 | 5.053 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.105 |
| 5" x ⅜" | 34.26 | 191 | U Du C Dc | 105,263 0.00 52,632 0.00 | 46,784 0.01 35,088 0.01 | 26,316 0.02 26,316 0.01 | 16,842 0.03 21,053 0.02 | 11,696 0.04 17,544 0.03 | 8,593 0.05 15,038 0.04 | 6,579 0.07 13,158 0.05 | 5,198 0.08 11,696 0.07 | 4,211 0.10 10,526 0.08 | 3,480 0.13 9,569 0.10 | 2,924 0.15 8,772 0.12 | 2,491 0.17 8,097 0.14 | 2,148 0.20 7,519 0.16 | 1,871 0.23 7,018 0.19 | 1,645 0.26 6,579 0.21 | 7.895 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 19.737 |
| 6" x ⅜" | 40.70 | 218 | U Du C Dc | 151,579 0.00 75,789 0.00 | 67,368 0.01 50,526 0.01 | 37,895 0.01 37,895 0.01 | 24,253 0.02 30,316 0.02 | 16,842 0.03 25,263 0.02 | 12,374 0.04 21,654 0.03 | 9,474 0.06 18,947 0.04 | 7,485 0.07 16,842 0.06 | 6,063 0.09 15,158 0.07 | 5,011 0.10 13,780 0.08 | 4,211 0.12 12,632 0.10 | 3,588 0.15 11,660 0.12 | 3,093 0.17 10,827 0.14 | 2,695 0.19 10,105 0.16 | 2,368 0.22 9,474 0.18 | 11.368 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34.105 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

38HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-5/8 | 5 | 7-3/8 | 9-3/4 | 12-1/8 | 14-1/2 | 16-7/8 | 19-1/4 | 21-5/8 | 24 |
| 3/8” Bars | 2-3/4 | 5-1/8 | 7-1/2 | 9-7/8 | 12-1/4 | 14-5/8 | 17 | 19-3/8 | 21-3/4 | 24-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 26-3/8 | 28-3/4 | 31-1/8 | 33-1/2 | 35-7/8 | 38-1/4 | 40-5/8 | 43 | 45-3/8 | 47-3/4 |
| 3/8” Bars | 26-1/2 | 28-7/8 | 31-1/4 | 33-5/8 | 36 | 38-3/8 | 40-3/4 | 43-1/8 | 45-1/2 | 47-7/8 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              65

LOAD TABLES | HEAVY DUTY, IMPERIAL

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 38HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 1" x ¼" | 5.62 | 42 | U Du C Dc | 1,579 0.03 789 0.02 | 702 0.06 526 0.05 | 395 0.11 395 0.09 | 253 0.17 316 0.14 | 175 0.25 263 0.20 | 129 0.34 226 0.27 |  |  |  |  |  |  |  |  |  | 0.118 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.044 |
| 1 ¼" x ¼" | 6.70 | 51 | U Du C Dc | 2,807 0.02 1,404 0.02 | 1,248 0.05 936 0.04 | 702 0.08 702 0.07 | 449 0.13 561 0.10 | 312 229 0.19 0.25 468 401 0.15 0.20 |  | 175 0.33 351 0.26 | 139 0.42 312 0.34 |  |  |  |  |  |  |  | 0.211 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.105 |
| 1 ½" x ¼" | 7.77 | 61 | U Du C Dc | 4,386 0.02 2,193 0.01 | 1,949 0.04 1,462 0.03 | 1,096 0.07 1,096 0.05 | 702 0.10 877 0.08 | 487 358 0.15 0.20 731 627 0.12 0.16 |  | 274 0.26 548 0.21 | 217 0.34 487 0.27 | 175 0.41 439 0.33 | 145 0.50 399 0.40 |  |  |  |  |  | 0.329 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.206 |
| 1 ½" x ⅜" | 11.17 | 67 | U Du C Dc | 6,579 0.02 3,289 0.01 | 2,924 0.04 2,193 0.03 | 1,645 0.07 1,645 0.05 | 1,053 0.10 1,316 0.08 | 731 537 0.15 0.20 1,096 940 0.12 0.16 |  | 411 0.26 822 0.21 | 325 0.34 731 0.27 | 263 0.41 658 0.33 | 217 0.50 598 0.40 | 183 0.60 548 0.48 |  |  |  |  | 0.493 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.308 |
| 2" x ¼" | 9.92 | 78 | U Du C Dc | 8,596 0.01 4,298 0.01 | 3,821 0.03 2,865 0.02 | 2,149 0.05 2,149 0.04 | 1,375 0.07 1,719 0.06 | 955 702 0.11 0.14 1,433 1,228 0.09 0.12 |  | 537 0.19 1,075 0.15 | 425 0.24 955 0.19 | 344 0.30 860 0.24 | 284 0.36 781 0.29 | 239 0.43 716 0.34 | 203 0.50 661 0.40 |  |  |  | 0.645 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.564 |
| 2 ½" x ¼" | 12.06 | 95 | U Du C Dc | 14,211 0.01 7,105 0.01 | 6,316 0.02 4,737 0.02 | 3,553 0.04 3,553 0.03 | 2,274 0.06 2,842 0.05 | 1,579 1,160 0.08 0.11 2,368 2,030 0.07 0.09 |  | 888 0.15 1,776 0.12 | 702 0.19 1,579 0.15 | 568 0.23 1,421 0.18 | 470 0.28 1,292 0.22 | 395 0.33 1,184 0.26 | 336 0.39 1,093 0.31 | 290 0.45 1,015 0.36 | 253 0.52 947 0.41 | 222 0.59 888 0.47 | 1.066 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.199 |
| 3" x ¼" | 14.21 | 110 | U Du C Dc | 21,228 0.01 10,614 0.01 | 9,435 0.02 7,076 0.01 | 5,307 0.03 5,307 0.02 | 3,396 0.05 4,246 0.04 | 2,359 1,733 0.07 0.09 3,538 3,033 0.05 0.07 |  | 1,327 0.12 2,654 0.10 | 1,048 0.15 2,359 0.12 | 849 0.19 2,123 0.15 | 702 0.23 1,930 0.18 | 590 0.27 1,769 0.22 | 502 0.32 1,633 0.25 | 433 0.37 1,516 0.29 | 377 0.42 1,415 0.34 | 332 0.48 1,327 0.39 | 1.592 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.189 |
| 3" x ⅜" | 20.83 | 122 | U Du C Dc | 31,842 0.01 15,921 0.01 | 14,152 0.02 10,614 0.01 | 7,961 0.03 7,961 0.02 | 5,095 0.05 6,368 0.04 | 3,538 2,599 0.07 0.09 5,307 4,549 0.05 0.07 |  | 1,990 0.12 3,980 0.10 | 1,572 0.15 3,538 0.12 | 1,274 0.19 3,184 0.15 | 1,053 0.23 2,895 0.18 | 885 0.27 2,654 0.22 | 754 0.32 2,449 0.25 | 650 0.37 2,274 0.29 | 566 0.42 2,123 0.34 | 498 0.48 1,990 0.39 | 2.388 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.284 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

38HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-5/8 | 5 | 7-3/8 | 9-3/4 | 12-1/8 | 14-1/2 | 16-7/8 | 19-1/4 | 21-5/8 | 24 |
| 3/8” Bars | 2-3/4 | 5-1/8 | 7-1/2 | 9-7/8 | 12-1/4 | 14-5/8 | 17 | 19-3/8 | 21-3/4 | 24-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 26-3/8 | 28-3/4 | 31-1/8 | 33-1/2 | 35-7/8 | 38-1/4 | 40-5/8 | 43 | 45-3/8 | 47-3/4 |
| 3/8” Bars | 26-1/2 | 28-7/8 | 31-1/4 | 33-5/8 | 36 | 38-3/8 | 40-3/4 | 43-1/8 | 45-1/2 | 47-7/8 |

66                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD IMPERIAL

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 38HW4 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 36 Surface: Serrated |  | U = Safe Uniform Load (lbs/ft2) Du = Deflection Due to Safe Uniform Load (in) C = Safe Concentrated Load (lbs/ft of grating width) Dc = Deflection Due to Safe Concentrated Load (in) Allowable Extreme Fiber Stress = 20 ksi |
| --- | --- | --- |

| Bearing Bar Size (inches) | Approx. Weight (lbs/ft2) | Ped. Span (inches) | Load / Deflection | Span (ft -in) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1' - 0" | 1' - 6" | 2' - 0" | 2' - 6" | 3' - 0" | 3' - 6" | 4' - 0" | 4' - 6" | 5' - 0" | 5' - 6" | 6' - 0" | 6' - 6" | 7' - 0" | 7' - 6" | 8' - 0" | Sx (in3)/ft |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (in4)/ft |
| 3 ½" x ¼" | 16.36 | 125 | U Du C Dc | 29,649 0.01 14,825 0.01 | 13,177 0.01 9,883 0.01 | 7,412 0.03 7,412 0.02 | 4,744 0.04 5,930 0.03 | 3,294 0.06 4,942 0.05 | 2,420 0.08 4,236 0.06 | 1,853 0.10 3,706 0.08 | 1,464 0.13 3,294 0.10 | 1,186 0.16 2,965 0.13 | 980 0.19 2,695 0.15 | 824 0.23 2,471 0.18 | 702 0.27 2,281 0.22 | 605 0.31 2,118 0.25 | 527 0.36 1,977 0.29 | 463 0.41 1,853 0.33 | 2.224 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.613 |
| 3 ½" x ⅜" | 24.60 | 138 | U Du C Dc | 44,474 0.01 22,237 0.01 | 19,766 0.01 14,825 0.01 | 11,118 0.03 11,118 0.02 | 7,116 0.04 8,895 0.03 | 4,942 0.06 7,412 0.05 | 3,631 0.08 6,353 0.06 | 2,780 0.10 5,559 0.08 | 2,196 0.13 4,942 0.10 | 1,779 0.16 4,447 0.13 | 1,470 0.19 4,043 0.15 | 1,235 0.23 3,706 0.18 | 1,053 0.27 3,421 0.22 | 908 0.31 3,177 0.25 | 791 0.36 2,965 0.29 | 695 0.41 2,780 0.33 | 3.336 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.420 |
| 4" x¼" | 18.51 | 139 | U Du C Dc | 39,474 0.01 19,737 0.00 | 17,544 0.01 13,158 0.01 | 9,868 0.02 9,868 0.02 | 6,316 0.03 7,895 0.03 | 4,386 0.05 6,579 0.04 | 3,222 0.07 5,639 0.05 | 2,467 0.09 4,934 0.07 | 1,949 0.11 4,386 0.09 | 1,579 0.14 3,947 0.11 | 1,305 0.17 3,589 0.13 | 1,096 0.20 3,289 0.16 | 934 0.23 3,036 0.19 | 806 0.27 2,820 0.22 | 702 0.31 2,632 0.25 | 617 0.35 2,467 0.28 | 2.961 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.551 |
| 4" x ⅜" | 27.82 | 154 | U Du C Dc | 59,211 0.01 29,605 0.00 | 26,316 0.01 19,737 0.01 | 14,803 0.02 14,803 0.02 | 9,474 0.03 11,842 0.03 | 6,579 0.05 9,868 0.04 | 4,834 0.07 8,459 0.05 | 3,701 0.09 7,401 0.07 | 2,924 0.11 6,579 0.09 | 2,368 0.14 5,921 0.11 | 1,957 0.17 5,383 0.13 | 1,645 0.20 4,934 0.16 | 1,401 0.23 4,555 0.19 | 1,208 0.27 4,229 0.22 | 1,053 0.31 3,947 0.25 | 925 0.35 3,701 0.28 | 4.441 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.326 |
| 5" x ⅜" | 34.26 | 183 | U Du C Dc | 95,000 0.00 47,500 0.00 | 42,222 0.01 31,667 0.01 | 23,750 0.02 23,750 0.01 | 15,200 0.03 19,000 0.02 | 10,556 0.04 15,833 0.03 | 7,755 0.05 13,571 0.04 | 5,938 0.07 11,875 0.06 | 4,691 0.09 10,556 0.07 | 3,800 0.11 9,500 0.09 | 3,140 0.13 8,636 0.11 | 2,639 0.16 7,917 0.13 | 2,249 0.18 7,308 0.15 | 1,939 0.21 6,786 0.17 | 1,689 0.25 6,333 0.20 | 1,484 0.28 5,938 0.22 | 7.125 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16.922 |
| 6" x ⅜" | 40.70 | 212 | U Du C Dc | 139,211 0.00 69,605 0.00 | 61,871 0.01 46,404 0.01 | 34,803 0.01 34,803 0.01 | 22,274 0.02 27,842 0.02 | 15,468 0.03 23,202 0.03 | 11,364 0.04 19,887 0.04 | 8,701 0.06 17,401 0.05 | 6,875 0.07 15,468 0.06 | 5,568 0.09 13,921 0.07 | 4,602 0.11 12,656 0.09 | 3,867 0.13 11,601 0.10 | 3,295 0.15 10,709 0.12 | 2,841 0.18 9,944 0.14 | 2,475 0.20 9,281 0.16 | 2,175 0.23 8,701 0.18 | 10.441 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30.017 |

Spans and loads in red exceed a deflection of 1/4” for uniform loads of 100 lbs./sq. ft. Experience has shown that 1/4” deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

38HW4 (in)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4” Bars | 2-5/8 | 5 | 7-3/8 | 9-3/4 | 12-1/8 | 14-1/2 | 16-7/8 | 19-1/4 | 21-5/8 | 24 |
| 3/8” Bars | 2-3/4 | 5-1/8 | 7-1/2 | 9-7/8 | 12-1/4 | 14-5/8 | 17 | 19-3/8 | 21-3/4 | 24-1/8 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 1/4” Bars | 26-3/8 | 28-3/4 | 31-1/8 | 33-1/2 | 35-7/8 | 38-1/4 | 40-5/8 | 43 | 45-3/8 | 47-3/4 |
| 3/8” Bars | 26-1/2 | 28-7/8 | 31-1/4 | 33-5/8 | 36 | 38-3/8 | 40-3/4 | 43-1/8 | 45-1/2 | 47-7/8 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               67

## NOTES                                                              LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - HD METRIC

Heavy-Duty Welded Grating has the strength for heavy- duty load areas such as airfields, industrial plants,

truck and bus terminals, parking lots and railroad

yards. Common uses are flooring, driveways, subway and tunnel ventilation grilles, curb inlet grates, ramps, docks, etc.

It is a sturdy grating to carry loads and maintain the

same level over many years of use. Whenever rolling wheel loads are to be used over the grating, we

recommend that the grating be load banded to add

lateral strength. Serrations are available on bars up to 10mm thick to provide additional traction for

rolling loads.

Standard heavy-duty bar grating is resistance-welded

for durability, strength and safety using an automated

electric/hydraulic welding process. High temperatures, combined with high pressure fuse the bearing bars and cross bars together to form a permanent joint.

HD Close-Mesh

Heavy-Duty

Wide-Gap

Extra-Wide-Gap

OVERVIEW

68                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  | www.vulcraft.com/grating                                                                                                              69

LOAD TABLES | HEAVY DUTY, METRIC

### VEHICULAR LOADS

DESIGN CRITERIA

The load and deflection tables on the following pages

have been prepared to provide the designer with a

convenient reference for the load carrying capabilities of typical heavy duty grating.

Static Loads

Uniform loads and concentrated loads per foot of grating width are given on 152.4mm increments for spans from

300mm to 2,438mm. The values in these load tables are based on allowable stresses for static loads.

Vehicular load tables are designed in accordance

with the 16th Edition of the American Association of

State Highway and Transportation Officials (AASHTO) for H-10 through H-25 loads with deflection limited

to the lesser of .125 inches (3.175 mm) or L/400

| Determine M: | FS M= 12 |  |  |  |
| --- | --- | --- | --- | --- |
| Substituting for M, solve for L: | (i) a M = | > L PL2 | (ii) a > L M = P(.25L - .125a) b |  |
|  |  | 8ab |  |  |
| Check D*: | D= | P1 [(2 | L3) - (a2 L) + (a3/4)] 96EI |  |

to a maximum simple span of 8’- 0” (2,438mm).

*Deflection should be limited to 1/400 span. M =   Bending Moment

S =   Section Modulus - mm3/m of grating width I =   Moment of Inertia - mm3/m of grating width

E =   Modulus of Elasticity

F =   Allowable Bending Stress L =   Simple Clear Span - mm D =   Deflection - mm

a =   Partial Load Contact Parallel to Span - mm

s =   Center-to-Center Spacing Between Bars - mm

b =   Partial Load Contact Dimension at 90° to span - mm b =   a + (2s)

P =   Total Wheel of Partial Load Including Impact - kN.

P1 =   P per bearing bar P1 =   P x (s/b)

Automobile and forklift loads are similarly evaluated

with loads calculated and distributed in accordance with the criteria shown below. If the load conditions

of your application are not adequately addressed

LOAD TABLES - HD METRIC

in the criteria presented, please contact Vulcraft for assistance in determining the proper grating for your application.

| Vehicular Load Table Criteria | H-255 | H-20/ HL-936 | H-15 | H-102 | Passenger Vehicles | 5 Ton Forklifts3 | 3 Ton Forklifts3 | 1 Ton Forklifts3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Truck/ Vehicle Weight (kN) |  |  |  |  | 28 | 64 | 44 | 19 |
| Load Capacity (kN) |  |  |  |  | 16 | 44 | 27 | 9 |
| Axle Load (kN) | 178 | 142 | 107 | 71 |  |  |  |  |
| Impact Factor | 30% | 30% | 30% | 30% | 30% | 30% | 30% | 30% |
| Total Load (kN) | 231 | 185 | 139 | 93 | 21 | 58 | 35 | 12 |
| % of Load on Drive Axel |  |  |  |  | 60% | 85% | 85% | 85% |
| Wheel Load (kN) | 116 | 93 | 69 | 46 | 10 | 29 | 17 | 6 |
| A-Length of distribution perpendicular to axle or parallel to main bars (mm) | 635 | 508 | 381 | 254 | 229 | 279 | 178 | 102 |
| C-Width of distribution parallel to axle or perpendicular to main bars (mm) | 635 | 508 | 381 | 254 | 229 | 279 | 178 | 102 |

Notes:

1. For continuous spans, use continuity factor = .80.

2. This distribution results in larger grating sizes for lighter trucks on shorter spans.

3. The fork lift wheel loads and load distribution patterns depicted above, generally, and only partially, represent the broad range of rubber-tired lift trucks available. For those applications falling outside of these examples, please contact Vulcraft.

4. Wheeled vehicles with urethane tires should NEVER be used in conjunction with open grid bar grating.

5. HS20 is the same as H20 and HS15 is the same as H15. The “S” stands for semi-trailer.

6. The “HL-93” notation shown with “H-20” represents AASHTO’s truck loading standard post-1993. Since, 1993, H-10, H-20, etc. have been retired in lieu of the “HL-93” loading which represents all trucks.

70                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               71

LOAD TABLES | HEAVY DUTY, METRIC

VEHICULAR LOADS                                                                                                                  VEHICULAR LOADS

| 60HW102 |  |  |  | Maximum Clear Span Between Supports (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx mm³/m | Ix mm⁴/m | Unit Wt. kPa | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 25 x 6 | 11,470 | 145.66E+3 | 26.61 | 358 | 296 | 236 | 177 | 241 | 181 | 137 | 142 |
| 32 x 6 | 17,980 | 285.84E+3 | 31.96 | 381 | 320 | 261 | 206 | 312 | 205 | 164 | 193 |
| 38 x 6 | 25,810 | 491.61E+3 | 37.24 | 409 | 349 | 292 | 241 | 399 | 233 | 197 | 255 |
| 38 x 10 | 38,310 | 729.74E+3 | 53.41 | 455 | 397 | 343 | 297 | 542 | 280 | 250 | 357 |
| 51 x 6 | 45,880 | 1.17E+6 | 47.87 | 481 | 424 | 371 | 329 | 621 | 306 | 280 | 414 |
| 64 x 6 | 71,680 | 2.28E+6 | 58.51 | 572 | 519 | 472 | 442 | 906 | 400 | 388 | 618 |
| 76 x 6 | 103,230 | 3.93E+6 | 69.14 | 684 | 635 | 596 | 581 | 1,255 | 514 | 520 | 868 |
| 76 x 10 | 153,230 | 5.84E+6 | 102.94 | 868 | 826 | 799 | 808 | 1,538* | 701 | 735 | 1,276 |
| 102 x 6 | 183,510 | 9.32E+6 | 90.41 | 970 | 932 | 912 | 935 | 1,790* | 805 | 855 | 1,503 |
| 102 x 10 | 272,400 | 13.84E+6 | 134.51 | 1,296 | 1,271 | 1,272 | 1,338 | 2,048* | 1,138 | 1,238 | 1,829* |
| 127 x 10 | 425,630 | 27.03E+6 | 166.08 | 1,785* | 1,795* | 1,823* | 1,885* | 2,558* | 1,699 | 1,849* | 2,287* |
| 152 x 10 | 612,900 | 46.70E+6 | 197.65 | 2,130* | 2,147* | 2,183* | 2,260* | 3,069* | 2,120* | 2,218* | 2,743* |

Note: All loads based on Smooth surface

| 30HW102 |  |  |  | Maximum Clear Span Between Supports (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx mm³/m | Ix mm⁴/m | Unit Wt. kPa | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 25 x 6 | 22,940 | 291.33E+3 | 47.87 | 393 | 331 | 270 | 212 | 324 | 210 | 165 | 183 |
| 32 x 6 | 35,960 | 571.69E+3 | 58.59 | 435 | 374 | 314 | 259 | 442 | 250 | 208 | 258 |
| 38 x 6 | 51,610 | 983.22E+3 | 69.14 | 486 | 426 | 369 | 318 | 586 | 299 | 261 | 349 |
| 38 x 10 | 76,610 | 1.46E+6 | 100.76 | 571 | 513 | 458 | 413 | 822 | 378 | 347 | 498 |
| 51 x 6 | 91,760 | 2.33E+6 | 90.41 | 618 | 561 | 508 | 466 | 954 | 422 | 395 | 580 |
| 64 x 6 | 143,370 | 4.55E+6 | 111.68 | 786 | 733 | 686 | 657 | 1,326* | 581 | 567 | 878 |
| 76 x 6 | 206,450 | 7.87E+6 | 132.95 | 993 | 944 | 905 | 890 | 1,589* | 776 | 777 | 1,242 |
| 76 x 10 | 306,450 | 11.68E+6 | 197.65 | 1,330 | 1,289 | 1,262 | 1,271 | 1,818* | 1,094 | 1,121 | 1,557* |
| 102 x 6 | 367,020 | 18.64E+6 | 175.48 | 1,518 | 1,480 | 1,460 | 1,483 | 2,116* | 1,270 | 1,312 | 1,812* |
| 102 x 10 | 544,800 | 27.68E+6 | 260.79 | 1,752* | 1,752* | 1,762* | 1,793* | 2,421* | 1,689* | 1,730* | 2,075* |
| 127 x 10 | 851,250 | 54.05E+6 | 323.93 | 2,174* | 2,179* | 2,196* | 2,239* | 3,024* | 2,109* | 2,161* | 2,593* |
| 152 x 10 | 1,225,800 | 93.41E+6 | 387.07 | 2,599* | 2,608* | 2,631* | 2,685* | 3,629* | 2,528* | 2,592* | 3,111* |

Indicates that value was controlled by L/400  ≤ 3mm deflection limit.

Indicates that value was controlled by L/400  ≤ 3mm deflection limit.

LOAD TABLES - HD METRIC

Note: All loads based on Smooth surface

Vulcraft’s structural fabrication services can be

leveraged to further aid you in getting a superior

solution for covering your concrete opening by also obtaining an Embed Frame with your grating.  A steel embed frame can improve the quality and lifespan of your project by:

• Shielding the concrete at the opening edges from cracking and chipping,

• Providing an edge for the opening when forming the concrete pour,

• Providing uniform elevation for the opening to minimize potential for uneven surfaces,

• And providing a smooth an uniform bearing surface for the grating, allowing for easier attachment and better performance over it’s lifetime.

Frames are available in normal rectangular

configurations only and will be supplied as a fully-

assembled, four-sided unit in sizes up to those that can safely be transported via normal flatbed carriers.  Sizes or configurations other than this should be discussed

with Vulcraft.  Embed frames can be supplied mill

finished, painted, or hot-dipped galvanized.  To order, please include a detail similar to the following with the Clear Opening Width and Span clearly defined as well as the desired quantities and finish.

Grating Frames

| 24HW102 |  |  |  | Maximum Clear Span Between Supports (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx mm³/m | Ix mm⁴/m | Unit Wt. kPa | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 25 x 6 | 28,670 | 364.16E+3 | 58.51 | 411 | 349 | 288 | 230 | 369 | 226 | 181 | 206 |
| 32 x 6 | 44,940 | 714.61E+3 | 71.91 | 463 | 403 | 343 | 288 | 512 | 275 | 233 | 293 |
| 38 x 6 | 64,520 | 1.23E+6 | 85.09 | 528 | 468 | 411 | 359 | 687 | 334 | 296 | 400 |
| 38 x 10 | 95,770 | 1.82E+6 | 124.44 | 633 | 575 | 521 | 475 | 974 | 431 | 399 | 575 |
| 51 x 6 | 114,700 | 2.91E+6 | 111.68 | 691 | 634 | 582 | 540 | 1,133 | 485 | 457 | 672 |
| 64 x 6 | 179,210 | 5.69E+6 | 138.26 | 901 | 848 | 802 | 772 | 1,706 | 679 | 664 | 1,021 |
| 76 x 6 | 258,060 | 9.83E+6 | 164.85 | 1,158 | 1,109 | 1,071 | 1,056 | 2,133* | 917 | 917 | 1,448 |
| 76 x 10 | 383,060 | 14.59E+6 | 245.01 | 1,578 | 1,537 | 1,511 | 1,521 | 2,441* | 1,305 | 1,330 | 2,067* |
| 102 x 6 | 458,780 | 23.31E+6 | 218.02 | 1,812 | 1,775 | 1,755 | 1,779 | 2,842* | 1,521 | 1,560 | 2,408* |
| 102 x 10 | 681,000 | 34.59E+6 | 323.93 | 2,353* | 2,357* | 2,371* | 2,409* | 3,252* | 2,212 | 2,296 | 2,756* |
| 127 x 10 | 1,064,070 | 67.57E+6 | 402.86 | 2,929* | 2,938* | 2,960* | 3,010* | 4,089* | 2,837* | 2,894* | 3,444* |
| 152 x 10 | 1,532,260 | 116.76E+6 | 481.78 | 3,508* | 3,521* | 3,549* | 3,610* | 4,876* | 3,402* | 3,472* | 4,133* |

Indicates that value was controlled by L/400  ≤ 3mm deflection limit.

Standard Duty

Heavy Duty

Note: All loads based on Smooth surface

| 48HW102 |  |  |  | Maximum Clear Span Between Supports (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing Bar Size | Sx mm³/m | Ix mm⁴/m | Unit Wt. kPa | H-25 | H-20 / HL-93 | H-15 | H-10 | Auto Traffic | 5-Ton Forklift | 3-Ton Forklift | 1-Ton Forklift |
| 25 x 6 | 14,340 | 182.08E+3 | 31.92 | 367 | 306 | 245 | 187 | 264 | 189 | 145 | 153 |
| 32 x 6 | 22,470 | 357.30E+3 | 38.62 | 396 | 335 | 275 | 220 | 348 | 217 | 176 | 211 |
| 38 x 6 | 32,260 | 614.51E+3 | 45.22 | 430 | 370 | 313 | 262 | 451 | 251 | 214 | 282 |
| 38 x 10 | 47,880 | 912.17E+3 | 65.24 | 486 | 428 | 374 | 329 | 619 | 307 | 277 | 397 |
| 51 x 6 | 57,350 | 1.46E+6 | 58.51 | 517 | 461 | 408 | 366 | 712 | 338 | 312 | 461 |
| 64 x 6 | 89,610 | 2.84E+6 | 71.80 | 630 | 577 | 530 | 501 | 1,048 | 449 | 437 | 692 |
| 76 x 6 | 129,030 | 4.92E+6 | 85.09 | 767 | 719 | 680 | 665 | 1,420* | 585 | 591 | 974 |
| 76 x 10 | 191,530 | 7.30E+6 | 126.62 | 992 | 951 | 924 | 935 | 1,625* | 808 | 842 | 1,430* |
| 102 x 6 | 229,390 | 11.65E+6 | 111.68 | 1,117 | 1,080 | 1,060 | 1,084 | 1,892* | 932 | 981 | 1,665* |
| 102 x 10 | 340,500 | 17.30E+6 | 166.08 | 1,517 | 1,493 | 1,495 | 1,563 | 2,163* | 1,328 | 1,427 | 1,905* |
| 127 x 10 | 532,030 | 33.78E+6 | 205.55 | 1,906* | 1,915* | 1,938* | 1,994* | 2,702* | 1,874* | 1,946* | 2,381* |
| 152 x 10 | 766,130 | 58.38E+6 | 245.01 | 2,276* | 2,290* | 2,322* | 2,392* | 3,242* | 2,247* | 2,334* | 2,857* |

Indicates that value was controlled by L/400  ≤ 3mm deflection limit.

72                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating                            VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               73

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - HD

| Grating Type: 30HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 47.9 | 1,560.00 | U Du C Dc | 272.0 0.5 41.5 0.4 | 121.0 1.2 27.7 0.9 | 68.1 2.1 20.7 1.7 | 43.6 3.3 16.6 2.6 | 30.3 4.7 13.8 3.8 | 22.2 6.4 11.9 5.2 | 17.0 8.4 10.4 6.7 | 13.4 10.6 9.2 8.5 | 10.9 13.1 8.3 10.5 | 9.0 15.9 7.5 12.7 | 9 |  | 6 6 |  |  | 22,940 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 291.33E+3 |
| 32 x 6 | 58.6 | 1,846.00 | U Du C Dc | 426.4 0.4 65.0 0.3 | 189.6 0.9 43.4 0.8 | 106.7 1.7 32.5 1.3 | 68.3 2.6 26.0 2.1 | 47.4 3.8 21.7 3.0 | 34.8 5.1 18.6 4.1 | 26.7 6.7 16.3 5.4 | 21.1 8.5 14.5 6.8 | 17.1 14.1 10.5 12.7 13.0 11.8 8.4 10.2 |  | 11.9 15.1 10.8 12.1 | 10.1 17.7 10.0 14.2 | 6 |  |  | 35,960 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 571.69E+3 |
| 38 x 6 | 69.1 | 2,114.00 | U Du C Dc | 612.1 0.4 93.3 0.3 | 272.2 0.8 62.2 0.6 | 153.1 1.4 46.7 1.1 | 98.0 2.2 37.4 1.8 | 68.1 3.2 31.1 2.5 | 50.0 4.3 26.7 3.4 | 38.3 5.6 23.3 4.5 | 30.3 7.1 20.8 5.7 | 24.5 20.3 8.8 10.6 18.7 17.0 7.0 8.5 |  | 17.0 12.6 15.6 10.1 | 14.5 14.8 14.4 11.8 | 12.5 17.2 13.3 13.7 |  |  | 51,610 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 983.22E+3 |
| 38 x 10 | 100.8 | 2,334.00 | U Du C Dc | 908.6 0.4 138.6 0.3 | 404.0 0.8 92.4 0.6 | 227.3 1.4 69.3 1.1 | 145.5 2.2 55.4 1.8 | 101.0 3.2 46.2 2.5 | 74.2 4.3 39.6 3.4 | 56.8 5.6 34.7 4.5 | 44.9 7.1 30.8 5.7 | 36.4 30.1 8.8 10.6 27.7 25.2 7.0 8.5 |  | 25.3 12.6 23.1 10.1 | 21.5 14.8 21.3 11.8 | 18.6 17.2 19.8 13.7 | 16.2 19.7 18.5 15.8 | 14.2 22.4 17.3 17.9 | 76,610 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.46E+6 |
| 51 x 6 | 90.4 | 2,623.00 | U Du C Dc | 1,088.2 0.3 165.9 0.2 | 483.8 0.6 110.7 0.5 | 272.2 1.1 83.0 0.8 | 174.2 1.6 66.4 1.3 | 121.0 2.4 55.3 1.9 | 88.9 3.2 47.4 2.6 | 68.1 4.2 41.5 3.4 | 53.8 5.3 36.9 4.3 | 43.6 36.0 6.6 7.9 33.2 30.2 5.3 6.4 |  | 30.3 9.5 27.7 7.6 | 25.8 11.1 25.5 8.9 | 22.2 12.9 23.7 10.3 | 19.4 14.8 22.1 11.8 | 17.0 16.8 20.8 13.4 | 91,760 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.33E+6 |
| 64 x 6 | 111.7 | 3,101.00 | U Du C Dc | 1,700.2 0.2 259.3 0.2 | 756.0 0.5 172.9 0.4 | 425.3 0.8 129.7 0.7 | 272.3 1.3 103.8 1.1 | 189.1 1.9 86.5 1.5 | 138.9 2.6 74.1 2.1 | 106.4 3.4 64.9 2.7 | 84.0 4.3 57.6 3.4 | 68.1 56.3 5.3 6.4 51.9 47.2 4.2 5.1 |  | 47.3 7.6 43.2 6.1 | 40.3 8.9 39.9 7.1 | 34.7 10.3 37.1 8.2 | 30.3 11.8 34.6 9.5 | 26.6 13.4 32.4 10.8 | 143,370 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.55E+6 |
| 76 x 6 | 132.9 | 3,556.00 | U Du C Dc | 2,448.3 0.2 373.4 0.1 | 1,088.6 0.4 249.0 0.3 | 612.5 0.7 186.7 0.6 | 392.0 1.1 149.4 0.9 | 272.3 1.6 124.5 1.3 | 200.1 2.1 106.7 1.7 | 153.2 2.8 93.4 2.2 | 121.0 3.5 83.0 2.8 | 98.0 81.0 4.4 5.3 74.7 67.9 3.5 4.2 |  | 68.1 6.3 62.3 5.0 | 58.0 7.4 57.5 5.9 | 50.0 8.6 53.4 6.9 | 43.6 9.9 49.8 7.9 | 38.3 11.2 46.7 9.0 | 206,450 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.87E+6 |
| 76 x 10 | 197.7 | 3,925.00 | U Du C Dc | 3,634.3 0.2 554.2 0.1 | 1,615.9 0.4 369.6 0.3 | 909.2 0.7 277.2 0.6 | 581.9 1.1 221.8 0.9 | 404.2 1.6 184.8 1.3 | 297.0 2.1 158.4 1.7 | 227.4 2.8 138.6 2.2 | 179.7 3.5 123.2 2.8 | 145.5 120.3 4.4 5.3 110.9 100.8 3.5 4.2 |  | 101.1 6.3 92.4 5.0 | 86.1 7.4 85.3 5.9 | 74.3 8.6 79.2 6.9 | 64.7 9.9 73.9 7.9 | 56.9 11.2 69.3 9.0 | 306,450 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.68E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 30HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 36 | 66 | 96 | 126 | 156 | 186 | 216 | 246 | 276 | 306 |
| 10mm Bars | 40 | 70 | 100 | 130 | 160 | 190 | 220 | 250 | 280 | 310 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 336 | 366 | 396 | 426 | 456 | 486 | 516 | 546 | 576 | 606 |
| 10mm Bars | 340 | 370 | 400 | 430 | 460 | 490 | 520 | 550 | 580 | 610 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 6mm Bars | 636 | 666 | 696 | 726 | 756 | 786 | 816 | 846 | 876 | 906 |
| 10mm Bars | 640 | 670 | 700 | 730 | 760 | 790 | 820 | 850 | 880 | 910 |

74                                                                                                               VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - HD

| Grating Type: 30HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size | Approx. Wieght |  | Ped. Span | Load / |  |  |  |  |  |  |  |  |  |  | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (mm) | (kg/m2) |  | (mm) | Deflection | 305 |  | 457 | 610 |  | 762 | 915 |  | 1067 |  | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 | 1981 |  | 2134 |  | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 |  | 154.2 | 3,991.00 | U Du C Dc | 3,332.5 0.2 508.2 0.1 |  | 1,481.7 0.3 338.9 0.3 | 833.7 0.6 254.2 0.5 |  | 533.6 0.9 203.4 0.8 | 370.6 1.4 169.5 1.1 |  | 272.3 1.8 145.3 1.5 |  | 208.5 2.4 127.1 1.9 |  | 164.7 3.0 113.0 2.4 |  | 133.4 3.8 101.7 3.0 |  | 110.3 4.5 92.4 3.6 |  | 92.7 5.4 84.7 4.3 | 79.0 6.3 78.2 5.1 |  | 68.1 7.4 72.6 5.9 |  | 59.3 8.4 67.8 6.8 |  | 52.2 9.6 63.6 7.7 | 281,000 12.49E+6 |
| 89 x 10 |  | 229.2 | 4,406.00 | U Du C Dc | 4,946.6 0.2 754.4 0.1 |  | 2,199.5 0.3 503.0 0.3 | 1,237.5 0.6 377.3 0.5 |  | 792.1 0.9 301.9 0.8 | 550.1 1.4 251.6 1.1 |  | 404.2 1.8 215.6 1.5 |  | 309.5 2.4 188.7 1.9 |  | 244.5 3.0 167.7 2.4 |  | 198.1 3.8 151.0 3.0 |  | 163.7 4.5 137.2 3.6 |  | 137.6 5.4 125.8 4.3 | 117.2 6.3 116.1 5.1 |  | 101.1 7.4 107.8 5.9 |  | 88.0 8.4 100.6 6.8 |  | 77.4 9.6 94.4 7.7 | 417110 18.54E+6 |
| 102 x 6 |  | 175.5 | 4,412.00 | U Du C Dc | 4,352.6 0.1 663.8 0.1 |  | 1,935.3 0.3 442.6 0.2 | 1,088.9 0.5 332.0 0.4 |  | 697.0 0.8 265.6 0.7 | 484.0 1.2 221.4 0.9 |  | 355.6 1.6 189.7 1.3 |  | 272.3 2.1 166.0 1.7 |  | 215.2 2.7 147.6 2.1 |  | 174.3 3.3 132.8 2.6 |  | 144.0 4.0 120.8 3.2 |  | 121.0 4.7 110.7 3.8 | 103.1 5.6 102.2 4.4 |  | 88.9 6.4 94.9 5.1 |  | 77.5 7.4 88.6 5.9 |  | 68.1 8.4 83.0 6.7 | 367,020 18.64E+6 |
| 102 x 10 |  | 260.8 | 4,870.00 | U Du C Dc | 6,460.9 0.1 985.3 0.1 |  | 2,872.8 0.3 657.0 0.2 | 1,616.3 0.5 492.8 0.4 |  | 1,034.6 0.8 394.3 0.7 | 718.5 1.2 328.6 0.9 |  | 527.9 1.6 281.6 1.3 |  | 404.2 2.1 246.4 1.7 |  | 319.4 2.7 219.1 2.1 |  | 258.7 3.3 197.2 2.6 |  | 213.8 4.0 179.2 3.2 |  | 179.7 4.7 164.3 3.8 | 153.1 5.6 151.7 4.4 |  | 132.0 6.4 140.8 5.1 |  | 115.0 7.4 131.4 5.9 |  | 101.1 8.4 123.3 6.7 | 544,800 27.68E+6 |
| 127 x 10 |  | 323.9 | 5,757.00 | U Du C Dc | 10,095.2 0.1 1,539.5 0.1 |  | 4,488.7 0.2 1,026.6 0.2 | 2,525.4 0.4 770.0 0.3 |  | 1,616.5 0.7 616.0 0.5 | 1,122.7 0.9 513.4 0.8 |  | 824.9 1.3 440.1 1.0 |  | 631.6 1.7 385.1 1.3 |  | 499.0 2.1 342.3 1.7 |  | 404.2 2.6 308.1 2.1 |  | 334.1 3.2 280.1 2.5 |  | 280.7 3.8 256.7 3.0 | 239.2 4.4 237.0 3.6 |  | 206.3 5.1 220.1 4.1 |  | 179.7 5.9 205.4 4.7 |  | 158.0 6.7 192.6 5.4 | 851,250 54.05E+6 |
| 152 x 10 |  | 387.1 | 6,600.00 | U Du C Dc | 14,537.0 0.1 2,216.9 0.1 |  | 6,463.7 0.2 1,478.3 0.2 | 3,636.6 0.4 1,108.8 0.3 |  | 2,327.8 0.5 887.1 0.4 | 1,616.6 0.8 739.3 0.6 |  | 1,187.8 1.1 633.7 0.9 |  | 909.5 1.4 554.5 1.1 |  | 718.6 1.8 492.9 1.4 |  | 582.1 2.2 443.6 1.8 |  | 481.1 2.6 403.3 2.1 |  | 404.2 3.2 369.7 2.5 | 344.5 3.7 341.3 3.0 |  | 297.0 4.3 316.9 3.4 |  | 258.7 4.9 295.8 3.9 |  | 227.5 5.6 277.3 4.5 | 1,225,800 93.41E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 30HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  |  | 2 | 3 |  |  | 4 |  |  | 5 |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  | 9 |  | 10 |  |  |  | 11 |  |
| 6mm Bars |  |  | 36 | 66 |  |  | 96 |  |  | 126 |  |  | 156 |  |  |  | 186 |  |  |  | 216 |  |  | 246 |  | 276 |  |  |  | 306 |  |
| 10mm Bars |  |  | 40 | 70 |  |  | 100 |  |  | 130 |  |  | 160 |  |  |  | 190 |  |  |  | 220 |  |  | 250 |  | 280 |  |  |  | 310 |  |
| # of Bars |  |  | 12 | 13 |  |  | 14 |  |  | 15 |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  | 19 |  | 20 |  |  |  | 21 |  |
| 6mm Bars |  |  | 336 | 366 |  |  | 396 |  |  | 426 |  |  | 456 |  |  |  | 486 |  |  |  | 516 |  |  | 546 |  | 576 |  |  |  | 606 |  |
| 10mm Bars |  |  | 340 | 370 |  |  | 400 |  |  | 430 |  |  | 460 |  |  |  | 490 |  |  |  | 520 |  |  | 550 |  | 580 |  |  |  | 610 |  |
| # of Bars |  |  | 22 | 23 |  |  | 24 |  |  | 25 |  |  | 26 |  |  |  | 27 |  |  |  | 28 |  |  | 29 |  | 30 |  |  |  | 31 |  |
| 6mm Bars |  |  | 636 | 666 |  |  | 696 |  |  | 726 |  |  | 756 |  |  |  | 786 |  |  |  | 816 |  |  | 846 |  | 876 |  |  |  | 906 |  |
| 10mm Bars |  |  | 640 | 670 |  |  | 700 |  |  | 730 |  |  | 760 |  |  |  | 790 |  |  |  | 820 |  |  | 850 |  | 880 |  |  |  | 910 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - HD

| Grating Type: 30HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 47.9 | 1,274.00 | U Du C Dc | 158.7 0.7 24.2 0.6 | 70.6 1.5 16.1 1.2 | 39.7 2.8 12.1 2.2 | 25.4 4.3 9.7 3.4 | 17.6 6.2 8.1 5.0 | 13.0 8.4 6.9 6.7 | 9.9 11.0 6.1 8.8 | 7.8 13.9 5.4 11.1 |  |  |  |  |  |  |  | 13,380 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 129.80E+3 |
| 32 x 6 | 58.6 | 1,578.00 | U Du C Dc | 280.7 0.5 42.8 0.4 | 124.8 1.2 28.5 0.9 | 70.2 2.1 21.4 1.7 | 44.9 3.2 17.1 2.6 | 31.2 4.7 14.3 3.7 | 22.9 6.3 12.2 5.1 | 17.6 13.9 8.3 10.5 10.7 9.5 6.6 8.4 |  | 11.2 12.9 8.6 10.3 | 9.3 15.7 7.8 12.5 |  |  |  |  |  | 23,670 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 305.31E+3 |
| 38 x 6 | 69.1 | 1,859.00 | U Du C Dc | 434.5 0.4 66.3 0.3 | 193.2 0.9 44.2 0.7 | 108.7 1.7 33.1 1.3 | 69.6 2.6 26.5 2.1 | 48.3 3.7 22.1 3.0 | 35.5 5.1 18.9 4.1 | 27.2 21.5 6.7 8.4 16.6 14.7 5.3 6.7 |  | 17.4 10.4 13.3 8.3 | 14.4 12.6 12.1 10.1 | 12.1 15.0 11.0 12.0 | 10.3 17.6 10.2 14.1 |  |  |  | 36,640 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 588.02E+3 |
| 38 x 10 | 100.8 | 2,052.00 | U Du C Dc | 644.9 0.4 98.4 0.3 | 286.8 0.9 65.6 0.7 | 161.3 1.7 49.2 1.3 | 103.3 2.6 39.4 2.1 | 71.7 3.7 32.8 3.0 | 52.7 5.1 28.1 4.1 | 40.3 31.9 6.7 8.4 24.6 21.9 5.3 6.7 |  | 25.8 10.4 19.7 8.3 | 21.3 12.6 17.9 10.1 | 17.9 15.0 16.4 12.0 | 15.3 17.6 15.1 14.1 | 13.2 20.4 14.1 16.3 |  |  | 54,380 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 872.84E+3 |
| 51 x 6 | 90.4 | 2,387.00 | U Du C Dc | 846.3 0.3 129.1 0.2 | 376.3 0.7 86.1 0.5 | 211.7 1.2 64.6 1.0 | 135.5 1.9 51.6 1.5 | 94.1 2.7 43.0 2.1 | 69.1 3.7 36.9 2.9 | 52.9 41.8 4.8 6.0 32.3 28.7 3.8 4.8 |  | 33.9 7.4 25.8 6.0 | 28.0 9.0 23.5 7.2 | 23.5 10.7 21.5 8.6 | 20.1 12.6 19.9 10.1 | 17.3 14.6 18.4 11.7 | 15.1 16.8 17.2 13.4 | 13.2 19.1 16.1 15.2 | 71,360 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.60E+6 |
| 64 x 6 | 111.7 | 2,879.00 | U Du C Dc | 1,394.1 0.2 212.6 0.2 | 619.9 0.5 141.8 0.4 | 348.8 0.9 106.3 0.7 | 223.2 1.5 85.1 1.2 | 155.0 2.1 70.9 1.7 | 113.9 2.8 60.8 2.3 | 87.2 68.9 3.7 4.7 53.2 47.3 3.0 3.8 |  | 55.8 5.8 42.5 4.6 | 46.1 7.0 38.7 5.6 | 38.8 8.4 35.5 6.7 | 33.0 9.8 32.7 7.8 | 28.5 11.4 30.4 9.1 | 24.8 13.1 28.4 10.4 | 21.8 14.8 26.6 11.9 | 117,560 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.38E+6 |
| 76 x 6 | 132.9 | 3,344.00 | U Du C Dc | 2,078.0 0.2 316.9 0.2 | 923.9 0.4 211.3 0.3 | 519.8 0.8 158.5 0.6 | 332.7 1.2 126.8 1.0 | 231.1 1.7 105.7 1.4 | 169.8 2.3 90.6 1.9 | 130.0 102.7 3.0 3.9 79.3 70.5 2.4 3.1 |  | 83.2 4.8 63.4 3.8 | 68.8 5.8 57.6 4.6 | 57.8 6.8 52.8 5.5 | 49.2 8.0 48.8 6.4 | 42.5 9.3 45.3 7.5 | 37.0 10.7 42.3 8.6 | 32.5 12.2 39.6 9.7 | 175,220 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.15E+6 |
| 76 x 10 | 195.5 | 3,691.00 | U Du C Dc | 3,084.5 0.2 470.4 0.2 | 1,371.5 0.4 313.7 0.3 | 771.6 0.8 235.3 0.6 | 493.9 1.2 188.2 1.0 | 343.0 1.7 156.9 1.4 | 252.0 2.3 134.5 1.9 | 193.0 152.5 3.0 3.9 117.7 104.6 2.4 3.1 |  | 123.5 4.8 94.1 3.8 | 102.1 5.8 85.6 4.6 | 85.8 6.8 78.4 5.5 | 73.1 8.0 72.4 6.4 | 63.0 9.3 67.2 7.5 | 54.9 10.7 62.8 8.6 | 48.3 12.2 58.8 9.7 | 260,090 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.13E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

30HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 36 | 66 | 96 | 126 | 156 | 186 | 216 | 246 | 276 | 306 |
| 10mm Bars | 40 | 70 | 100 | 130 | 160 | 190 | 220 | 250 | 280 | 310 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 336 | 366 | 396 | 426 | 456 | 486 | 516 | 546 | 576 | 606 |
| 10mm Bars | 340 | 370 | 400 | 430 | 460 | 490 | 520 | 550 | 580 | 610 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 6mm Bars | 636 | 666 | 696 | 726 | 756 | 786 | 816 | 846 | 876 | 906 |
| 10mm Bars | 640 | 670 | 700 | 730 | 760 | 790 | 820 | 850 | 880 | 910 |

76                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - HD

| Grating Type: 30HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) |  | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection |  |  |  |  |  |  |  | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 |  | 1219 | 1372 |  | 1524 |  | 1677 |  | 1829 |  | 1981 |  | 2134 |  | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 |  | 154.2 | 3,788.00 | U Du C Dc | 2,897.8 0.2 441.9 0.1 | 1,288.5 0.4 294.7 0.3 | 724.9 0.6 221.0 0.5 | 464.0 1.0 176.8 0.8 | 322.3 1.4 147.4 1.2 | 236.8 2.0 126.3 1.6 |  | 181.3 2.6 110.5 2.1 | 143.2 3.3 98.3 2.6 |  | 116.0 4.0 88.4 3.2 |  | 95.9 4.9 80.4 3.9 |  | 80.6 5.8 73.7 4.6 |  | 68.7 6.8 68.0 5.4 |  | 59.2 7.9 63.2 6.3 |  | 51.6 9.1 59.0 7.2 |  | 45.4 10.3 55.3 8.2 | 244,350 10.13E+6 |
| 89 x 10 |  | 229.2 | 4,181.00 | U Du C Dc | 4,301.5 0.2 656.0 0.1 | 1,912.6 0.4 437.4 0.3 | 1,076.1 0.6 328.1 0.5 | 688.8 1.0 262.5 0.8 | 478.4 1.4 218.8 1.2 | 351.5 2.0 187.5 1.6 |  | 269.1 2.6 164.1 2.1 | 212.6 3.3 145.8 2.6 |  | 172.2 4.0 131.3 3.2 |  | 142.3 4.9 119.3 3.9 |  | 119.6 5.8 109.4 4.6 |  | 101.9 6.8 101.0 5.4 |  | 87.9 7.9 93.8 6.3 |  | 76.6 9.1 87.5 7.2 |  | 67.3 10.3 82.1 8.2 | 362710 15.03E+6 |
| 102 x 6 |  | 175.5 | 4,215.00 | U Du C Dc | 3,853.7 0.1 587.7 0.1 | 1,713.5 0.3 391.9 0.3 | 964.1 0.6 293.9 0.4 | 617.1 0.9 235.2 0.7 | 428.6 1.3 196.0 1.0 | 314.9 1.7 168.0 1.4 |  | 241.1 2.2 147.0 1.8 | 190.5 2.8 130.7 2.3 |  | 154.3 3.5 117.6 2.8 |  | 127.5 4.2 106.9 3.4 |  | 107.2 5.0 98.0 4.0 |  | 91.3 5.9 90.5 4.7 |  | 78.7 6.8 84.0 5.5 |  | 68.6 7.9 78.4 6.3 |  | 60.3 8.9 73.5 7.1 | 324,960 15.53E+6 |
| 102 x 10 |  | 260.8 | 4,652.00 | U Du C Dc | 5,720.3 0.1 872.4 0.1 | 2,543.5 0.3 581.7 0.3 | 1,431.0 0.6 436.3 0.4 | 916.0 0.9 349.1 0.7 | 636.1 1.3 290.9 1.0 | 467.4 1.7 249.4 1.4 |  | 357.9 2.2 218.2 1.8 | 282.8 2.8 194.0 2.3 |  | 229.1 3.5 174.6 2.8 |  | 189.3 4.2 158.7 3.4 |  | 159.1 5.0 145.5 4.0 |  | 135.5 5.9 134.3 4.7 |  | 116.9 6.8 124.7 5.5 |  | 101.8 7.9 116.4 6.3 |  | 89.5 8.9 109.1 7.1 | 482,360 23.06E+6 |
| 127 x 10 |  | 323.9 | 5,552.00 | U Du C Dc | 9,163.8 0.1 1,397.5 0.1 | 4,074.6 0.2 931.9 0.2 | 2,292.5 0.4 699.0 0.4 | 1,467.4 0.7 559.2 0.6 | 1,019.1 1.0 466.0 0.8 | 748.8 1.4 399.5 1.1 |  | 573.3 1.8 349.5 1.4 | 453.0 2.2 310.7 1.8 |  | 366.9 2.8 279.6 2.2 |  | 303.3 3.3 254.2 2.7 |  | 254.8 4.0 233.0 3.2 |  | 217.1 4.7 215.1 3.7 |  | 187.2 5.4 199.8 4.3 |  | 163.1 6.2 186.4 5.0 |  | 143.4 7.1 174.8 5.6 | 772,720 46.75E+6 |
| 152 x 10 |  | 387.1 | 6,405.00 | U Du C Dc | 13,414.9 0.1 2,045.8 0.1 | 5,964.8 0.2 1,364.1 0.2 | 3,355.9 0.4 1,023.2 0.3 | 2,148.1 0.6 818.6 0.5 | 1,491.8 0.8 682.2 0.7 | 1,096.1 1.1 584.8 0.9 |  | 839.3 1.5 511.7 1.2 | 663.1 1.8 454.8 1.5 |  | 537.2 2.3 409.4 1.8 |  | 443.9 2.8 372.2 2.2 |  | 373.0 3.3 341.1 2.6 |  | 317.9 3.9 314.9 3.1 |  | 274.1 4.5 292.4 3.6 |  | 238.8 5.1 272.9 4.1 |  | 210.0 5.8 255.9 4.7 | 1,131,180 82.80E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 30HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  |  | 2 | 3 |  | 4 |  | 5 |  | 6 |  |  | 7 |  |  |  | 8 |  | 9 |  |  |  | 10 |  |  |  | 11 |  |
| 6mm Bars |  |  | 36 | 66 |  | 96 |  | 126 |  | 156 |  |  | 186 |  |  |  | 216 |  | 246 |  |  |  | 276 |  |  |  | 306 |  |
| 10mm Bars |  |  | 40 | 70 |  | 100 |  | 130 |  | 160 |  |  | 190 |  |  |  | 220 |  | 250 |  |  |  | 280 |  |  |  | 310 |  |
| # of Bars |  |  | 12 | 13 |  | 14 |  | 15 |  | 16 |  |  | 17 |  |  |  | 18 |  | 19 |  |  |  | 20 |  |  |  | 21 |  |
| 6mm Bars |  |  | 336 | 366 |  | 396 |  | 426 |  | 456 |  |  | 486 |  |  |  | 516 |  | 546 |  |  |  | 576 |  |  |  | 606 |  |
| 10mm Bars |  |  | 340 | 370 |  | 400 |  | 430 |  | 460 |  |  | 490 |  |  |  | 520 |  | 550 |  |  |  | 580 |  |  |  | 610 |  |
| # of Bars |  |  | 22 | 23 |  | 24 |  | 25 |  | 26 |  |  | 27 |  |  |  | 28 |  | 29 |  |  |  | 30 |  |  |  | 31 |  |
| 6mm Bars |  |  | 636 | 666 |  | 696 |  | 726 |  | 756 |  |  | 786 |  |  |  | 816 |  | 846 |  |  |  | 876 |  |  |  | 906 |  |
| 10mm Bars |  |  | 640 | 670 |  | 700 |  | 730 |  | 760 |  |  | 790 |  |  |  | 820 |  | 850 |  |  |  | 880 |  |  |  | 910 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Weight (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 58.5 | 1,649.00 | U Du C Dc | 340.0 0.5 51.9 0.4 | 151.2 1.2 34.6 0.9 | 85.1 2.1 25.9 1.7 | 54.5 3.3 20.8 2.6 | 37.8 4.7 17.3 3.8 | 27.8 6.4 14.8 5.2 | 21.3 8.4 13.0 6.7 | 16.8 10.6 11.5 8.5 | 13.6 13.1 10.4 10.5 | 11.3 15.9 9.4 12.7 |  | 0 |  |  |  | 28,670 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 364.16E+3 |
| 32 x 6 | 71.9 | 1,952.00 | U Du C Dc | 533.0 0.4 81.3 0.3 | 237.0 0.9 54.2 0.8 | 133.3 1.7 40.7 1.3 | 85.3 2.6 32.5 2.1 | 59.3 3.8 27.1 3.0 | 43.6 5.1 23.2 4.1 | 33.3 6.7 20.3 5.4 | 26.3 8.5 18.1 6.8 | 21.3 17.6 10.5 12.7 16.3 14.8 8.4 10.2 |  | 14.8 15.1 13.6 12.1 | 12.6 17.7 12.5 14.2 |  | 9 |  | 44,940 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 714.61E+3 |
| 38 x 6 | 85.1 | 2,235.00 | U Du C Dc | 765.1 0.4 116.7 0.3 | 340.2 0.8 77.8 0.6 | 191.4 1.4 58.4 1.1 | 122.5 2.2 46.7 1.8 | 85.1 3.2 38.9 2.5 | 62.5 4.3 33.4 3.4 | 47.9 5.6 29.2 4.5 | 37.8 7.1 25.9 5.7 | 30.6 25.3 8.8 10.6 23.3 21.2 7.0 8.5 |  | 21.3 12.6 19.5 10.1 | 18.1 14.8 18.0 11.8 | 15.6 17.2 16.7 13.7 | 13.6 19.7 15.6 15.8 |  | 64,520 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.23E+6 |
| 38 x 10 | 124.4 | 2,468.00 | U Du C Dc | 1,135.7 0.4 173.2 0.3 | 505.0 0.8 115.5 0.6 | 284.1 1.4 86.6 1.1 | 181.9 2.2 69.3 1.8 | 126.3 3.2 57.8 2.5 | 92.8 4.3 49.5 3.4 | 71.1 5.6 43.3 4.5 | 56.1 7.1 38.5 5.7 | 45.5 37.6 8.8 10.6 34.7 31.5 7.0 8.5 |  | 31.6 12.6 28.9 10.1 | 26.9 14.8 26.7 11.8 | 23.2 17.2 24.8 13.7 | 20.2 19.7 23.1 15.8 | 17.8 22.4 21.7 17.9 | 95,770 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.82E+6 |
| 51 x 6 | 111.7 | 2,774.00 | U Du C Dc | 1,360.2 0.3 207.4 0.2 | 604.8 0.6 138.3 0.5 | 340.3 1.1 103.7 0.8 | 217.8 1.6 83.0 1.3 | 151.3 2.4 69.2 1.9 | 111.1 3.2 59.3 2.6 | 85.1 4.2 51.9 3.4 | 67.2 5.3 46.1 4.3 | 54.5 45.0 6.6 7.9 41.5 37.7 5.3 6.4 |  | 37.8 9.5 34.6 7.6 | 32.2 11.1 31.9 8.9 | 27.8 12.9 29.6 10.3 | 24.2 14.8 27.7 11.8 | 21.3 16.8 25.9 13.4 | 114,700 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.91E+6 |
| 64 x 6 | 138.3 | 3,279.00 | U Du C Dc | 2,125.3 0.2 324.1 0.2 | 945.0 0.5 216.1 0.4 | 531.7 0.8 162.1 0.7 | 340.3 1.3 129.7 1.1 | 236.4 1.9 108.1 1.5 | 173.7 2.6 92.6 2.1 | 133.0 3.4 81.1 2.7 | 105.1 4.3 72.1 3.4 | 85.1 70.3 5.3 6.4 64.9 59.0 4.2 5.1 |  | 59.1 7.6 54.0 6.1 | 50.4 8.9 49.9 7.1 | 43.4 10.3 46.3 8.2 | 37.8 11.8 43.2 9.5 | 33.3 13.4 40.5 10.8 | 179,210 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.69E+6 |
| 76 x 6 | 164.8 | 3,760.00 | U Du C Dc | 3,060.4 0.2 466.7 0.1 | 1,360.8 0.4 311.2 0.3 | 765.6 0.7 233.4 0.6 | 490.1 1.1 186.8 0.9 | 340.3 1.6 155.6 1.3 | 250.1 2.1 133.4 1.7 | 191.5 2.8 116.7 2.2 | 151.3 3.5 103.8 2.8 | 122.5 101.3 4.4 5.3 93.4 84.9 3.5 4.2 |  | 85.1 6.3 77.8 5.0 | 72.5 7.4 71.8 5.9 | 62.5 8.6 66.7 6.9 | 54.5 9.9 62.3 7.9 | 47.9 11.2 58.4 9.0 | 258,060 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.83E+6 |
| 76 x 10 | 245.0 | 4,150.00 | U Du C Dc | 4,542.8 0.2 692.8 0.1 | 2,019.9 0.4 462.0 0.3 | 1,136.5 0.7 346.5 0.6 | 727.4 1.1 277.2 0.9 | 505.2 1.6 231.0 1.3 | 371.2 2.1 198.0 1.7 | 284.2 2.8 173.3 2.2 | 224.6 3.5 154.0 2.8 | 181.9 150.3 4.4 5.3 138.6 126.0 3.5 4.2 |  | 126.3 6.3 115.5 5.0 | 107.6 7.4 106.6 5.9 | 92.8 8.6 99.0 6.9 | 80.9 9.9 92.4 7.9 | 71.1 11.2 86.7 9.0 | 383,060 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.59E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 24HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 30 | 54 | 78 | 102 | 126 | 150 | 174 | 198 | 222 | 246 |
| 10mm Bars | 34 | 58 | 82 | 106 | 130 | 154 | 178 | 202 | 226 | 250 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 270 | 294 | 318 | 342 | 366 | 390 | 414 | 438 | 462 | 486 |
| 10mm Bars | 274 | 298 | 322 | 346 | 370 | 394 | 418 | 442 | 466 | 490 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 6mm Bars | 510 | 534 | 558 | 582 | 606 | 630 | 654 | 678 | 702 | 726 |
| 10mm Bars | 514 | 538 | 562 | 586 | 610 | 634 | 658 | 682 | 706 | 730 |

78                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size | Approx. Weight |  | Ped. Span | Load / | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (mm) | (kg/m2) |  | (mm) | Deflection | 305 |  | 457 | 610 |  | 762 | 915 |  | 1067 |  | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 | 1981 |  | 2134 | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 |  | 191.4 | 4,220.00 | U Du C Dc | 4,165.6 0.2 635.3 0.1 |  | 1,852.2 0.3 423.6 0.3 | 1,042.1 0.6 317.7 0.5 |  | 667.0 0.9 254.2 0.8 | 463.2 1.4 211.8 1.1 |  | 340.4 1.8 181.6 1.5 |  | 260.6 2.4 158.9 1.9 |  | 205.9 3.0 141.2 2.4 |  | 166.8 3.8 127.1 3.0 |  | 137.9 4.5 115.6 3.6 |  | 115.8 5.4 105.9 4.3 | 98.7 6.3 97.8 5.1 |  | 85.1 7.4 90.8 5.9 | 74.1 8.4 84.7 6.8 |  | 65.2 9.6 79.5 7.7 | 351,250 15.61E+6 |
| 89 x 10 |  | 284.5 | 4,658.00 | U Du C Dc | 6,183.3 0.2 943.0 0.1 |  | 2,749.3 0.3 628.8 0.3 | 1,546.8 0.6 471.6 0.5 |  | 990.1 0.9 377.3 0.8 | 687.6 1.4 314.5 1.1 |  | 505.2 1.8 269.5 1.5 |  | 386.8 2.4 235.9 1.9 |  | 305.7 3.0 209.7 2.4 |  | 247.6 3.8 188.7 3.0 |  | 204.6 4.5 171.5 3.6 |  | 171.9 5.4 157.2 4.3 | 146.5 6.3 145.1 5.1 |  | 126.3 7.4 134.8 5.9 | 110.1 8.4 125.8 6.8 |  | 96.8 9.6 118.0 7.7 | 521390 23.18E+6 |
| 102 x 6 |  | 218.0 | 4,665.00 | U Du C Dc | 5,440.8 0.1 829.7 0.1 |  | 2,419.2 0.3 553.3 0.2 | 1,361.1 0.5 415.0 0.4 |  | 871.2 0.8 332.0 0.7 | 605.1 1.2 276.7 0.9 |  | 444.6 1.6 237.2 1.3 |  | 340.4 2.1 207.5 1.7 |  | 269.0 2.7 184.5 2.1 |  | 217.9 3.3 166.0 2.6 |  | 180.1 4.0 150.9 3.2 |  | 151.3 4.7 138.4 3.8 | 128.9 5.6 127.7 4.4 |  | 111.2 6.4 118.6 5.1 | 96.8 7.4 110.7 5.9 |  | 85.2 8.4 103.8 6.7 | 458,780 23.31E+6 |
| 102 x 10 |  | 323.9 | 5,149.00 | U Du C Dc | 8,076.1 0.1 1,231.6 0.1 |  | 3,591.0 0.3 821.3 0.2 | 2,020.4 0.5 616.0 0.4 |  | 1,293.2 0.8 492.8 0.7 | 898.1 1.2 410.7 0.9 |  | 659.9 1.6 352.1 1.3 |  | 505.3 2.1 308.1 1.7 |  | 399.2 2.7 273.8 2.1 |  | 323.4 3.3 246.5 2.6 |  | 267.3 4.0 224.0 3.2 |  | 224.6 4.7 205.4 3.8 | 191.4 5.6 189.6 4.4 |  | 165.0 6.4 176.0 5.1 | 143.7 7.4 164.3 5.9 |  | 126.4 8.4 154.1 6.7 | 681,000 34.59E+6 |
| 127 x 10 |  | 402.9 | 6,087.00 | U Du C Dc | 12,618.9 0.1 1,924.4 0.1 |  | 5,610.9 0.2 1,283.2 0.2 | 3,156.8 0.4 962.5 0.3 |  | 2,020.6 0.7 770.1 0.5 | 1,403.3 0.9 641.7 0.8 |  | 1,031.1 1.3 550.1 1.0 |  | 789.5 1.7 481.3 1.3 |  | 623.8 2.1 427.9 1.7 |  | 505.3 2.6 385.1 2.1 |  | 417.6 3.2 350.1 2.5 |  | 350.9 3.8 320.9 3.0 | 299.0 4.4 296.2 3.6 |  | 257.8 5.1 275.1 4.1 | 224.6 5.9 256.7 4.7 |  | 197.5 6.7 240.7 5.4 | 1,064,070 67.57E+6 |
| 152 x 10 |  | 481.8 | 6,979.00 | U Du C Dc | 18,171.3 0.1 2,771.1 0.1 |  | 8,079.7 0.2 1,847.8 0.2 | 4,545.8 0.4 1,386.0 0.3 |  | 2,909.7 0.5 1,108.9 0.4 | 2,020.8 0.8 924.1 0.6 |  | 1,484.8 1.1 792.1 0.9 |  | 1,136.8 1.4 693.1 1.1 |  | 898.3 1.8 616.1 1.4 |  | 727.6 2.2 554.5 1.8 |  | 601.3 2.6 504.1 2.1 |  | 505.3 3.2 462.1 2.5 | 430.6 3.7 426.6 3.0 |  | 371.3 4.3 396.1 3.4 | 323.4 4.9 369.7 3.9 |  | 284.4 5.6 346.7 4.5 | 1,532,260 116.76E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 24HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  |  | 2 | 3 |  |  | 4 |  |  | 5 |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  | 9 |  | 10 |  |  | 11 |  |
| 6mm Bars |  |  | 30 | 54 |  |  | 78 |  |  | 102 |  |  | 126 |  |  |  | 150 |  |  |  | 174 |  |  | 198 |  | 222 |  |  | 246 |  |
| 10mm Bars |  |  | 34 | 58 |  |  | 82 |  |  | 106 |  |  | 130 |  |  |  | 154 |  |  |  | 178 |  |  | 202 |  | 226 |  |  | 250 |  |
| # of Bars |  |  | 12 | 13 |  |  | 14 |  |  | 15 |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  | 19 |  | 20 |  |  | 21 |  |
| 6mm Bars |  |  | 270 | 294 |  |  | 318 |  |  | 342 |  |  | 366 |  |  |  | 390 |  |  |  | 414 |  |  | 438 |  | 462 |  |  | 486 |  |
| 10mm Bars |  |  | 274 | 298 |  |  | 322 |  |  | 346 |  |  | 370 |  |  |  | 394 |  |  |  | 418 |  |  | 442 |  | 466 |  |  | 490 |  |
| # of Bars |  |  | 22 | 23 |  |  | 24 |  |  | 25 |  |  | 26 |  |  |  | 27 |  |  |  | 28 |  |  | 29 |  | 30 |  |  | 31 |  |
| 6mm Bars |  |  | 510 | 534 |  |  | 558 |  |  | 582 |  |  | 606 |  |  |  | 630 |  |  |  | 654 |  |  | 678 |  | 702 |  |  | 726 |  |
| 10mm Bars |  |  | 514 | 538 |  |  | 562 |  |  | 586 |  |  | 610 |  |  |  | 634 |  |  |  | 658 |  |  | 682 |  | 706 |  |  | 730 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 58.5 | 1,348.00 | U Du C Dc | 198.4 0.7 30.3 0.6 | 88.2 1.5 20.2 1.2 | 49.6 2.8 15.1 2.2 | 31.8 4.3 12.1 3.4 | 22.1 6.2 10.1 5.0 | 16.2 8.4 8.6 6.7 | 12.4 11.0 7.6 8.8 | 9.8 13.9 6.7 11.1 |  |  |  |  |  |  |  | 16,730 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 162.25E+3 |
| 32 x 6 | 71.9 | 1,669.00 | U Du C Dc | 350.8 0.5 53.5 0.4 | 156.0 1.2 35.7 0.9 | 87.8 2.1 26.8 1.7 | 56.2 3.2 21.4 2.6 | 39.0 4.7 17.8 3.7 | 28.7 6.3 15.3 5.1 | 21.9 17.3 8.3 10.5 13.4 11.9 6.6 8.4 |  | 14.0 12.9 10.7 10.3 | 11.6 15.7 9.7 12.5 |  |  |  |  |  | 29,580 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 381.63E+3 |
| 38 x 6 | 85.1 | 1,966.00 | U Du C Dc | 543.1 0.4 82.8 0.3 | 241.5 0.9 55.2 0.7 | 135.9 1.7 41.4 1.3 | 87.0 2.6 33.1 2.1 | 60.4 3.7 27.6 3.0 | 44.4 5.1 23.7 4.1 | 34.0 26.8 6.7 8.4 20.7 18.4 5.3 6.7 |  | 21.7 10.4 16.6 8.3 | 18.0 12.6 15.1 10.1 | 15.1 15.0 13.8 12.0 | 12.9 17.6 12.7 14.1 |  |  |  | 45,800 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 735.03E+3 |
| 38 x 10 | 124.4 | 2,170.00 | U Du C Dc | 806.2 0.4 122.9 0.3 | 358.5 0.9 82.0 0.7 | 201.7 1.7 61.5 1.3 | 129.1 2.6 49.2 2.1 | 89.7 3.7 41.0 3.0 | 65.9 5.1 35.1 4.1 | 50.4 39.9 6.7 8.4 30.8 27.3 5.3 6.7 |  | 32.3 10.4 24.6 8.3 | 26.7 12.6 22.4 10.1 | 22.4 15.0 20.5 12.0 | 19.1 17.6 18.9 14.1 | 16.5 20.4 17.6 16.3 | 14.3 23.4 16.4 18.7 |  | 67,980 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.09E+6 |
| 51 x 6 | 111.7 | 2,524.00 | U Du C Dc | 1,057.9 0.3 161.3 0.2 | 470.4 0.7 107.6 0.5 | 264.6 1.2 80.7 1.0 | 169.4 1.9 64.6 1.5 | 117.6 2.7 53.8 2.1 | 86.4 3.7 46.1 2.9 | 66.2 52.3 4.8 6.0 40.4 35.9 3.8 4.8 |  | 42.4 7.4 32.3 6.0 | 35.0 9.0 29.3 7.2 | 29.4 10.7 26.9 8.6 | 25.1 12.6 24.8 10.1 | 21.6 14.6 23.1 11.7 | 18.8 16.8 21.5 13.4 | 16.6 19.1 20.2 15.2 | 89,200 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.00E+6 |
| 64 x 6 | 138.3 | 3,044.00 | U Du C Dc | 1,742.6 0.2 265.8 0.2 | 774.8 0.5 177.2 0.4 | 435.9 0.9 132.9 0.7 | 279.0 1.5 106.3 1.2 | 193.8 2.1 88.6 1.7 | 142.4 2.8 76.0 2.3 | 109.0 86.1 3.7 4.7 66.5 59.1 3.0 3.8 |  | 69.8 5.8 53.2 4.6 | 57.7 7.0 48.3 5.6 | 48.5 8.4 44.3 6.7 | 41.3 9.8 40.9 7.8 | 35.6 11.4 38.0 9.1 | 31.0 13.1 35.5 10.4 | 27.3 14.8 33.2 11.9 | 146,940 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.22E+6 |
| 76 x 6 | 164.8 | 3,535.00 | U Du C Dc | 2,597.4 0.2 396.1 0.2 | 1,154.9 0.4 264.1 0.3 | 649.8 0.8 198.1 0.6 | 415.9 1.2 158.5 1.0 | 288.9 1.7 132.1 1.4 | 212.2 2.3 113.2 1.9 | 162.5 128.4 3.0 3.9 99.1 88.1 2.4 3.1 |  | 104.0 4.8 79.3 3.8 | 86.0 5.8 72.1 4.6 | 72.2 6.8 66.1 5.5 | 61.5 8.0 61.0 6.4 | 53.1 9.3 56.6 7.5 | 46.2 10.7 52.8 8.6 | 40.7 12.2 49.6 9.7 | 219,020 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.69E+6 |
| 76 x 10 | 242.8 | 3,902.00 | U Du C Dc | 3,855.6 0.2 588.0 0.2 | 1,714.3 0.4 392.1 0.3 | 964.5 0.8 294.1 0.6 | 617.4 1.2 235.3 1.0 | 428.8 1.7 196.1 1.4 | 315.0 2.3 168.1 1.9 | 241.2 190.6 3.0 3.9 147.1 130.7 2.4 3.1 |  | 154.4 4.8 117.7 3.8 | 127.6 5.8 107.0 4.6 | 107.2 6.8 98.0 5.5 | 91.4 8.0 90.5 6.4 | 78.8 9.3 84.0 7.5 | 68.6 10.7 78.4 8.6 | 60.3 12.2 73.6 9.7 | 325,110 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.41E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

24HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 30 | 54 | 78 | 102 | 126 | 150 | 174 | 198 | 222 | 246 |
| 10mm Bars | 34 | 58 | 82 | 106 | 130 | 154 | 178 | 202 | 226 | 250 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 270 | 294 | 318 | 342 | 366 | 390 | 414 | 438 | 462 | 486 |
| 10mm Bars | 274 | 298 | 322 | 346 | 370 | 394 | 418 | 442 | 466 | 490 |
| # of Bars | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 6mm Bars | 510 | 534 | 558 | 582 | 606 | 630 | 654 | 678 | 702 | 726 |
| 10mm Bars | 514 | 538 | 562 | 586 | 610 | 634 | 658 | 682 | 706 | 730 |

80                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - HD CLOSE-MESH

| Grating Type: 24W102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) |  | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection |  |  |  |  |  |  |  | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 |  | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 |  | 1981 |  | 2134 |  | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 |  | 191.4 | 4,005.00 | U Du C Dc | 3,622.3 0.2 552.4 0.1 | 1,610.6 0.4 368.3 0.3 | 906.2 0.6 276.3 0.5 | 580.0 1.0 221.0 0.8 | 402.8 1.4 184.2 1.2 | 296.0 2.0 157.9 1.6 |  | 226.6 2.6 138.2 2.1 |  | 179.1 3.3 122.8 2.6 |  | 145.0 4.0 110.5 3.2 |  | 119.9 4.9 100.5 3.9 |  | 100.7 5.8 92.1 4.6 |  | 85.8 6.8 85.0 5.4 |  | 74.0 7.9 79.0 6.3 |  | 64.5 9.1 73.7 7.2 |  | 56.7 10.3 69.1 8.2 | 305,440 12.66E+6 |
| 89 x 10 |  | 284.5 | 4,421.00 | U Du C Dc | 5,376.8 0.2 820.0 0.1 | 2,390.7 0.4 546.8 0.3 | 1,345.1 0.6 410.1 0.5 | 861.0 1.0 328.1 0.8 | 597.9 1.4 273.4 1.2 | 439.3 2.0 234.4 1.6 |  | 336.4 2.6 205.1 2.1 |  | 265.8 3.3 182.3 2.6 |  | 215.3 4.0 164.1 3.2 |  | 177.9 4.9 149.2 3.9 |  | 149.5 5.8 136.7 4.6 |  | 127.4 6.8 126.2 5.4 |  | 109.9 7.9 117.2 6.3 |  | 95.7 9.1 109.4 7.2 |  | 84.2 10.3 102.6 8.2 | 453390 18.79E+6 |
| 102 x 6 |  | 218.0 | 4,457.00 | U Du C Dc | 4,817.1 0.1 734.6 0.1 | 2,141.9 0.3 489.8 0.3 | 1,205.1 0.6 367.4 0.4 | 771.3 0.9 294.0 0.7 | 535.7 1.3 245.0 1.0 | 393.6 1.7 210.0 1.4 |  | 301.4 2.2 183.7 1.8 |  | 238.1 2.8 163.3 2.3 |  | 192.9 3.5 147.0 2.8 |  | 159.4 4.2 133.6 3.4 |  | 134.0 5.0 122.5 4.0 |  | 114.1 5.9 113.1 4.7 |  | 98.4 6.8 105.0 5.5 |  | 85.7 7.9 98.0 6.3 |  | 75.4 8.9 91.9 7.1 | 406,190 19.42E+6 |
| 102 x 10 |  | 323.9 | 4,919.00 | U Du C Dc | 7,150.4 0.1 1,090.4 0.1 | 3,179.4 0.3 727.1 0.3 | 1,788.8 0.6 545.4 0.4 | 1,145.0 0.9 436.3 0.7 | 795.2 1.3 363.6 1.0 | 584.3 1.7 311.7 1.4 |  | 447.3 2.2 272.7 1.8 |  | 353.5 2.8 242.4 2.3 |  | 286.3 3.5 218.2 2.8 |  | 236.6 4.2 198.4 3.4 |  | 198.8 5.0 181.8 4.0 |  | 169.4 5.9 167.9 4.7 |  | 146.1 6.8 155.9 5.5 |  | 127.3 7.9 145.5 6.3 |  | 111.9 8.9 136.4 7.1 | 602,940 28.82E+6 |
| 127 x 10 |  | 402.9 | 5,870.00 | U Du C Dc | 11,454.8 0.1 1,746.9 0.1 | 5,093.2 0.2 1,164.8 0.2 | 2,865.6 0.4 873.7 0.4 | 1,834.2 0.7 699.0 0.6 | 1,273.9 1.0 582.5 0.8 | 936.0 1.4 499.3 1.1 |  | 716.6 1.8 436.9 1.4 |  | 566.2 2.2 388.4 1.8 |  | 458.7 2.8 349.6 2.2 |  | 379.1 3.3 317.8 2.7 |  | 318.5 4.0 291.3 3.2 |  | 271.4 4.7 268.9 3.7 |  | 234.0 5.4 249.7 4.3 |  | 203.9 6.2 233.0 5.0 |  | 179.3 7.1 218.5 5.6 | 965,900 58.44E+6 |
| 152 x 10 |  | 481.8 | 6,772.00 | U Du C Dc | 16,768.6 0.1 2,557.2 0.1 | 7,456.0 0.2 1,705.2 0.2 | 4,194.9 0.4 1,279.0 0.3 | 2,685.1 0.6 1,023.3 0.5 | 1,864.8 0.8 852.8 0.7 | 1,370.2 1.1 731.0 0.9 |  | 1,049.1 1.5 639.6 1.2 |  | 828.9 1.8 568.6 1.5 |  | 671.4 2.3 511.7 1.8 |  | 554.9 2.8 465.2 2.2 |  | 466.3 3.3 426.4 2.6 |  | 397.3 3.9 393.6 3.1 |  | 342.6 4.5 365.5 3.6 |  | 298.4 5.1 341.2 4.1 |  | 262.4 5.8 319.9 4.7 | 1,413,980 103.50E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 24HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  |  | 2 | 3 |  | 4 |  | 5 |  | 6 |  |  |  | 7 |  |  |  | 8 |  | 9 |  |  |  | 10 |  |  |  | 11 |  |
| 6mm Bars |  |  | 30 | 54 |  | 78 |  | 102 |  | 126 |  |  |  | 150 |  |  |  | 174 |  | 198 |  |  |  | 222 |  |  |  | 246 |  |
| 10mm Bars |  |  | 34 | 58 |  | 82 |  | 106 |  | 130 |  |  |  | 154 |  |  |  | 178 |  | 202 |  |  |  | 226 |  |  |  | 250 |  |
| # of Bars |  |  | 12 | 13 |  | 14 |  | 15 |  | 16 |  |  |  | 17 |  |  |  | 18 |  | 19 |  |  |  | 20 |  |  |  | 21 |  |
| 6mm Bars |  |  | 270 | 294 |  | 318 |  | 342 |  | 366 |  |  |  | 390 |  |  |  | 414 |  | 438 |  |  |  | 462 |  |  |  | 486 |  |
| 10mm Bars |  |  | 274 | 298 |  | 322 |  | 346 |  | 370 |  |  |  | 394 |  |  |  | 418 |  | 442 |  |  |  | 466 |  |  |  | 490 |  |
| # of Bars |  |  | 22 | 23 |  | 24 |  | 25 |  | 26 |  |  |  | 27 |  |  |  | 28 |  | 29 |  |  |  | 30 |  |  |  | 31 |  |
| 6mm Bars |  |  | 510 | 534 |  | 558 |  | 582 |  | 606 |  |  |  | 630 |  |  |  | 654 |  | 678 |  |  |  | 702 |  |  |  | 726 |  |
| 10mm Bars |  |  | 514 | 538 |  | 562 |  | 586 |  | 610 |  |  |  | 634 |  |  |  | 658 |  | 682 |  |  |  | 706 |  |  |  | 730 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 81 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - WIDE-GAP

| Grating Type: 48HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 31.9 | 1,387.00 | U Du C Dc | 170.0 0.5 25.9 0.4 | 75.6 1.2 17.3 0.9 | 42.5 2.1 13.0 1.7 | 27.2 3.3 10.4 2.6 | 18.9 4.7 8.6 3.8 | 13.9 6.4 7.4 5.2 | 10.6 8.4 6.5 6.7 | 8.4 10.6 5.8 8.5 | 6.8 13.1 5.2 10.5 |  |  |  |  |  |  | 14,340 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 182.08E+3 |
| 32 x 6 | 38.6 | 1,641.00 | U Du C Dc | 266.5 0.4 40.6 0.3 | 118.5 0.9 27.1 0.8 | 66.7 1.7 20.3 1.3 | 42.7 2.6 16.3 2.1 | 29.6 3.8 13.6 3.0 | 21.8 5.1 11.6 4.1 | 16.7 6.7 10.2 5.4 | 13.2 10.7 8.5 10.5 9.0 8.1 6.8 8.4 |  | 8.8 12.7 7.4 10.2 |  |  |  |  |  | 22,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 357.31E+3 |
| 38 x 6 | 45.2 | 1,880.00 | U Du C Dc | 382.6 0.4 58.3 0.3 | 170.1 0.8 38.9 0.6 | 95.7 1.4 29.2 1.1 | 61.3 2.2 23.3 1.8 | 42.5 3.2 19.5 2.5 | 31.3 4.3 16.7 3.4 | 23.9 5.6 14.6 4.5 | 18.9 15.3 7.1 8.8 13.0 11.7 5.7 7.0 |  | 12.7 10.6 10.6 8.5 | 10.6 12.6 9.7 10.1 | 9.1 14.8 9.0 11.8 |  |  |  | 32,260 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 614.52E+3 |
| 38 x 10 | 65.2 | 2,075.00 | U Du C Dc | 567.9 0.4 86.6 0.3 | 252.5 0.8 57.7 0.6 | 142.1 1.4 43.3 1.1 | 90.9 2.2 34.7 1.8 | 63.1 3.2 28.9 2.5 | 46.4 4.3 24.8 3.4 | 35.5 5.6 21.7 4.5 | 28.1 22.7 7.1 8.8 19.3 17.3 5.7 7.0 |  | 18.8 10.6 15.8 8.5 | 15.8 12.6 14.4 10.1 | 13.5 14.8 13.3 11.8 | 11.6 17.2 12.4 13.7 |  |  | 47,880 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 912.17E+3 |
| 51 x 6 | 58.5 | 2,332.00 | U Du C Dc | 680.1 0.3 103.7 0.2 | 302.4 0.6 69.2 0.5 | 170.1 1.1 51.9 0.8 | 108.9 1.6 41.5 1.3 | 75.6 2.4 34.6 1.9 | 55.6 3.2 29.6 2.6 | 42.5 4.2 25.9 3.4 | 33.6 27.2 5.3 6.6 23.1 20.8 4.3 5.3 |  | 22.5 7.9 18.9 6.4 | 18.9 9.5 17.3 7.6 | 16.1 11.1 16.0 8.9 | 13.9 12.9 14.8 10.3 | 12.1 14.8 13.8 11.8 | 10.6 16.8 13.0 13.4 | 57,350 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.46E+6 |
| 64 x 6 | 71.8 | 2,757.00 | U Du C Dc | 1,062.7 0.2 162.1 0.2 | 472.5 0.5 108.1 0.4 | 265.8 0.8 81.1 0.7 | 170.2 1.3 64.8 1.1 | 118.2 1.9 54.0 1.5 | 86.8 2.6 46.3 2.1 | 66.5 3.4 40.5 2.7 | 52.5 42.6 4.3 5.3 36.0 32.4 3.4 4.2 |  | 35.2 6.4 29.5 5.1 | 29.6 7.6 27.0 6.1 | 25.2 8.9 24.9 7.1 | 21.7 10.3 23.2 8.2 | 18.9 11.8 21.6 9.5 | 16.6 13.4 20.3 10.8 | 89,610 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.84E+6 |
| 76 x 6 | 85.1 | 3,161.00 | U Du C Dc | 1,530.2 0.2 233.4 0.1 | 680.4 0.4 155.6 0.3 | 382.8 0.7 116.7 0.6 | 245.0 1.1 93.4 0.9 | 170.2 1.6 77.8 1.3 | 125.0 2.1 66.7 1.7 | 95.7 2.8 58.4 2.2 | 75.6 61.3 3.5 4.4 51.9 46.7 2.8 3.5 |  | 50.6 5.3 42.5 4.2 | 42.6 6.3 38.9 5.0 | 36.3 7.4 35.9 5.9 | 31.3 8.6 33.4 6.9 | 27.2 9.9 31.1 7.9 | 23.9 11.2 29.2 9.0 | 129,030 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.92E+6 |
| 76 x 10 | 126.6 | 3,490.00 | U Du C Dc | 2,271.4 0.2 346.4 0.1 | 1,010.0 0.4 231.0 0.3 | 568.2 0.7 173.3 0.6 | 363.7 1.1 138.6 0.9 | 252.6 1.6 115.5 1.3 | 185.6 2.1 99.0 1.7 | 142.1 2.8 86.6 2.2 | 112.3 91.0 3.5 4.4 77.0 69.3 2.8 3.5 |  | 75.2 5.3 63.0 4.2 | 63.2 6.3 57.8 5.0 | 53.8 7.4 53.3 5.9 | 46.4 8.6 49.5 6.9 | 40.4 9.9 46.2 7.9 | 35.5 11.2 43.3 9.0 | 191,530 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.30E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 48HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 54 | 102 | 150 | 198 | 246 | 294 | 342 | 390 | 438 | 486 |
| 10mm Bars | 58 | 106 | 154 | 202 | 250 | 298 | 346 | 394 | 442 | 490 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 534 | 582 | 630 | 678 | 726 | 774 | 822 | 870 | 918 | 966 |
| 10mm Bars | 538 | 586 | 634 | 682 | 730 | 778 | 826 | 874 | 922 | 970 |

82                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - WIDE-GAP

| Grating Type: 48HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size | Approx. Wieght | Ped. Span | Load / | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (mm) | (kg/m2) | (mm) | Deflection | 305 |  | 457 |  | 610 |  | 762 |  | 915 | 1067 |  | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 |  | 1981 | 2134 |  | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 | 98.4 | 3,549.00 | U Du C Dc | 2,082.8 0.2 317.6 0.1 |  | 926.1 0.3 211.8 0.3 |  | 521.0 0.6 158.9 0.5 |  | 333.5 0.9 127.1 0.8 |  | 231.6 1.4 105.9 1.1 | 170.2 1.8 90.8 1.5 |  | 130.3 2.4 79.4 1.9 |  | 103.0 3.0 70.6 2.4 |  | 83.4 3.8 63.6 3.0 |  | 68.9 4.5 57.8 3.6 |  | 57.9 5.4 53.0 4.3 |  | 49.4 6.3 48.9 5.1 | 42.6 7.4 45.4 5.9 |  | 37.1 8.4 42.4 6.8 |  | 32.6 9.6 39.7 7.7 | 175,630 7.81E+6 |
| 89 x 10 | 146.4 | 3,917.00 | U Du C Dc | 3,091.6 0.2 471.5 0.1 |  | 1,374.7 0.3 314.4 0.3 |  | 773.4 0.6 235.8 0.5 |  | 495.1 0.9 188.7 0.8 |  | 343.8 1.4 157.2 1.1 | 252.6 1.8 134.8 1.5 |  | 193.4 2.4 117.9 1.9 |  | 152.8 3.0 104.8 2.4 |  | 123.8 3.8 94.3 3.0 |  | 102.3 4.5 85.8 3.6 |  | 86.0 5.4 78.6 4.3 |  | 73.3 6.3 72.6 5.1 | 63.2 7.4 67.4 5.9 |  | 55.0 8.4 62.9 6.8 |  | 48.4 9.6 59.0 7.7 | 260700 11.59E+6 |
| 102 x 6 | 111.7 | 3,923.00 | U Du C Dc | 2,720.4 0.1 414.9 0.1 |  | 1,209.6 0.3 276.6 0.2 |  | 680.5 0.5 207.5 0.4 |  | 435.6 0.8 166.0 0.7 |  | 302.5 1.2 138.3 0.9 | 222.3 1.6 118.6 1.3 |  | 170.2 2.1 103.8 1.7 |  | 134.5 2.7 92.2 2.1 |  | 108.9 3.3 83.0 2.6 |  | 90.0 4.0 75.5 3.2 |  | 75.6 4.7 69.2 3.8 |  | 64.5 5.6 63.9 4.4 | 55.6 6.4 59.3 5.1 |  | 48.4 7.4 55.3 5.9 |  | 42.6 8.4 51.9 6.7 | 229,390 11.65E+6 |
| 102 x 10 | 166.1 | 4,330.00 | U Du C Dc | 4,038.1 0.1 615.8 0.1 |  | 1,795.5 0.3 410.6 0.2 |  | 1,010.2 0.5 308.0 0.4 |  | 646.6 0.8 246.4 0.7 |  | 449.1 1.2 205.4 0.9 | 329.9 1.6 176.0 1.3 |  | 252.6 2.1 154.0 1.7 |  | 199.6 2.7 136.9 2.1 |  | 161.7 3.3 123.2 2.6 |  | 133.6 4.0 112.0 3.2 |  | 112.3 4.7 102.7 3.8 |  | 95.7 5.6 94.8 4.4 | 82.5 6.4 88.0 5.1 |  | 71.9 7.4 82.2 5.9 |  | 63.2 8.4 77.0 6.7 | 340,500 17.30E+6 |
| 127 x 10 | 205.5 | 5,119.00 | U Du C Dc | 6,309.5 0.1 962.2 0.1 |  | 2,805.4 0.2 641.6 0.2 |  | 1,578.4 0.4 481.3 0.3 |  | 1,010.3 0.7 385.0 0.5 |  | 701.7 0.9 320.9 0.8 | 515.5 1.3 275.0 1.0 |  | 394.7 1.7 240.7 1.3 |  | 311.9 2.1 213.9 1.7 |  | 252.6 2.6 192.5 2.1 |  | 208.8 3.2 175.0 2.5 |  | 175.5 3.8 160.5 3.0 |  | 149.5 4.4 148.1 3.6 | 128.9 5.1 137.5 4.1 |  | 112.3 5.9 128.4 4.7 |  | 98.7 6.7 120.4 5.4 | 532,030 33.78E+6 |
| 152 x 10 | 245.0 | 5,869.00 | U Du C Dc | 9,085.6 0.1 1,385.6 0.1 |  | 4,039.8 0.2 923.9 0.2 |  | 2,272.9 0.4 693.0 0.3 |  | 1,454.8 0.5 554.4 0.4 |  | 1,010.4 0.8 462.1 0.6 | 742.4 1.1 396.1 0.9 |  | 568.4 1.4 346.6 1.1 |  | 449.1 1.8 308.1 1.4 |  | 363.8 2.2 277.3 1.8 |  | 300.7 2.6 252.1 2.1 |  | 252.7 3.2 231.1 2.5 |  | 215.3 3.7 213.3 3.0 | 185.6 4.3 198.0 3.4 |  | 161.7 4.9 184.8 3.9 |  | 142.2 5.6 173.3 4.5 | 766,130 58.38E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 48HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  | 2 | 3 |  |  | 4 |  |  |  | 5 |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 | 10 |  |  |  | 11 |  |
| 6mm Bars |  | 54 | 102 |  |  | 150 |  |  |  | 198 |  |  | 246 |  |  |  | 294 |  |  |  | 342 |  |  |  | 390 | 438 |  |  |  | 486 |  |
| 10mm Bars |  | 58 | 106 |  |  | 154 |  |  |  | 202 |  |  | 250 |  |  |  | 298 |  |  |  | 346 |  |  |  | 394 | 442 |  |  |  | 490 |  |
| # of Bars |  | 12 | 13 |  |  | 14 |  |  |  | 15 |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |  | 19 | 20 |  |  |  | 21 |  |
| 6mm Bars |  | 534 | 582 |  |  | 630 |  |  |  | 678 |  |  | 726 |  |  |  | 774 |  |  |  | 822 |  |  |  | 870 | 918 |  |  |  | 966 |  |
| 10mm Bars |  | 538 | 586 |  |  | 634 |  |  |  | 682 |  |  | 730 |  |  |  | 778 |  |  |  | 826 |  |  |  | 874 | 922 |  |  |  | 970 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - WIDE-GAP

| Grating Type: 48HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 31.9 | 1,133.00 | U Du C Dc | 99.2 0.7 15.1 0.6 | 44.1 1.5 10.1 1.2 | 24.8 2.8 7.6 2.2 | 15.9 4.3 6.1 3.4 | 11.0 6.2 5.0 5.0 | 8.1 8.4 4.3 6.7 | 6.2 11.0 3.8 8.8 |  |  |  |  |  |  |  |  | 8,360 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 81.13E+3 |
| 32 x 6 | 38.6 | 1,403.00 | U Du C Dc | 175.4 0.5 26.8 0.4 | 78.0 1.2 17.8 0.9 | 43.9 2.1 13.4 1.7 | 28.1 3.2 10.7 2.6 | 19.5 4.7 8.9 3.7 | 14.3 11.0 6.3 8.3 7.6 6.7 5.1 6.6 |  | 8.7 10.5 5.9 8.4 | 7.0 12.9 5.4 10.3 |  |  |  |  |  |  | 14,790 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 190.82E+3 |
| 38 x 6 | 45.2 | 1,653.00 | U Du C Dc | 271.6 0.4 41.4 0.3 | 120.7 0.9 27.6 0.7 | 67.9 1.7 20.7 1.3 | 43.5 2.6 16.6 2.1 | 30.2 3.7 13.8 3.0 | 22.2 17.0 5.1 6.7 11.8 10.4 4.1 5.3 |  | 13.4 8.4 9.2 6.7 | 10.9 10.4 8.3 8.3 | 9.0 12.6 7.5 10.1 |  |  |  |  |  | 22,900 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 367.51E+3 |
| 38 x 10 | 65.2 | 1,825.00 | U Du C Dc | 403.1 0.4 61.5 0.3 | 179.2 0.9 41.0 0.7 | 100.8 1.7 30.7 1.3 | 64.5 2.6 24.6 2.1 | 44.8 3.7 20.5 3.0 | 32.9 25.2 5.1 6.7 17.6 15.4 4.1 5.3 |  | 19.9 8.4 13.7 6.7 | 16.1 10.4 12.3 8.3 | 13.3 12.6 11.2 10.1 | 11.2 15.0 10.3 12.0 |  |  |  |  | 33,990 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 545.53E+3 |
| 51 x 6 | 58.5 | 2,123.00 | U Du C Dc | 528.9 0.3 80.7 0.2 | 235.2 0.7 53.8 0.5 | 132.3 1.2 40.3 1.0 | 84.7 1.9 32.3 1.5 | 58.8 2.7 26.9 2.1 | 43.2 33.1 3.7 4.8 23.1 20.2 2.9 3.8 |  | 26.1 6.0 17.9 4.8 | 21.2 7.4 16.1 6.0 | 17.5 9.0 14.7 7.2 | 14.7 10.7 13.5 8.6 | 12.5 12.6 12.4 10.1 | 10.8 14.6 11.5 11.7 |  |  | 44,600 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 999.06E+3 |
| 64 x 6 | 71.8 | 2,560.00 | U Du C Dc | 871.3 0.2 132.9 0.2 | 387.4 0.5 88.6 0.4 | 218.0 0.9 66.5 0.7 | 139.5 1.5 53.2 1.2 | 96.9 2.1 44.3 1.7 | 71.2 54.5 2.8 3.7 38.0 33.2 2.3 3.0 |  | 43.1 4.7 29.5 3.8 | 34.9 5.8 26.6 4.6 | 28.8 7.0 24.2 5.6 | 24.2 8.4 22.2 6.7 | 20.6 9.8 20.5 7.8 | 17.8 11.4 19.0 9.1 | 15.5 13.1 17.7 10.4 | 13.6 14.8 16.6 11.9 | 73,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.11E+6 |
| 76 x 6 | 85.1 | 2,973.00 | U Du C Dc | 1,298.7 0.2 198.1 0.2 | 577.5 0.4 132.1 0.3 | 324.9 0.8 99.1 0.6 | 208.0 1.2 79.3 1.0 | 144.4 1.7 66.0 1.4 | 106.1 81.3 2.3 3.0 56.6 49.5 1.9 2.4 |  | 64.2 3.9 44.0 3.1 | 52.0 4.8 39.6 3.8 | 43.0 5.8 36.0 4.6 | 36.1 6.8 33.0 5.5 | 30.8 8.0 30.5 6.4 | 26.5 9.3 28.3 7.5 | 23.1 10.7 26.4 8.6 | 20.3 12.2 24.8 9.7 | 109,510 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.84E+6 |
| 76 x 10 | 124.4 | 3,281.00 | U Du C Dc | 1,927.8 0.2 294.0 0.2 | 857.2 0.4 196.0 0.3 | 482.3 0.8 147.0 0.6 | 308.7 1.2 117.6 1.0 | 214.4 1.7 98.0 1.4 | 157.5 120.6 2.3 3.0 84.0 73.5 1.9 2.4 |  | 95.3 3.9 65.4 3.1 | 77.2 4.8 58.8 3.8 | 63.8 5.8 53.5 4.6 | 53.6 6.8 49.0 5.5 | 45.7 8.0 45.3 6.4 | 39.4 9.3 42.0 7.5 | 34.3 10.7 39.2 8.6 | 30.2 12.2 36.8 9.7 | 162,560 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.71E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

48HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 54 | 102 | 150 | 198 | 246 | 294 | 342 | 390 | 438 | 486 |
| 10mm Bars | 58 | 106 | 154 | 202 | 250 | 298 | 346 | 394 | 442 | 490 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 534 | 582 | 630 | 678 | 726 | 774 | 822 | 870 | 918 | 966 |
| 10mm Bars | 538 | 586 | 634 | 682 | 730 | 778 | 826 | 874 | 922 | 970 |

84                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - WIDE-GAP

| Grating Type: 48HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 89 x 6 | 98.4 | 3,368.00 | U Du C Dc | 1,811.1 0.2 276.2 0.1 | 805.3 0.4 184.2 0.3 | 453.1 0.6 138.1 0.5 | 290.0 1.0 110.5 0.8 | 201.4 1.4 92.1 1.2 | 148.0 2.0 79.0 1.6 | 113.3 2.6 69.1 2.1 | 89.5 3.3 61.4 2.6 | 72.5 4.0 55.3 3.2 | 59.9 4.9 50.2 3.9 | 50.4 5.8 46.1 4.6 | 42.9 6.8 42.5 5.4 | 37.0 7.9 39.5 6.3 | 32.2 9.1 36.8 7.2 | 28.3 10.3 34.6 8.2 | 152,720 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.33E+6 |
| 89 x 10 | 146.4 | 3,717.00 | U Du C Dc | 2,688.4 0.2 410.0 0.1 | 1,195.4 0.4 273.4 0.3 | 672.5 0.6 205.1 0.5 | 430.5 1.0 164.1 0.8 | 299.0 1.4 136.7 1.2 | 219.7 2.0 117.2 1.6 | 168.2 2.6 102.5 2.1 | 132.9 3.3 91.2 2.6 | 107.6 4.0 82.0 3.2 | 89.0 4.9 74.6 3.9 | 74.8 5.8 68.4 4.6 | 63.7 6.8 63.1 5.4 | 54.9 7.9 58.6 6.3 | 47.8 9.1 54.7 7.2 | 42.1 10.3 51.3 8.2 | 226690 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.40E+6 |
| 102 x 6 | 111.7 | 3,748.00 | U Du C Dc | 2,408.6 0.1 367.3 0.1 | 1,070.9 0.3 244.9 0.3 | 602.5 0.6 183.7 0.4 | 385.7 0.9 147.0 0.7 | 267.9 1.3 122.5 1.0 | 196.8 1.7 105.0 1.4 | 150.7 2.2 91.9 1.8 | 119.1 2.8 81.7 2.3 | 96.4 3.5 73.5 2.8 | 79.7 4.2 66.8 3.4 | 67.0 5.0 61.3 4.0 | 57.1 5.9 56.5 4.7 | 49.2 6.8 52.5 5.5 | 42.9 7.9 49.0 6.3 | 37.7 8.9 46.0 7.1 | 203,100 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.71E+6 |
| 102 x 10 | 166.1 | 4,137.00 | U Du C Dc | 3,575.2 0.1 545.2 0.1 | 1,589.7 0.3 363.6 0.3 | 894.4 0.6 272.7 0.4 | 572.5 0.9 218.2 0.7 | 397.6 1.3 181.8 1.0 | 292.1 1.7 155.9 1.4 | 223.7 2.2 136.4 1.8 | 176.7 2.8 121.2 2.3 | 143.2 3.5 109.1 2.8 | 118.3 4.2 99.2 3.4 | 99.4 5.0 90.9 4.0 | 84.7 5.9 83.9 4.7 | 73.0 6.8 77.9 5.5 | 63.6 7.9 72.7 6.3 | 56.0 8.9 68.2 7.1 | 301,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.41E+6 |
| 127 x 10 | 205.5 | 4,936.00 | U Du C Dc | 5,727.4 0.1 873.4 0.1 | 2,546.6 0.2 582.4 0.2 | 1,432.8 0.4 436.9 0.4 | 917.1 0.7 349.5 0.6 | 636.9 1.0 291.3 0.8 | 468.0 1.4 249.7 1.1 | 358.3 1.8 218.5 1.4 | 283.1 2.2 194.2 1.8 | 229.3 2.8 174.8 2.2 | 189.5 3.3 158.9 2.7 | 159.3 4.0 145.7 3.2 | 135.7 4.7 134.4 3.7 | 117.0 5.4 124.8 4.3 | 101.9 6.2 116.5 5.0 | 89.6 7.1 109.3 5.6 | 482,950 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 29.22E+6 |
| 152 x 10 | 245.0 | 5,695.00 | U Du C Dc | 8,384.3 0.1 1,278.6 0.1 | 3,728.0 0.2 852.6 0.2 | 2,097.5 0.4 639.5 0.3 | 1,342.5 0.6 511.6 0.5 | 932.4 0.8 426.4 0.7 | 685.1 1.1 365.5 0.9 | 524.5 1.5 319.8 1.2 | 414.5 1.8 284.3 1.5 | 335.7 2.3 255.9 1.8 | 277.5 2.8 232.6 2.2 | 233.2 3.3 213.2 2.6 | 198.7 3.9 196.8 3.1 | 171.3 4.5 182.8 3.6 | 149.2 5.1 170.6 4.1 | 131.2 5.8 160.0 4.7 | 706,990 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 51.75E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

48HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 54 | 102 | 150 | 198 | 246 | 294 | 342 | 390 | 438 | 486 |
| 10mm Bars | 58 | 106 | 154 | 202 | 250 | 298 | 346 | 394 | 442 | 490 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 534 | 582 | 630 | 678 | 726 | 774 | 822 | 870 | 918 | 966 |
| 10mm Bars | 538 | 586 | 634 | 682 | 730 | 778 | 826 | 874 | 922 | 970 |

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              85

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 60HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 26.6 | 1,312.00 | U Du C Dc | 136.0 0.5 20.7 0.4 | 60.5 1.2 13.8 0.9 | 34.0 2.1 10.4 1.7 | 21.8 3.3 8.3 2.6 | 15.1 4.7 6.9 3.8 | 11.1 6.4 5.9 5.2 | 8.5 8.4 5.2 6.7 | 6.7 10.6 4.6 8.5 |  | 9 |  |  | 6 |  |  | 11,470 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 145.66E+3 |
| 32 x 6 | 32.0 | 1,552.00 | U Du C Dc | 213.2 0.4 32.5 0.3 | 94.8 0.9 21.7 0.8 | 53.3 1.7 16.3 1.3 | 34.1 2.6 13.0 2.1 | 23.7 3.8 10.8 3.0 | 17.4 5.1 9.3 4.1 | 13.3 10.5 6.7 8.5 8.1 7.2 5.4 6.8 |  | 8.5 10.5 6.5 8.4 | 7.1 12.7 5.9 10.2 | 9 |  |  |  |  | 17,980 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 285.84E+3 |
| 38 x 6 | 37.2 | 1,778.00 | U Du C Dc | 306.0 0.4 46.7 0.3 | 136.1 0.8 31.1 0.6 | 76.6 1.4 23.3 1.1 | 49.0 2.2 18.7 1.8 | 34.0 3.2 15.6 2.5 | 25.0 4.3 13.3 3.4 | 19.1 15.1 5.6 7.1 11.7 10.4 4.5 5.7 |  | 12.3 8.8 9.3 7.0 | 10.1 10.6 8.5 8.5 | 8.5 12.6 7.8 10.1 |  |  |  |  | 25,810 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 491.61E+3 |
| 38 x 10 | 53.4 | 1,962.00 | U Du C Dc | 454.3 0.4 69.3 0.3 | 202.0 0.8 46.2 0.6 | 113.6 1.4 34.7 1.1 | 72.7 2.2 27.7 1.8 | 50.5 3.2 23.1 2.5 | 37.1 4.3 19.8 3.4 | 28.4 22.5 5.6 7.1 17.3 15.4 4.5 5.7 |  | 18.2 8.8 13.9 7.0 | 15.0 10.6 12.6 8.5 | 12.6 12.6 11.6 10.1 | 10.8 14.8 10.7 11.8 | 9. 9. |  |  | 38,310 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 729.74E+3 |
| 51 x 6 | 47.9 | 2,206.00 | U Du C Dc | 544.1 0.3 83.0 0.2 | 241.9 0.6 55.3 0.5 | 136.1 1.1 41.5 0.8 | 87.1 1.6 33.2 1.3 | 60.5 2.4 27.7 1.9 | 44.5 3.2 23.7 2.6 | 34.0 26.9 4.2 5.3 20.8 18.4 3.4 4.3 |  | 21.8 6.6 16.6 5.3 | 18.0 7.9 15.1 6.4 | 15.1 9.5 13.8 7.6 | 12.9 11.1 12.8 8.9 | 11.1 12.9 11.9 10.3 | 9.7 14.8 11.1 11.8 |  | 45,880 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.17E+6 |
| 64 x 6 | 58.5 | 2,608.00 | U Du C Dc | 850.1 0.2 129.6 0.2 | 378.0 0.5 86.4 0.4 | 212.7 0.8 64.8 0.7 | 136.1 1.3 51.9 1.1 | 94.5 1.9 43.2 1.5 | 69.5 2.6 37.1 2.1 | 53.2 42.0 3.4 4.3 32.4 28.8 2.7 3.4 |  | 34.0 5.3 25.9 4.2 | 28.1 6.4 23.6 5.1 | 23.6 7.6 21.6 6.1 | 20.1 8.9 20.0 7.1 | 17.4 10.3 18.5 8.2 | 15.1 11.8 17.3 9.5 | 13.3 13.4 16.2 10.8 | 71,680 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.28E+6 |
| 76 x 6 | 69.1 | 2,990.00 | U Du C Dc | 1,224.2 0.2 186.7 0.1 | 544.3 0.4 124.5 0.3 | 306.2 0.7 93.4 0.6 | 196.0 1.1 74.7 0.9 | 136.1 1.6 62.3 1.3 | 100.0 2.1 53.4 1.7 | 76.6 60.5 2.8 3.5 46.7 41.5 2.2 2.8 |  | 49.0 4.4 37.4 3.5 | 40.5 5.3 34.0 4.2 | 34.0 6.3 31.1 5.0 | 29.0 7.4 28.7 5.9 | 25.0 8.6 26.7 6.9 | 21.8 9.9 24.9 7.9 | 19.2 11.2 23.4 9.0 | 103,230 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.93E+6 |
| 76 x 10 | 102.9 | 3,300.00 | U Du C Dc | 1,817.1 0.2 277.1 0.1 | 808.0 0.4 184.8 0.3 | 454.6 0.7 138.6 0.6 | 291.0 1.1 110.9 0.9 | 202.1 1.6 92.4 1.3 | 148.5 2.1 79.2 1.7 | 113.7 89.8 2.8 3.5 69.3 61.6 2.2 2.8 |  | 72.8 4.4 55.5 3.5 | 60.1 5.3 50.4 4.2 | 50.5 6.3 46.2 5.0 | 43.1 7.4 42.7 5.9 | 37.1 8.6 39.6 6.9 | 32.3 9.9 37.0 7.9 | 28.4 11.2 34.7 9.0 | 153,230 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.84E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

### 60HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 66 | 126 | 186 | 246 | 306 | 366 | 426 | 486 | 546 | 606 |
| 10mm Bars | 70 | 130 | 190 | 250 | 310 | 370 | 430 | 490 | 550 | 610 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 666 | 726 | 786 | 846 | 906 | 966 | 1026 | 1086 | 1146 | 1206 |
| 10mm Bars | 670 | 730 | 790 | 850 | 910 | 970 | 1030 | 1090 | 1150 | 1210 |

86                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 60HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Smooth |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size | Approx. Wieght | Ped. Span | Load / | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (mm) | (kg/m2) | (mm) | Deflection | 305 |  | 457 |  | 610 |  | 762 |  | 915 | 1067 |  | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 |  | 1981 | 2134 |  | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 | 79.8 | 3,356.00 | U Du C Dc | 1,666.2 0.2 254.1 0.1 |  | 740.9 0.3 169.4 0.3 |  | 416.8 0.6 127.1 0.5 |  | 266.8 0.9 101.7 0.8 |  | 185.3 1.4 84.7 1.1 | 136.1 1.8 72.6 1.5 |  | 104.2 2.4 63.6 1.9 |  | 82.4 3.0 56.5 2.4 |  | 66.7 3.8 50.8 3.0 |  | 55.1 4.5 46.2 3.6 |  | 46.3 5.4 42.4 4.3 |  | 39.5 6.3 39.1 5.1 | 34.0 7.4 36.3 5.9 |  | 29.7 8.4 33.9 6.8 |  | 26.1 9.6 31.8 7.7 | 140,500 6.25E+6 |
| 89 x 10 | 118.7 | 3,705.00 | U Du C Dc | 2,473.3 0.2 377.2 0.1 |  | 1,099.7 0.3 251.5 0.3 |  | 618.7 0.6 188.7 0.5 |  | 396.0 0.9 150.9 0.8 |  | 275.1 1.4 125.8 1.1 | 202.1 1.8 107.8 1.5 |  | 154.7 2.4 94.3 1.9 |  | 122.3 3.0 83.9 2.4 |  | 99.0 3.8 75.5 3.0 |  | 81.9 4.5 68.6 3.6 |  | 68.8 5.4 62.9 4.3 |  | 58.6 6.3 58.1 5.1 | 50.5 7.4 53.9 5.9 |  | 44.0 8.4 50.3 6.8 |  | 38.7 9.6 47.2 7.7 | 208560 9.27E+6 |
| 102 x 6 | 90.4 | 3,710.00 | U Du C Dc | 2,176.3 0.1 331.9 0.1 |  | 967.7 0.3 221.3 0.2 |  | 544.4 0.5 166.0 0.4 |  | 348.5 0.8 132.8 0.7 |  | 242.0 1.2 110.7 0.9 | 177.8 1.6 94.9 1.3 |  | 136.2 2.1 83.0 1.7 |  | 107.6 2.7 73.8 2.1 |  | 87.1 3.3 66.4 2.6 |  | 72.0 4.0 60.4 3.2 |  | 60.5 4.7 55.3 3.8 |  | 51.6 5.6 51.1 4.4 | 44.5 6.4 47.4 5.1 |  | 38.7 7.4 44.3 5.9 |  | 34.1 8.4 41.5 6.7 | 183,510 9.32E+6 |
| 102 x 10 | 134.5 | 4,095.00 | U Du C Dc | 3,230.5 0.1 492.6 0.1 |  | 1,436.4 0.3 328.5 0.2 |  | 808.1 0.5 246.4 0.4 |  | 517.3 0.8 197.1 0.7 |  | 359.3 1.2 164.3 0.9 | 264.0 1.6 140.8 1.3 |  | 202.1 2.1 123.2 1.7 |  | 159.7 2.7 109.5 2.1 |  | 129.4 3.3 98.6 2.6 |  | 106.9 4.0 89.6 3.2 |  | 89.8 4.7 82.2 3.8 |  | 76.5 5.6 75.8 4.4 | 66.0 6.4 70.4 5.1 |  | 57.5 7.4 65.7 5.9 |  | 50.6 8.4 61.6 6.7 | 272,400 13.84E+6 |
| 127 x 10 | 166.1 | 4,841.00 | U Du C Dc | 5,047.6 0.1 769.8 0.1 |  | 2,244.3 0.2 513.3 0.2 |  | 1,262.7 0.4 385.0 0.3 |  | 808.2 0.7 308.0 0.5 |  | 561.3 0.9 256.7 0.8 | 412.4 1.3 220.0 1.0 |  | 315.8 1.7 192.5 1.3 |  | 249.5 2.1 171.1 1.7 |  | 202.1 2.6 154.0 2.1 |  | 167.0 3.2 140.0 2.5 |  | 140.4 3.8 128.4 3.0 |  | 119.6 4.4 118.5 3.6 | 103.1 5.1 110.0 4.1 |  | 89.8 5.9 102.7 4.7 |  | 79.0 6.7 96.3 5.4 | 425,630 27.03E+6 |
| 152 x 10 | 197.7 | 5,550.00 | U Du C Dc | 7,268.5 0.1 1,108.4 0.1 |  | 3,231.9 0.2 739.1 0.2 |  | 1,818.3 0.4 554.4 0.3 |  | 1,163.9 0.5 443.6 0.4 |  | 808.3 0.8 369.6 0.6 | 593.9 1.1 316.8 0.9 |  | 454.7 1.4 277.2 1.1 |  | 359.3 1.8 246.4 1.4 |  | 291.0 2.2 221.8 1.8 |  | 240.5 2.6 201.6 2.1 |  | 202.1 3.2 184.8 2.5 |  | 172.2 3.7 170.6 3.0 | 148.5 4.3 158.4 3.4 |  | 129.4 4.9 147.9 3.9 |  | 113.8 5.6 138.7 4.5 | 612,900 46.70E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 60HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  | 2 | 3 |  |  | 4 |  |  |  | 5 |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  | 10 |  | 11 |  |
| 6mm Bars |  | 66 | 126 |  |  | 186 |  |  |  | 246 |  |  | 306 |  |  |  | 366 |  |  |  | 426 |  |  |  | 486 |  |  | 546 |  | 606 |  |
| 10mm Bars |  | 70 | 130 |  |  | 190 |  |  |  | 250 |  |  | 310 |  |  |  | 370 |  |  |  | 430 |  |  |  | 490 |  |  | 550 |  | 610 |  |
| # of Bars |  | 12 | 13 |  |  | 14 |  |  |  | 15 |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |  | 19 |  |  | 20 |  | 21 |  |
| 6mm Bars |  | 666 | 726 |  |  | 786 |  |  |  | 846 |  |  | 906 |  |  |  | 966 |  |  |  | 1026 |  |  |  | 1086 |  |  | 1146 |  | 1206 |  |
| 10mm Bars |  | 670 | 730 |  |  | 790 |  |  |  | 850 |  |  | 910 |  |  |  | 970 |  |  |  | 1030 |  |  |  | 1090 |  |  | 1150 |  | 1210 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 87 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LOAD TABLES | HEAVY DUTY, METRIC

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 60HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing Bar Size (mm) | Approx. Wieght (kg/m2) | Ped. Span (mm) | Load / Deflection | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 305 | 457 | 610 | 762 | 915 | 1067 | 1219 | 1372 | 1524 | 1677 | 1829 | 1981 | 2134 | 2286 | 2438 | Sx (mm3)/m |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ix (mm4)/m |
| 25 x 6 | 26.6 | 1,072.00 | U Du C Dc | 79.3 0.7 12.1 0.6 | 35.3 1.5 8.1 1.2 | 19.8 2.8 6.1 2.2 | 12.7 4.3 4.8 3.4 | 8.8 6.2 4.0 5.0 | 6.5 8.4 3.5 6.7 | 5.0 11.0 3.0 8.8 |  |  |  |  |  |  |  |  | 6,690 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 64.90E+3 |
| 32 x 6 | 32.0 | 1,327.00 | U Du C Dc | 140.3 0.5 21.4 0.4 | 62.4 1.2 14.3 0.9 | 35.1 2.1 10.7 1.7 | 22.5 3.2 8.6 2.6 | 15.6 4.7 7.1 3.7 | 11.5 8.8 6.3 8.3 6.1 5.4 5.1 6.6 |  | 6.9 10.5 4.8 8.4 |  |  |  |  |  |  |  | 11,830 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 152.65E+3 |
| 38 x 6 | 37.2 | 1,563.00 | U Du C Dc | 217.2 0.4 33.1 0.3 | 96.6 0.9 22.1 0.7 | 54.3 1.7 16.6 1.3 | 34.8 2.6 13.3 2.1 | 24.2 3.7 11.0 3.0 | 17.8 13.6 5.1 6.7 9.5 8.3 4.1 5.3 |  | 10.7 8.4 7.4 6.7 | 8.7 10.4 6.6 8.3 | 7.2 12.6 6.0 10.1 |  |  |  |  |  | 18,320 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 294.01E+3 |
| 38 x 10 | 53.4 | 1,726.00 | U Du C Dc | 322.5 0.4 49.2 0.3 | 143.4 0.9 32.8 0.7 | 80.7 1.7 24.6 1.3 | 51.6 2.6 19.7 2.1 | 35.9 3.7 16.4 3.0 | 26.3 20.2 5.1 6.7 14.1 12.3 4.1 5.3 |  | 15.9 8.4 10.9 6.7 | 12.9 10.4 9.8 8.3 | 10.7 12.6 8.9 10.1 | 9.0 15.0 8.2 12.0 |  |  |  |  | 27,190 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 436.42E+3 |
| 51 x 6 | 47.9 | 2,007.00 | U Du C Dc | 423.1 0.3 64.5 0.2 | 188.1 0.7 43.0 0.5 | 105.9 1.2 32.3 1.0 | 67.8 1.9 25.8 1.5 | 47.1 2.7 21.5 2.1 | 34.6 26.5 3.7 4.8 18.4 16.1 2.9 3.8 |  | 20.9 6.0 14.3 4.8 | 16.9 7.4 12.9 6.0 | 14.0 9.0 11.7 7.2 | 11.8 10.7 10.8 8.6 | 10.0 12.6 9.9 10.1 | 8.6 14.6 9.2 11.7 |  |  | 35,680 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 799.25E+3 |
| 64 x 6 | 58.5 | 2,421.00 | U Du C Dc | 697.1 0.2 106.3 0.2 | 309.9 0.5 70.9 0.4 | 174.4 0.9 53.2 0.7 | 111.6 1.5 42.5 1.2 | 77.5 2.1 35.4 1.7 | 57.0 43.6 2.8 3.7 30.4 26.6 2.3 3.0 |  | 34.5 4.7 23.6 3.8 | 27.9 5.8 21.3 4.6 | 23.1 7.0 19.3 5.6 | 19.4 8.4 17.7 6.7 | 16.5 9.8 16.4 7.8 | 14.2 11.4 15.2 9.1 | 12.4 13.1 14.2 10.4 | 10.9 14.8 13.3 11.9 | 58,780 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.69E+6 |
| 76 x 6 | 69.1 | 2,812.00 | U Du C Dc | 1,039.0 0.2 158.4 0.2 | 462.0 0.4 105.7 0.3 | 259.9 0.8 79.2 0.6 | 166.4 1.2 63.4 1.0 | 115.5 1.7 52.8 1.4 | 84.9 65.0 2.3 3.0 45.3 39.6 1.9 2.4 |  | 51.4 3.9 35.2 3.1 | 41.6 4.8 31.7 3.8 | 34.4 5.8 28.8 4.6 | 28.9 6.8 26.4 5.5 | 24.6 8.0 24.4 6.4 | 21.2 9.3 22.6 7.5 | 18.5 10.7 21.1 8.6 | 16.3 12.2 19.8 9.7 | 87,610 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.08E+6 |
| 76 x 10 | 100.8 | 3,103.00 | U Du C Dc | 1,542.2 0.2 235.2 0.2 | 685.7 0.4 156.8 0.3 | 385.8 0.8 117.6 0.6 | 247.0 1.2 94.1 1.0 | 171.5 1.7 78.4 1.4 | 126.0 96.5 2.3 3.0 67.2 58.8 1.9 2.4 |  | 76.2 3.9 52.3 3.1 | 61.8 4.8 47.1 3.8 | 51.0 5.8 42.8 4.6 | 42.9 6.8 39.2 5.5 | 36.5 8.0 36.2 6.4 | 31.5 9.3 33.6 7.5 | 27.4 10.7 31.4 8.6 | 24.1 12.2 29.4 9.7 | 130,050 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.56E+6 |

Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional.

60HW102 (mm)

| # of Bars | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6mm Bars | 66 | 126 | 186 | 246 | 306 | 366 | 426 | 486 | 546 | 606 |
| 10mm Bars | 70 | 130 | 190 | 250 | 310 | 370 | 430 | 490 | 550 | 610 |
| # of Bars | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 6mm Bars | 666 | 726 | 786 | 846 | 906 | 966 | 1026 | 1086 | 1146 | 1206 |
| 10mm Bars | 670 | 730 | 790 | 850 | 910 | 970 | 1030 | 1090 | 1150 | 1210 |

88                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

LOAD TABLES - HD METRIC

LOAD TABLES - EXTRA-WIDE-GAP

| Grating Type: 60HW102 Design Code: NAAMM MBG 534-19 Material: ASTM A1011CS Grade 250 Surface: Serrated |  | U = Safe Uniform Load (kPa) Du = Deflection Due to Safe Uniform Load (mm) C = Safe Concentrated Load (kN/meter of grating width) Dc = Deflection Due to Safe Concentrated Load (mm) Allowable Extreme Fiber Stress = 137.9 MPa |
| --- | --- | --- |

| Bearing | Approx. |  | Ped. |  | SPAN (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Section Properties |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bar Size (mm) | Wieght (kg/m2) |  | Span (mm) | Load / Deflection | 305 |  | 457 |  | 610 |  | 762 |  | 915 |  | 1067 | 1219 |  | 1372 |  | 1524 |  | 1677 |  | 1829 |  | 1981 |  | 2134 | 2286 |  | 2438 | Sx (mm3)/m Ix (mm4)/m |
| 89 x 6 |  | 79.8 | 3,185.00 | U Du C Dc | 1,448.9 0.2 221.0 0.1 |  | 644.2 0.4 147.3 0.3 |  | 362.5 0.6 110.5 0.5 |  | 232.0 1.0 88.4 0.8 |  | 161.1 1.4 73.7 1.2 |  | 118.4 2.0 63.2 1.6 | 90.6 2.6 55.3 2.1 |  | 71.6 3.3 49.1 2.6 |  | 58.0 4.0 44.2 3.2 |  | 47.9 4.9 40.2 3.9 |  | 40.3 5.8 36.8 4.6 |  | 34.3 6.8 34.0 5.4 |  | 29.6 7.9 31.6 6.3 | 25.8 9.1 29.5 7.2 |  | 22.7 10.3 27.6 8.2 | 122,180 5.06E+6 |
| 89 x 10 |  | 118.7 | 3,516.00 | U Du C Dc | 2,150.7 0.2 328.0 0.1 |  | 956.3 0.4 218.7 0.3 |  | 538.0 0.6 164.0 0.5 |  | 344.4 1.0 131.2 0.8 |  | 239.2 1.4 109.4 1.2 |  | 175.7 2.0 93.8 1.6 | 134.6 2.6 82.0 2.1 |  | 106.3 3.3 72.9 2.6 |  | 86.1 4.0 65.6 3.2 |  | 71.2 4.9 59.7 3.9 |  | 59.8 5.8 54.7 4.6 |  | 51.0 6.8 50.5 5.4 |  | 43.9 7.9 46.9 6.3 | 38.3 9.1 43.8 7.2 |  | 33.7 10.3 41.0 8.2 | 181360 7.52E+6 |
| 102 x 6 |  | 90.4 | 3,544.00 | U Du C Dc | 1,926.9 0.1 293.8 0.1 |  | 856.8 0.3 195.9 0.3 |  | 482.0 0.6 147.0 0.4 |  | 308.5 0.9 117.6 0.7 |  | 214.3 1.3 98.0 1.0 |  | 157.4 1.7 84.0 1.4 | 120.5 2.2 73.5 1.8 |  | 95.3 2.8 65.3 2.3 |  | 77.2 3.5 58.8 2.8 |  | 63.8 4.2 53.5 3.4 |  | 53.6 5.0 49.0 4.0 |  | 45.7 5.9 45.2 4.7 |  | 39.4 6.8 42.0 5.5 | 34.3 7.9 39.2 6.3 |  | 30.2 8.9 36.8 7.1 | 162,480 7.77E+6 |
| 102 x 10 |  | 134.5 | 3,912.00 | U Du C Dc | 2,860.2 0.1 436.2 0.1 |  | 1,271.7 0.3 290.8 0.3 |  | 715.5 0.6 218.2 0.4 |  | 458.0 0.9 174.5 0.7 |  | 318.1 1.3 145.5 1.0 |  | 233.7 1.7 124.7 1.4 | 178.9 2.2 109.1 1.8 |  | 141.4 2.8 97.0 2.3 |  | 114.5 3.5 87.3 2.8 |  | 94.7 4.2 79.3 3.4 |  | 79.5 5.0 72.7 4.0 |  | 67.8 5.9 67.1 4.7 |  | 58.4 6.8 62.3 5.5 | 50.9 7.9 58.2 6.3 |  | 44.8 8.9 54.6 7.1 | 241,180 11.53E+6 |
| 127 x 10 |  | 166.1 | 4,668.00 | U Du C Dc | 4,581.9 0.1 698.7 0.1 |  | 2,037.3 0.2 465.9 0.2 |  | 1,146.2 0.4 349.5 0.4 |  | 733.7 0.7 279.6 0.6 |  | 509.5 1.0 233.0 0.8 |  | 374.4 1.4 199.7 1.1 | 286.7 1.8 174.8 1.4 |  | 226.5 2.2 155.4 1.8 |  | 183.5 2.8 139.8 2.2 |  | 151.6 3.3 127.1 2.7 |  | 127.4 4.0 116.5 3.2 |  | 108.6 4.7 107.6 3.7 |  | 93.6 5.4 99.9 4.3 | 81.5 6.2 93.2 5.0 |  | 71.7 7.1 87.4 5.6 | 386,360 23.37E+6 |
| 152 x 10 |  | 197.7 | 5,386.00 | U Du C Dc | 6,707.5 0.1 1,022.9 0.1 |  | 2,982.4 0.2 682.1 0.2 |  | 1,678.0 0.4 511.6 0.3 |  | 1,074.0 0.6 409.3 0.5 |  | 745.9 0.8 341.1 0.7 |  | 548.1 1.1 292.4 0.9 | 419.6 1.5 255.8 1.2 |  | 331.6 1.8 227.4 1.5 |  | 268.6 2.3 204.7 1.8 |  | 222.0 2.8 186.1 2.2 |  | 186.5 3.3 170.6 2.6 |  | 158.9 3.9 157.5 3.1 |  | 137.0 4.5 146.2 3.6 | 119.4 5.1 136.5 4.1 |  | 105.0 5.8 128.0 4.7 | 565,590 41.40E+6 |
| Spans and loads in red exceed a deflection of 6mm for uniform loads of 5kPa. Experience has shown that 6mm deflection is the maximum deflection to give pedestrian comfort, but can be exceeded for other types of loads at the discretion of the specifying professional. 60HW102 (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of Bars |  |  | 2 | 3 |  |  | 4 |  |  |  | 5 |  |  |  | 6 |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  | 10 |  | 11 |  |
| 6mm Bars |  |  | 66 | 126 |  |  | 186 |  |  |  | 246 |  |  |  | 306 |  |  | 366 |  |  |  | 426 |  |  |  | 486 |  |  | 546 |  | 606 |  |
| 10mm Bars |  |  | 70 | 130 |  |  | 190 |  |  |  | 250 |  |  |  | 310 |  |  | 370 |  |  |  | 430 |  |  |  | 490 |  |  | 550 |  | 610 |  |
| # of Bars |  |  | 12 | 13 |  |  | 14 |  |  |  | 15 |  |  |  | 16 |  |  | 17 |  |  |  | 18 |  |  |  | 19 |  |  | 20 |  | 21 |  |
| 6mm Bars |  |  | 666 | 726 |  |  | 786 |  |  |  | 846 |  |  |  | 906 |  |  | 966 |  |  |  | 1026 |  |  |  | 1086 |  |  | 1146 |  | 1206 |  |
| 10mm Bars |  |  | 670 | 730 |  |  | 790 |  |  |  | 850 |  |  |  | 910 |  |  | 970 |  |  |  | 1030 |  |  |  | 1090 |  |  | 1150 |  | 1210 |  |
| VULCRAFT GRATING ¦ www.vulcraft.com/grating 89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## LOAD TABLES | STAIR TREADS, IMPERIAL

Welded steel stair treads are the most widely used for their strength and ease of installation and are

universally used in most industrial and commercial applications. Both can be ordered with a serrated surface for additional safety.

Rectangular bar stair treads provide a high strength and stiffness-to-weight ratio and are available with a serrated surface when additional safety is required.

# Stair Tread Types

Checker Plate Nosing (CP)

Cast Aluminum Abrasive Nosing (CAA)

Standard Sizes

Bearing Bars: 1” x 3/16”, 1-1/4” x 3/16”, 1-1/2” x 3/16”

Widths: 9-3/4”, 10-15/16”, 12-1/8”

Lengths: 30”, 36”

All standard sizes available in painted and galvanized finishes.

Custom sizes can be provided also per the dimensions shown in the tables on p. 91.

90

All stair treads are custom fabricated to meet the size, width and length specifications of a particular job. In

addition, end plates can be custom fabricated to meet special bolt hole size or location requirements.

Steel nosings are available to add strength at the point of greatest impact and provide a definitive visible edge for extra safety. Choose our checkered plate nosing for normal use.

Stair Tread Tolerances and Details

| No. of Bearing Bars and Nosing | Bearing Bar | **Bolt Hole Spacing “A” |
| --- | --- | --- |
|  | 3/16” |  |

| 5 | 6-3/16” | 2-1/2” |
| --- | --- | --- |
| 6 | 7-3/8” | 4-1/2” |
| 7 | 8-9/16” | 4-1/2” |
| 8 | 9-3/4” | 7” |
| 9 | 10-15/16” | 7” |
| 10 | 12-1/8” | 7” |

Typical Stair Tread Stringer Detail

Tread Length and Width Tolerance

VULCRAFT GRATING  |  www.vulcraft.com/grating

End Plate Dimensions

| Grating Depth | “B” Dimension | “C” Dimension |
| --- | --- | --- |
| up to 1-1/4” | 1-3/4” | 2-1/2” |
| 1-1/2” | 2-1/4” | 3” |

See Tread Width and Bolt Hole Spacing for “A” Dimension

Tread Width and Bolt Hole Spacing

11W4

19W4

| No. of Bearing Bars and Nosing | Bearing Bar | **Bolt Hole Spacing “A” |
| --- | --- | --- |
|  | 3/16” |  |

Tread Width

Tread Width

| 9 | 6-15/16” | 2-1/2” |
| --- | --- | --- |
| 11 | 8-5/16” | 4-1/2” |
| 13 | 8-9/16” | 4-1/2” |
| 15 | 11-1/16” | 7” |
| 16 | 11-3/4” | 7” |
| 18 | 13-1/8” | 7” |

**See Drawing Above

**See Drawing Above

LOAD TABLES - STAIR TREADS IMPERIAL

Maximum Tread Length (in)

| Bar Size (in) | Smooth |  |  | Serrated |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 19W4 | 11W4 | 7W4 | 19W4 | 11W4 | 7W4 |
| 1 x 3/16 | 41 | 51 | 59 | 34 | 45 | 51 |
| 11/4 x 3/16 | 56 | 66 | 66 | 50 | 58 | 66 |
| 11/2 x 3/16 | 66 | 66 | 76 | 63 | 66 | 68 |

Data Created Based on NAAMM 531-17:

1. 300-lb point load at center, loading only first five bars, up to 5‘-6”.

2. 300-lb point loads at third-points, loading only first five bars, when > 5’-6”.

3. L/240 deflection limit on both.

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                               91

## LOAD TABLES | STAIR TREADS, METRIC

| Welded steel stair treads are the most widely used for their strength and ease of installation and are universally used in most industrial and commercial applications. Both can be ordered with a serrated surface for additional safety. Rectangular bar stair treads provide a high strength and stiffness-to-weight ratio and are available with a serrated surface when additional safety is required. | All stair treads are custom fabricated to meet the size, width and length specifications of a particular job. In addition, end plates can be custom fabricated to meet special bolt hole size or location requirements. Steel nosings are available to add strength at the point of greatest impact and provide a definitive visible edge for extra safety. Choose our checkered plate nosing for normal use. |
| --- | --- |

Stair Tread Types

Stair Tread Tolerances and Details

| No. of Bearing Bars and Nosing | Bearing Bar | **Bolt Hole Spacing “A” |
| --- | --- | --- |
|  | 5mm |  |

Checker Plate Nosing (CP)

| 5 | 157mm | 64mm |
| --- | --- | --- |
| 6 | 187mm | 114mm |
| 7 | 217mm | 114mm |
| 8 | 248mm | 178mm |
| 9 | 278mm | 178mm |
| 10 | 308mm | 178mm |

Typical Stair Tread Stringer Detail

Cast Aluminum Abrasive Nosing (CAA)

Standard Sizes

Bearing Bars: 25mm x 5mm, 32mm x 5mm, 38mm x 5mm

Widths: 248mm, 278mm, 308mm

Lengths: 762mm, 914mm

Tread Length and Width Tolerance

All standard sizes available in painted and galvanized finishes.

Custom sizes can be provided also per the dimensions shown in the tables on p. 93.

92                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

End Plate Dimensions

| Grating Depth | “B” Dimension | “C” Dimension |
| --- | --- | --- |
| up to 32m | 44mm | 64mm |
| 38mm | 57mm | 76mm |

See Tread Width and Bolt Hole Spacing for “A” Dimension

Tread Width and Bolt Hole Spacing

11W4

19W4

| No. of Bearing Bars and Nosing | Bearing Bar | **Bolt Hole Spacing “A” |
| --- | --- | --- |
|  | 5mm |  |

Tread Width

Tread Width

| 9 | 176mm | 64mm |
| --- | --- | --- |
| 11 | 211mm | 115m |
| 13 | 217mm | 115mm |
| 15 | 281mm | 178mm |
| 16 | 298mm | 178mm |
| 18 | 333mm | 178mm |

**See Drawing Above

**See Drawing Above

LOAD TABLES - STAIR TREADS METRIC

Maximum Tread Length (in)

| Bar Size (mm) | Smooth |  |  | Serrated |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 19W4 | 11W4 | 7W4 | 19W4 | 11W4 | 7W4 |
| 25 x 5 | 41 | 51 | 59 | 34 | 45 | 51 |
| 32 x 5 | 56 | 66 | 66 | 50 | 58 | 66 |
| 38 x 5 | 66 | 66 | 76 | 63 | 66 | 68 |

Data Created Based on NAAMM 531-17:

1. 1.32kN point load at center, loading only first five bars, up to 1.7m.

2. 1.32kN point loads at third-points, loading only first five bars, when > 1.7m.

3. L/240 deflection limit on both.

VULCRAFT GRATING  |  www.vulcraft.com/grating                                                                                                              93

Vulcraft Grating Plant Locations

WEST

GratingSalesWest@vulcraft.com

Brigham City, Utah

1875 West Highway 13 South Brigham City, UT 84302

EAST

GratingSalesEast@vulcraft.com

Grapeland, Texas

175 County Road 2345 Grapeland, TX 75844

94                                                                                                              VULCRAFT GRATING  |  www.vulcraft.com/grating

www.vulcraft.com

Powerful Partnerships. Powerful Results