LICHTGITTER -

QUALITY SINCE 1929

I   li      t   tt

TABLE OF CONTENT

Lichtgitter presentation

Products + Gratings + Perforated metal planks + GRP-Products + Spiral Staircases

Applications + Architecture + Storage and Logistics + Offshore

A BRIGHT OUTLOOK SINCE 1929

LICHTGITTER WAS ESTABLISHED IN 1929 IN ORDER TO CARRY OUT THE SPECIALIZED MANUFACTURING OF

GRATINGS.

By the continuous monitoring of our performance, and quality

systems, together with innovation in manufacturing techniques, we have ensured Lichtgitter’s place in the forefront of manufac- turers of industrial floor coverings with subsidiaries all over the world.

Apart from the use in the industrial and construction sector,

gratings are also increasingly used in architectural applications. Whether as facade gratings, sun protection gratings, ceiling grids or as walkways or platforms, there are no limits to the use of

gratings.

Besides the production of pressure-locked gratings, forge-welded gratings and perforated metal planks, a various offer of GRP pro- ducts are part of the production range. Our current product range also includes profiled chequer plates, spiral staircases, stairtreads and ladder rungs. Together with our steel service centre powder coating and hot dip galvanizing plants we provide a large vertical production depth for the benefit of our customers.

FOUNDED

95EN0MATPIES

SUBSIDIARIES

MORE THAN

1,500

EXPERTS

IN

WORLDWIDE

COUNTRIES

THE LICHTGITTER GROUP

To meet the demands of our international customers and to put into effect the integration of the European and non-European

P R ES ENTAT ION LICHGITTER

grating markets, Lichtgitter has established subsidiaries and

partners, not only throughout Europe, but also worldwide. This

has formed the basis for the creation of a global network for both distribution and production outlets.

Decades of experience in manufacturing of gratings are not always sufficient in itself. Only fully verified standards and properly

followed instructions, along with further innovative development of production procedures and materials, can satisfy our client’s

demands and expectations, in respect of quality.

Certfied according

DIN EN ISO 9001:2015

OUR COMPREHENSIVE SERVICES HELP TO ACHIEVE YOUR GOALS QUICKLY. FROM THE VERY FIRST CONSULTA- TION, WE OFFER YOU COMPETENT, PRODUCT AND OBJECT-RELATED ADVICE WITH PROFESSIONAL AND TECH- NICAL KNOW-HOW.

through to the preparation of installation plans and com- plete technical documentation. Packing lists, safe packing and commissioning are also part of our service.

In this way, our specialists can develop innovative solu- tions to meet your specific requirements. We are happy to assist you with your project from the planning stage

Technical advice on the calculation and selection of our different products

Preparation of auditable static verification

Preparation of

installation plans

Production of 2D- and 3D drawings

Providing of tender documents

Measurement on-site

Appropriate packaging and consignement

Assembly for testing reasons and trainings

OUR MISSION IS AS SIMPLE AS IT IS CHALLENGING: TO PROVIDE OUR CUSTOMERS WITH INNOVA- TIVE PRODUCTS THAT MEET THE HIGHEST QUALITY STANDARDS. THIS IS WHAT OUR CUSTOMERS HAVE APPRECIATED FOR MANY YEARS.

planks, GRP products and spiral staircases in a wide variety of designs. Tailored to your needs.

In addition to pressure locked and forge welded gratings, our range includes perforated metal

SUBSIDIARIES AND PARTNERS WORLDWIDE

PRODUCTION SITES GERMANY

Austria

Bulgaria

Czech Republic

Denmark

Norway Qatar

Romania

Slovakia

Spain

Sweden

Switzerland

Turkey

United States

Lichtgitter GmbH, 48703 Stadtlohn

Lichtgitter GFK GmbH & Co. KG, 48703 Stadtlohn

Finland

France

Great Britain

Greece

Italy

Lichtgitter Blechprofilroste GmbH & Co. KG, 72172 Sulz/Neckar

Lichtgitter Treppen GmbH & Co. KG, 16278 Pinnow

P R ES ENTAT ION LICHGITTER

Netherlands

PRODUCTS

PRODUCTS

# FORGE WELDED GRATINGS IN USE

PRODUCTS

| GRATINGS FORGE WELDED GRATINGS Forge-welded gratings consist of twisted or round cross bars, which are pressed into the bearing bars under high pressure. They are fully electrically welded. Forge welded gratings are mainly used in the industrial sector as floorings, platforms and all kinds of walkways. Decades of experience have made the Lichtgitter Group one of the world‘s leading manufacturers of forge welded gratings. In addition to custom-made and high-quality special products, Lichtgitter also offers a wide range of industrial gratings, stair treads and panels from stock - of course also with round instead of twisted crossbars and as offshore gratings. | Forge welded grating Forge welded g rat ing w ith round Of fshore-grating cross bars |  |
| --- | --- | --- |

# TECHNICAL DATA

| Bearing bar spacing | Cross bar spacing | Bearing bar thickness | Bearing bar height |
| --- | --- | --- | --- |
| 15,06 mm | 18,40 mm | 2 mm | 25 mm |
| 16,66 mm | 19,00 mm | 3 mm | 30 mm |
| 21,64 mm | 23,40 mm | 4 mm | 40 mm |
| 23,69 mm | 24,00 mm | 5 mm | 45 mm |
| 24,85 mm | 38,10 mm |  | 50 mm |
| 30,16 mm | 50,80 mm |  | 60 mm |
| 33,17 mm | 76,20 mm |  | 70 mm |
| 34,33 mm | 101,60 mm |  | 80 mm |
| 41,46 mm |  |  |  |

All features can be combined to a certain extent with each other. Please contact us to discuss the producibility of the gratings. Our sales advisors will be happy to help.

If a slip resistance class higher than R10 is required, this

requirement is met by means of an anti-slip punched bearing bar.

PRESSURE LOCKED GRATINGS

Pressure locked gratings are used for walkways,

shelvings, platforms and cellar shafts. Due to the  infinite number of variations that can be produced, this type of grating is used where architecture and aesthetics are important. Heavy duty gratings are used where extremely high loads need to be ab-

sorbed, e.g. in traffic areas.

In the manufacturing of pressure locked gratings,

unweakened cross bars are pressed into specially

slotted bearing bars at high pressure. Lichtgitter

manufactures up to 3,500 different types of pressure locked gratings made of steel, stainless steel and

aluminium on its adjustable production facilities.

C lose- meshed pressure locked

gratings

Pressure locked grating anti-slip                            Heavy duty gratings

# TECHNICAL DATA

| Bearing bar spacing | Cross bar spacing | Bearing bar thickness | Bearing bar height |
| --- | --- | --- | --- |
| 22,22 mm | 11,11 mm | 2 mm | 25 mm |
| 33,33 mm | 16,66 mm | 3 mm | 30 mm |
| 44,44 mm | 21,00 mm |  | 35 mm |
| 66,66 mm | 22,22 mm |  | 40 mm |
|  | 33,33 mm |  | 45 mm |
|  | 44,44 mm |  | 50 mm |
|  | 66,66 mm |  | 60 mm |
|  | 100,00 mm |  | 70 mm |

All features can be combined to a certain extent with each other. Please contact us to discuss the producibility of the gratings. Our sales advisors will be happy to help.

If a slip resistance class higher than R10 is required, this require- ment is met by means of an anti-slip punched bearing bar.

PRODUCTS

PRESSURE LOCKED GRATINGS

IN USE

STAIR TREADS AND SPECIAL

GRATINGS IN USE

PRODUCTS

COMBI DECK

Combi decks are gratings with on top welded chequer plates. They combine ideally the advantages of both

products. Chequer plates combine a closed surface

with a slip-resistant effect. Gratings, on the other

hand, have a high load capacity with a low tare weight.

Lichtgitter-Combi deck is used for the industrial sector, e.g. chemical/petrochemical industry, power plants

and cement industry as well as for bulk material and offshore applications.

Forge-welded

stairtreads

Pressure locked stairtreads

STAIRTREADS

Lichtgitter customised and standard treads are made   in same types and surface structures as the associated landings of pressure-locked gratings, forge-welded

gratings and GRP-Gratings. In this way, a staircase has a continuously consistent look. We achieve optimal

safety of the stairs by introducing perforated nosing.

The treads can be made of steel, stainless steel, aluminium or fibreglass.

SPECIAL GRATINGS

Based on our many years of experience, we are constantly developing new/innovative applica- tion solutions for our customers. Lichtgitter-

Full cell gratings                     Powder coated gratings                                 Louvres gratings                                Barefoot gratings

Special-gratings are always individually adapted to customer requirements.

## PERFORATED METAL PLANKS

| TYPE BZ The perforated metal plank type BZ “Zahn” offers a high degree of slip resistance due to its serrated surface structure. For this reason, the BZ plank is especially suitable in those environments, where fats and oils are used. | BZ |
| --- | --- |
| TYPE BP The type BP is characterised by its smooth surface and high load-bearing capacity. For this reason, this type of profile metal plank is often used to cover large industrial areas where high demands are placed on the load capacity. Type BP can also be supplied without studs and is therefore very suitable for use in shelving applications. | BP-Ö |
| TYPE BN-O With perforated metal planks, type BN-O “Noppe, offen“ the bothsided punched holes have a diameter 8 mm and allows an excellent standing stability. Furthermore the drain holes exhibit good drainage. The BN-O grating guarantees a high stability for normal pedestrian traffic. | BN-O |
| TYPE BN-G The upwards pressed punched nobs of the perforated metal plank, type BN-G „Noppe, geschlossen“ are closed and offer a high slip resistance. BN-G gratings are often used in indoor environments and there, where, on the one hand, a closed surface is desired and on the other, good accessibility must be ensured. | BN-G |

TYPE BR

BR

The perforated metal plank type BR “Raute” is particularly suitable for areas of heavy loads on small contact areas due to its rhomb  profile with extra high punching (punched holes 2.5 mm).

BZ-GV

BZ-GP

BZ-G

BP-ON

BN-OP5

BN-OP

BN-OD

BN-OL

BN-GVI

BN-GA

PRODUCTS

MODELS

PERFORATED METAL PLANKS

IN USE

GRP pultruded

GRP yellow

GRP

APPLICATION OF GRP-PRODUCTS

IN THE OFFSHORE SECTOR AND

INDUSTRY

GRP PRODUCTS

MOULDED GRP-GRATINGS

Moulded GRP-gratings are produced in a negative mould. Depend- ing on the grating type, height, pitch and bar width, the appropriate mould is selected. The glass fibres are woven in a bond into the

form. The number and position of the glass fibres is determined for each type. The resin mix is produced in a mixing tool.

Their special advantage: Their weatherproof material means they require virtually no maintenance. In addition to our standard

panels, we produce many other types of GRP gratings in a variety of sizes and colours.

PULTRUDED GRP-GRATINGS

Pultruded GRP gratings are easy to install, highly durable and dimensionally stable to withstand extreme conditions.

Stairtread

Handrail profile

Square spar profile

MORE GRP-PRODUCTS

GRP structures, GRP railings, GRP ladders and GRP profiles are also part of the Lichtgitter Group‘s product range.

PRODUCTS

GRP PRODUCTS IN USE

## SPIRAL STAIRCASES

TYPE LG STANDARD

We manufacture the spiral staircases, type LG Standard in a hot-dip galvanized version according to DIN EN ISO 1461.

Our modular system is primarily used in the construction of

one-to two-storey staircases in industrial construction, but is also used in the private sector. Diameters up to 2,000 mm can be realised.

TYPE LG SPEZIAL

We manufacture the spiral staircases, type LG in a hot-dip

galvanized version according to DIN EN ISO 1461. This version of the staircase is characterised by the flexibility in height and diameter. This construction is used, among other things, as an escape staircase in multi-storey buildings, as platform access in industrial construction, as well as in the private sector.

SPIRAL STAIRCASES

PRODUCTS

IN USE

APPLICATIONS

APPLICATIONS

ARCHITECTURE

There are no limits to the use of gratings.

Their functionality and aesthetics allow them to be used as a decora- tive facade covering, a suspended ceiling or for sun protection.

APPLICATIONS

### MODELS

FLUSH FITTING SOLUTIONS

1. ANGLED Z-PROFILE

• binding in direction of cross bar standardized with Z-Profile 50/21/12/2 mm as angled bracket

+ optimum utilization of the height

3. ZU-PROFILE

• binding in direction of cross bar with ZU-Profile 20/70/20/12/2 mm (alternative with 40 mm

edging)

+ additional safety by locking at the supports

OVERLAY SOLUTIONS

2. BACK STOP

• binding in direction of cross bar standardized with Z-Profile 50/21/12/2 mm and additional upstand as back stop

+ sliding goods are prevented

3. WITH FIRE BAFFLES

• binding in direction of cross bar standardized with Z-Profile and fire baffles

+ higher fire safety (prevents chimney effect)

+

different dimensions of profiles, notches and prefabrications are possible on request

1. LOCKING MECHANISM

• binding in direction of cross bar with a deep

binding bar + 15 mm above bearing bar height as a locking mechanism

+ direct locking mechanism in the rack

2. DOUBLE LOCKING MECHANISM

• binding bar + 15 mm above bearing bar

• binding in direction of cross bar with deep binding bar generally + 10 mm or 15 mm above bearing bar including 2 cut outs with 56 mm × 15 mm as

additional locking mechanism

+ double locking mechanism

APPLICATIONS

STORAGE AND LOGISTICS

Many years of experience in the use of industrial floor coverings confirms that there are always new applications.

The advantages of using gratings and

perforated metal surfaces in the ware- housing and logistics sector are obvious:

Efficient, safe, quickly mounted, high free cross section= light and water permeabil- ity = increased fire protection.

Our team will help you choose the best product type for your application.

OFFSHORE

True to the motto “Lichtgitter – Every-  thing from one source” we offer you the range to meet your wind energy plant

requirements with all types of industrial floor coverings.

Whether platforms of gratings or che- quer plates or transformer coverings

to the point of helihoist, we give you an all-inclusive package backed-up by the established Lichtgitter quality. Due to our long lasting experience you can be assured that we will ensure quality and safety at all levels.

The advantage of our complete offering is that you can discuss everything with

one supplier and that you get all the pro- duct-related services like support during the planning period, preparation of lay- out-drawing, completion of the technical documentation from one source.

APPLICATIONS

Helihoist

Grating air strainer

Internal platforms

Transformer Coverings

GRP Handrails

GRP Vertical ladders

External platforms

GRP Cable tray

GRP-ladder

www.lichtgitter.com

EVERYTHING FROM ONE SOURCE:

forge-welded gratings, pressure-locked gratings,

perforated metal planks, GRP-gratings,

chequer plates, spiral staircases, stairtreads,

ladder rungs, hot-dip galvanising, powder-coating,

steel service.

Lichtgitter GmbH

Siemensstraße 1

D-48703 Stadtlohn

Fon   + 49. 25 63.911-0

Mail   info@lichtgitter.com

Lichtgitter Blechprofilroste GmbH & Co. KG

Neckarwiesen 20

D-72172 Sulz  |  Neckar

Fon   + 49. 74 54.95 82 -0

Mail   sulz@lichtgitter.com

Lichtgitter Treppen

GmbH & Co. KG

Industrie- und Gewerbegebiet 89 DE-16278 Pinnow

Fon   + 49. 33 33 5.30 17-0

Mail   pinnow@lichtgitter.com

Lichtgitter GFK GmbH & Co. KG

Siemensstraße 1

D-48703 Stadtlohn

Fon   + 49. 25 63.911-0

Mail   info@lichtgitter.com