METAL	BAR GRATING

AMERICAN NATIONAL STANDARD

MBG 534

ANSI/NAAMM MANUAL

-24

GR ATINGS

5

ANS I/NAAM M — MB5-3

XXXXXXXXXXX

METAL BAR GRATING ENGINEERING

DESIGN MANUAL

SECOND EDITION

ANS I/NAAM M — MBG 53

XXXXXXXXXXX

GR ATINGS

5

A Division of

NATIONAL ASSOCIATION OF

ARCHITECTURAL METAL MANUFACTURERS

This manual  was   developed  by  representative  members of  the  Metal  Bar  Grating  Division  (MBG)  of  the  National Association of Architectural Metal Manufacturers (NAAMM) to provide their opinion and guidance on the procedures used in design  calculations  for  metal   bar  grating.  This   manual contains  advisory  information  only  and  is  published  as  a public service by NAAMM. NAAMM disclaims all liabilityof any kind  for  the   use,  application  or  adaptation  of  material published in this manual.

Copyright © 2024

National Association  of Architectural  Metal  Manufacturers All Rights Reserved

METAL BAR GRATING

ENGINEERING DESIGN

MANUAL

Second Edition

NAAMM MBG 534-24

Published and distributed by the

NATIONAL ASSOCIATION OF ARCHITECTURAL METAL MANUFACTURERS

800 Roosevelt Road

Bldg. C, Suite 312

Glen Ellyn, Illinois 60137

Phone: (630) 942-6591

email: info@naamm.org

# METAL BAR GRATING

This manual sets forth procedures used in design calculations for metal bar grating. The load bearing capabilities and deflections of grating are based on the structural properties of the bearing bars and the number of bearing bars supporting the load. Grating is designed so that the allowablestresses of the metals used are not exceeded when the design loads are applied or, if deflection governs, the  specified  allowances  for  deflection  are   not  exceeded.   Metric   properties,  with  sample calculations, are included for metric conversions. All calculations are based on the AISC Manual of Steel construction, 15th edition.  All spans are designed as simple spans.

The  concentrated,  uniform  and  partially  distributed  uniform  loads  used  in  the  calculations  are modeled as static loads. Static loads are typically used to evaluate the functionality for live loading pedestrian load applications. Examples 1 – 5 on the following pages present the formulas used for calculating the load and deflection values of static loads.

Heavy rolling loads are defined as Vehicular Loads. Examples 6 – 7 present the formulas used to calculate the values for welded and pressure locked gratings when subjected to vehicular loads. Example 8 presents the formulas used to calculate the values for riveted gratings subjected to vehicular loads.

The Load Criteria presented for Vehicular Loads on page 12 is intended to serve as a guide for common vehicular applications. This criteria is not intended to be all-inclusive and if your application is not clearly represented by one of these options, contact NAAMM or your nearest NAAMM MBG member company for assistance in evaluating your specific application.

NOMENCLATURE

a = length of partially distributed uniform load or vehicular load, parallel with bearing bars, in.

b = thickness of rectangular bearing bar, in.

c = width of partially distributed uniform load or vehicular load, perpendicular to bearing bars, in. d = depth of rectangular bearing bar, in.

Ac = distance center to center of main bars, riveted grating, in.

Ar = face to face distance between bearing bars in riveted grating, in.

Aw = center to center distance between bearing bars in welded and pressure locked gratings, in.

C = concentrated load at midspan, pfw

Dc = deflection under concentrated load, in.

Du  = deflection under uniform load, in.

E = modulus of elasticity, psi F = allowable stress, psi

I = moment of inertia, in4

IH20  = moment of inertia of grating under H20 loading, in4

Ib  = I of bearing bar, in4

Ig = I of grating per foot of width, in4

In = moment of inertia of nosing, in4

K = number of bars per foot of grating width, 12"/Aw L = clear span of grating, in. (simply supported)

M = bending moment, Ib-in

Mb = maximum M of bearing bar, Ib-in

Mg = maximum M of grating per foot of width, Ib-in

N = number of bearing bars in grating assumed to carry load NbH20  = number of main bearing bars under load H20

NcH20  = number of connecting bearing bars under load H20 Pb  = load per bar, Ib

Pu = total partially distributed uniform load, Ib

PuH20  = wheel load, H20, Ib Pw = wheel load, lb

S = section modulus, in3

Sb = S of bearing bar, in3

Sg = S of grating per foot of width, in3

SH20b  = section modulus at bottom of grating under H20 loading, in3 Sn  = section modulus of nosing, in3

U = uniform load, psf

ABBREVIATIONS in. = inch

ft = foot

Ib = pounds

Ib-in = pound-inches

pfw = pounds per foot of grating width psf = pounds per square foot

psi = pounds per square inch

METAL PROPERTIES                               ENGLISH

|  | Allowable |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Design | Yield | Tensile | Modulus of |
|  | Stress | Strength | Strength | Elasticity |
|  | F psi | Fy psi | Fu psi | E psi |
| Steel |  |  |  |  |
| ASTMA1011 CS Type B | 18,000 | 30,000(1) |  | 29,000,000 |
| ASTMA1011 SS GR36 | 20,000 | 36,000 | 53,000 | 29,000,000 |
| ASTMA36 | 20,000 | 36,000 | 58,000 | 29,000,000 |
| Stainless Steel |  |  |  |  |
| ASTMA666 Type 304 | 20,000 | 30,000 | 75,000 | 28,000,000 |
| ASTMA666 Type 304L | 16,500 | 25,000 | 70,000 | 28,000,000 |
| ASTMA666 Type 316 | 20,000 | 30,000 | 75,000 | 28,000,000 |
| ASTMA666 Type 316L Aluminum* | 16,500 | 25,000 | 70,000 | 28,000,000 |
| ASTM B221 6061-T6 | 12,000 | 35,000 | 38,000 | 10,000,000 |
| ASTM B221 6063-T6 | 12,000 | 25,000 | 30,000 | 10,000,000 |
| ASTM B221 6005A-T61 | 12,000 | 35,000 | 38,000 | 10,000,000 |
| ASTM B221 6105-T5 | 12,000 | 35,000 | 38,000 | 10,000,000 |

•     None of the designs for MBG grating are for welded aluminum construction. The allowable stresses listed

may be below the values recommended by the Aluminum Association to assure acceptable serviceability and performance

METRIC

|  | Allowable |  |  |  |
| --- | --- | --- | --- | --- |
| Material | Design | Yield | Tensile | Modulus of |
|  | StressF | Strength | Strength | ElasticityE |
|  | MPa | Fy MPa | Fu MPa | MPa |
| Steel |  |  |  |  |
| ASTMA1011M CS Type B | 124.11 | 205(1) |  | 200,000 |
| ASTMA1011M SS GR250 | 137.90 | 250 | 365 | 200,000 |
| ASTMA36M | 137.90 | 250 | 400 | 200,000 |
| Stainless Steel |  |  |  |  |
| ASTMA666 Type 304 | 137.90 | 205 | 515 | 193,000 |
| ASTMA666 Type 304L | 113.77 | 170 | 485 | 193,000 |
| ASTMA666 Type 316 | 137.90 | 205 | 515 | 193,000 |
| ASTMA666 Type 316L Aluminum | 113.77 | 170 | 485 | 193,000 |
| ASTM B221M 6061-T6 | 82.74 | 240 | 260 | 69,000 |
| ASTM B221M 6063-T6 | 82.74 | 170 | 205 | 69,000 |
| ASTM B221M 6005-T61 | 82.74 | 240 | 260 | 69,000 |
| ASTM B221M 6105-T5 | 82.74 | 240 | 260 | 69,000 |

(1) Per ASTM A-1011 the typical (nonmandatory) range for yield strength for ASTM A 1011 CS Type B is 30 to 50 psi, or 205 to 344 MPa

FORMULAS

1.   Number of bearing bars per foot of width for welded grating, K = 12/AW

2.   Section modulus of rectangular bearing bar, Sb  = bd2/6 in3.

3.   Section modulus of grating per foot of width, Sg  = Kbd2/6 in3  = KSb  in3

4.   Section modulus required for given moment and allowable stress, S = M/F in3

5.   Moment of inertia of rectangular bearing bar, Ib  = bd3/12 in4  = Sb d/2 in4

6.   Moment of inertia of grating per foot of width, Ig  = Kbd3/12 in4  = Klb  in4

7.   Bending moment for given allowable stress and section modulus, M = SF Ib-in

The following formulas are for simply supported beams with  maximum  moments  and  deflections occurring at mid-span.

8.   Maximum bending moment under concentrated load M = CL/4 Ib-in per foot of grating width

9.   Concentrated load to produce maximum bending moment C = 4M/L Ib per foot of grating width

10. Maximum bending moment under uniform load

M = UL2/(8 x 12) = UL2/96 Ib-in per foot of grating width

11. Uniform load to produce maximum bending moment U = 96M/L2  psf

12. Maximum bending moment due to partially distributed uniform load M = Pu  (2L - a)/8 Ib-in

13. Maximum deflection under concentrated load Dc  = CL3/48EIg  in.

14. Moment of inertia for given deflection under concentrated load Ig  = CL3/48EDc  in4

15. Maximum deflection under uniform load

Du  = 5UL4/(384 x 12Elg) = 5UL4/4608EIg  in.

16. Moment of inertia for given deflection under uniform load

Ig  = 5UL4/4608EDu  in4

17. Maximum deflection under partially distributed uniform load Du  = Pu((a/2)3  + L3  - a2  L/2)/48Elb N in.

# SAMPLE CALCULATIONS

Example  1

These calculations show the procedures used to prepare data for metal bar grating load tables.

The concentrated midspan and uniform load bearing capabilities of W-19-4 (1-1/2 x 3/16) welded

A1011 CS Type B carbon steel grating and the corresponding midspan deflections will be calculated.

Allowable stress, F = 18,000 psi

Modulus of elasticity, E = 29,000,000 psi Span, L = 54 in.

Bearing bar spacing, Aw  = 1.1875 in.

Number of bearing bars per foot of width K = 12/Aw  = 12/1.1875 = 10.105

Section modulus of grating per foot of width

Sg = Kbd2/6 = 10.105 x 0.1875 (1.5)2/6 = 0.711 in3

Moment of inertia of grating per foot of width

Ig = Kbd3/12 = 10.105 x 0.1875 (1 .5)3/12 = 0.533 in4

Maximum bending moment for grating per foot of width Mg  = FSg  = 18,000 x 0.711 = 12,800 Ib-in

Concentrated Load

Load, C = 4Mg  /L = 4 x 12,800/54 = 948 pfw

Defl, Dc = CL3/48Elg  = 948 x (54)3/(48 x 29,000,000 x 0.533) = 0.201 in.

Uniform Load

Load, U = 96Mg /L2  = 96 x 12,800/(54)2  = 421 psf

Defl, Du  = 5UL4/4608Elg = 5 x 421 x (54)4/(4608 x 29,000,000 x 0.533) = 0.251 in.

Concentrated Mid Span Load per foot of width

Uniform Load per square foot

# GRATING SELECTION

Example  2 - Concentrated Load

Required: A welded ASTM A36 steel grating Type W-22-4 to support a concentrated load, C, of 4,000 pounds per foot of width at midspan on a clear span of 8'-0". Deflection, D, is not to exceed the 0.25" recommended for pedestrian comfort.

Allowable stress, F = 20,000 psi

Modulus of elasticity, E = 29,000,000 psi Span, L = 96in.

Bearing bar spacing, Aw  = 1.375 in.         K = 12/Aw  = 12 / 1.375 = 8.727

For a span of 8'-0", the minimum size bearing bar to sustain a 4,000 pfw load is:

3 x 3/8

Ig = Klb  = 8.727 x 0.8438 = 7.364 in4       Sg  = KSb  = 4.909 in3

C = 4Mg/L = 4 x F x Sg/96 = 4 x 20,000 x 4.909/96 = 4,091 pfw

Dc = CL3/48Elg  = 4,000 x (96)3/(48 x 29,000,000 x 7.364) = 0.345 in.

Since this exceeds the recommended limitation, a grating with a larger moment of inertia is needed to keep the deflection less than 0.25 in.

Ig = CL3/48EDc  = 4,000 x (96)3/(48 x 29,000,000 x 0.25) = 10.17 in4 Using the next larger size:

3-1/2 x 3/8

Ig = 8.727 x 1.3398 = 11.693 in4

Sg = 8.727 x 0.7656 = 6.682 in3

C = 4 x 20,000 x 6.682/96 = 5,568 pfw

D = 5,568 x (96)3/(48 x 29,000,000 x 11.693) = 0.303 in.

Deflection is directly proportional to load:

Dc  = 0.303 x 4,000/5,568 = 0.217 in. ≤ 0.25 in. OK

# Example  3 - Uniform Load

Required: A 6063-T6 aluminum grating Type  P-19-4 to support a  uniform  load,  U, of 300  pounds per square foot on a clear span of 5'-0". Deflection, D, is not to exceed the 0.25" recommended for pedestrian comfort.

Allowable stress, F = 12,000 psi

Modulus of elasticity, E = 10,000,000 psi Span, L = 60 in.

Bearing bar spacing, Aw = 1.1875 in.       K = 12/Aw  = 1211.1875 = 10.105

For a span of 5’-0”, the minimum size bearing bar to sustain a 300 psf load is:

1-3/4x3/16

Ig = Klb  = 10.105 x 0.0837 = 0.846 in4

Sg = KSb  = 10.105 x 0.0957 = 0.967 in3

U = 96Mg/L2  = 96 x F x Sg/(60)2  = 96 x 12,000 x 0.967/(60)2  = 309 psf

Du = 5UL4/4608Elg  = 5 x 309 x (60)4/(4608 x 10,000,000 x 0.846) = 0.514 in.

Deflection is directly proportional to load:

Du  = 0.514 x 300/309 = 0.499 in.

Since this exceeds the recommended limitation, a grating with a larger moment of inertia is needed to keep the deflection less than 0.25 in.

Ig = 5UL4/4608EDu  = 5 x 300(60)4/(4608 x 10 x 106 x 0.25) = 1.6875 in4

Using a larger size:

2-1/4 x 3/16    Ig  =1.798 in4          U=512 psf

D = 0.400 in.

Du  = 0.400 x 300/512 = 0.234 in. ≤ 0.25 in. OK

Note: Uniform loads in these examples and in the standard load tables do not include the weight of the gratings. In designing for uniform live loads the weight of the grating, as well as any other dead load, must be added.

# Example  4 -Partially   Distributed Uniform   Load

Required: A welded ASTM A1011 CS Type B steel grating Type W-19-4 to support a partially distributed uniform load, Pu, of 1,500 pounds over an area of 6" x 9" centered at midspan on a clear span of 3'-6". Deflection, D, is not to exceed the 0.25" recommended for pedestrian comfort.

Allowable stress, F = 18,000 psi

Modulus of elasticity, E = 29,000,000 psi Span, L = 42 in.

Bearing bar spacing, Aw  = 1.1875 in.

Since the 6" x 9" load is rectangular, two conditions must be investigated to determine which condition places the greater stress on the grating:

Condition ‘A’ - 6" dimension parallel to bearing bars

Condition ‘B’ - 9" dimension parallel to bearing bars

Condition ‘A’                  a = 6"                    c = 9"

Find maximum bending moment with load centered at midspan M = Pu(2L-a)/8 = 1,500 x (2 x 42 - 6)/8 = 14,625 Ib-in

Condition A

Find number of bars supporting load N = c/Aw  = 9/1.1875 = 7.58

Maximum bending moment per bearing bar Mb  = M/N =14,625/7.58 = 1,929 Ib-in

Condition ‘B’                  a = 9"                    c = 6"

Find maximum bending moment with load centered at midspan

M = Pu  (2L - a)/8 = 1,500 x (2 x 42 - 9)/8 = 14,063 Ib-in

Condition B

Find number of bars supporting load N = c/Aw  = 6/1.1875 = 5.05

Mb = M/N = 14,063/5.05 = 2,785 Ib-in

Condition ‘B’ produces greater bending moment and will be used to establish grating size. Required section modulus, Sb  = Mb/F = 2,785/18,000 = 0.155 in3

From Table A on page 19, select:

2-1/4 x 3/16 bar        Sb  = 0.1582 in3               Ib  = 0.1780 in4

Check deflection:

Du = Pu((a/2)3  + L3  - a2  L/2)/48Elb  N

= 1,500 x ((9/2)3  + 423  - 92  x 42/2)/(48 x 29 x 106  x 0.1780 x 5.05) = 0.087 in. < 0.25 in.

# STAIR TREADS

Grating stair tread design calculations are based on the following assumptions:

1. The front or nosing area of the tread receives the greatest load and use under normal conditions. The back or rear area of the tread is seldom used.

1.1. The strength of the tread is determined by and limited to the front 5 inches of the tread.

1.2.  The  bearing  area  of  the  nosing  is  approximately  1-1/4"  wide.  The  number  of  bearing  bars considered to be carrying the load depends on their center  to center  spacing and is  determined bythe formula:

N = (5 - nosing width)/(center to center distance of bars) +1

For 1-3/16 in. spacing,

N = (5 - 1.25)/(1.1875) + 1 = 4

For 15/16 in. spacing,

N = (5 - 1.25)/(0.9375) + 1 = 5

In calculating the strength of treads, the nosing plus “N” bearing bars is used.

2. For steel grating, all nosing is considered to have the strength of an angle 1-1/4" x 1-1/4" x 1/8". (S = 0.049 in3 ; I = 0.044 in4)

For aluminum grating, all nosing is considered to have the strength of an angle 1-1/4" x 1-1/4" x 3/16". (S = 0.071 in3 ; I = 0.061 in4)

These are the standard support angles used for cast abrasive nosings. Other shapes and configu- rations  may  be  used  by various  manufacturers,  but  to conform  to  NAAMM  Standards, the  nosing must meet or exceed the physical characteristics of these angles.

3. The tread is to support a midspan concentrated design load of 300 pounds with a deflection not to exceed L/240. L =

4. Treads over 5'-6" are to support the design load of 300 pounds at the one-third points of the span since loads may be applied at two points on longer treads. Deflection should not exceed L/240.

L =

Example 5

SAMPLE CALCULATIONS FOR TREAD DESIGN

5A.  Determine the  maximum span  length for a tread with  1-1/4"  x  3/16"  bearing  bars  on  1-3/16" centers. Use ASTMA1011 CS Type B steel.

Allowable design stress             F = 18,000 psi

Nosing

1-1/4 x 3/16 bar

Sn = 0.049 in3 Sb  = 0.0488 in3

In = 0.044 in4   Ib = 0.0305 in4

N = (5 - 1.25)/(1 .1875) + 1 = 4

Design S = Sn  + NSb  = 0.049 + 4 x 0.0488 = 0.244 in3 Design I = In  + NIb  = 0.044 + 4 x 0.0305 = 0.166 in4

Resisting Moment, M = SF = 0.244 x 18,000 = 4,390 Ib-in

Tread Span Length, L= 4M/C = 4 x 4,390 / 300 = 58.5 in.

L =  =   = 56.6 in. (controls)

5B. Determine the maximum span length for a tread with 1 x 3/16" bearing bars on 1-3/16" centers. Use ASTMA1011 CS Type B steel.

1 x 3/16 bar                   Sb  = 0.0313 in3                           Ib  = 0.0156 in4 Nosing properties and allowable design stress from “5A.”

Design S = 0.049 + 4 x 0.0313 = 0.1742 in3 Design I = 0.044 + 4 x 0.0156 = 0.1064 in4

Resisting Moment, M = 0.1742 x 18,000 = 3,140 Ib-in

Tread Span Length, L = 4M/C = 4 x 3,140 / 300 = 41.8 in. (controls)

L =  =  = 45.3 in.

5C.  Determine the  maximum span  length for a tread with  1-1/4" x 3/16" bearing  bars on  1-3/16” centers. Use 6063-T6 aluminum.

Allowable Design Stress, F = 12,000 psi

Nosing                           Sn  = 0.071 in3                               In  = 0.061 in4

1-1/4" x 3/16"                 Sb  = 0.0488 in3                              Ib  = 0.0305 in4

Design S = 0.071 + 4 x 0.0488 = 0.266 in3 Design I = 0.061 + 4 x 0.0305 = 0.183 in4

Resisting Moment, M = SF = 0.266 x 12,000 = 3,190 Ib-in

Tread Span Length, L = 4M/C = 4 x 3,190/300 = 42.5 in.

L =  =  = 34.9 in. (controls)

5D. Determine the maximum span length for a tread with 1" x 3/16" bearing bars on 1-3/16" centers. Use 6063-T6 aluminum.

1" x 3/16" bar                Sb  = 0.0313 in3                           Ib  = 0.0156 in4

Nosing properties and allowable design stress from “5C” .

Design S = 0.071 + 4 x 0.0313 = 0.1962 in3 Design I = 0.061 + 4 x 0.0156 = 0.1234 in4

Resisting Moment, M = SF = 0.1962 x 12,000 = 2,350 Ib-in Tread Span Length, L = 4M/C = 4 x 2,350/300 = 31.3 in.

L =  =   = 28.6 in. (controls)

5E. Determine the bearing bar size for a tread with bearing bars at 1-3/16" on center having a length of 6'-4". Use ASTM A1011 CS Type B steel. Assume a bearing bar thickness of 3/16 and find the

bearing bar depth:

Load distribution for treads over 5'-6":

300 lbs          300 lbs

1/3

1/3      l        1/3        l

Tread Length

Design Formula: L =

L = 76 inches

E = 29,000,000 lb/in2 P = 300 lbs

I = 8.5PL² / E = 8.5 x 300 x 76² / 29,000,000 = 0.5079 in4

Design I = In + N Ib In = 0.044 in4

I = 0.05079 in4

N = 4

Ib = (I – I )/N = (0.05079 - 0.044) / 4 = 0.1160 in4

Ib = bd3/12

Ib = moment of inertia per bearing bar, in4

b = thickness of bearing bar, in d = depth of bearing bar, in

d3 = (12 x Ib )/b = (12 x (0.116))/(.1875) = 7.424 in3 d = 1.9508 in

Therefore, for a Type 19-4 tread with a length of 6'-4" the recommended bearing bar size would be 2 x 3/16".

Check for stress: F = 18,000 psi

S = S n+ NS  = 0.049 + 4(0.125) = 0.549 in3

Resisting moment M = SF = 0.549 x 18,000 = 9,882 in lbs Maximum moment applied = 300 (76/3) = 7,600 in lbs

Deflection controls

# Vehicular Loads

Welded and  Pressure  Locked  Construction

The following load criteria and design calculations (Examples 6 and 7) were used to prepare data for the Vehicular Load Tables found in NAAMM publication MBG-532, “Heavy Duty Metal Bar Grating Manual” . These examples apply to welded and pressure locked gratings only (for riveted gratings see the design calculations related to Example 8).

Load distribution criteria conforms with the AASHTO Standard Specification for Highway Bridges, 16th Edition, paragraph 3.27.3.1. This specification for open floors states that H-20 / HS-20 loads shall be distributed over an area 20" x 20" plus one bearing bar on each side of the wheel load. The wheel load for H-20 / HS-20 loads is 20,800 lbs. which includes a 30% impact factor per paragraph 3.8.2.1.

For maximum service life it is recommended that deflection for gratings subject to vehicular loads be restricted to the lesser of .125 inches or L/400. Additionally, to reduce the effects of impact and fatigue, it is recommended that gratings subject to heavy, high speed or multi-directional traffic be specified with load carrying banding.

## LOAD CRITERIA – Vehicular  Loads (Applicable to all gratings)

|  | AASHTO H-25 | AASHTO H-20 | AASHTO H-15 | Passenger Vehicles | 5 Ton Forklifts | 3 Ton Forklifts | 1 Ton Forklifts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Truck/Vehicle Weight (pounds) |  |  |  | 6,322 | 14,400 | 9,800 | 4,200 |
| Load Capacity (pounds) |  |  |  | 3,578 | 10,000 | 6,000 | 2,000 |
| Axle Load (pounds) | 40,000 | 32,000 | 24,000 |  |  |  |  |
| Impact Factor | 30% | 30% | 30% | 30% | 30% | 30% | 30% |
| Total Load (pounds) | 52,000 | 41,600 | 31,200 | 12,870 | 31,720 | 20,540 | 8,060 |
| % of load on drive axle |  |  |  | 60% | 85% | 85% | 85% |
| Pw = Wheel Load (pounds) | 26,000 | 20,800 | 15,600 | 3,861 | 13,481 | 8,730 | 3,426 |
| a = Length of Distribution parallel to bearing bars (inches) | 25 | 20 | 15 | 9 | 11 | 7 | 4 |
| c = Width of Distribution perpendicular to bearing bars (inches) | 25 * | 20 * | 15 * | 9 * | 11 * | 7 * | 4 * |
|  | * plus 2 x the center/center spacing of bearing bars |  |  |  |  |  |  |

Design Calculations:

Sb = section modulus per bar, in3

F = allowable stress, psi

a = length of distribution parallel to bearing bar, in

c = width of distribution perpendicular to bearing bar, in Pb  = load per bar, lb

E = modulus of elasticity, psi Ib  = I of bearing bar, in4

L = clear span, in

Maximum Span = 4((Sb / (Pb/F)) + (a / 8)

Deflection - Max Span = [P b/(96)(E)(Ib)]  [2(L)3 - a2  x L + a3/4]

Example  6 - Vehicular  Load  Calculations

Required:  Determine  maximum span for A-36 carbon steel grating, type W-22-4 3-1/2" x 3/8" for AASHTO H-20 loads. Deflection to be the lesser of .125 inches or L / 400.

| Proposed Grating: | W-22-4 3-1/2" x 3/8" |
| --- | --- |
| Bearing bar spacing | 1.375" |
| Bearing bar depth | 3-1/2" |
| Bearing bar thickness | 0.375" |

Area of Distribution:

a = Parallel to bearing bars = 20"

c = Perpendicular to bearing bars = 22.75" Pw  = Wheel Load = 20,800 lb.

Material: A-36 Carbon Steel

F = 20,0000 psi

E = 29,000,000 psi

N = Number of bars supporting load = c / 1.375 = 16.545 Pb  = load per bar = (Pw  / N) = 1,257 lb.

Pb / F = .0629, in2

Ib = bd3 / 12 = 1.3398 in4

Maximum Span = 4((Sb  / (Pb/F)) + (a / 8)) = 58"

Deflection - Maximum Span = [Pb  / (96)(E)(Ib)]  [2(L)3  - a2 x L + a3/4] = .124"

Partially Distributed Vehicular Loads

The following design calculations were used to prepare data for vehicular loads when the tire contact length exceeds the clear span of the grating.

Example  7 – Partially   Distributed Vehicular   Load

Required: Determine a type W-22-4, A-36 carbon steel grating, capable of supporting H-20 loads over a 15 inch clear span, maximum deflection not exceeding the lesser of .125 inches or L/400.

Load Condition: AASHTO H-20

Area of Distribution:

a = Length of distribution parallel with bearing bars = 20"

c = Width of distribution perpendicular to bearing bars = 22.75"

Pw  = Wheel load (pounds) = 20,800 lb.

Proposed Grating:          W-22-4  1-1/2" x 1/4"

Bearing bar spacing         1.375"

Bearing bar depth            1.50"

Bearing bar thickness      .25"

Material:            A-36 Carbon Steel

F = 20,000 psi

E = 29,000,000 psi

L = clear span in inches = 15"

M = maximum bending moment = (Pw/a)(L²/8) = 29,250 in. lb.

Sg  = required section modulus of grating = M / F = 1.4625 in³

Aw  = center to center spacing of bearing bars = 1.375"

N = number of bars supporting load = c / Aw  = 16.5455

Sb = required section modulus per bar = Sg / N = .089 in³/bar

Section Modulus Required per Bar = .089 in³

Section Modulus of Proposed Bar = 0.094 in³

Deflection Calculation:

Ib = Moment of inertia of proposed bar = b x d³ / 12 = 0.070 in4/bar Ig  = Moment of inertia - grating = Ib  x N = 1.163 in4

Deflection =  ((5*(Pw/a) L4)) / (384 EIg)

Deflection equals = 0.020"

(Deflection ≤ .125" and L/400)

Riveted  Heavy  Duty  Gratings

The following design calculations were used to prepare data for the Vehicular Load Tables found in NAAMM publication MBG-532, “Heavy Duty Metal Bar Grating Manual” . These design calculations and Example 8 apply specifically to type R-37-5 Riveted Grating.

Example 8 – Riveted  Heavy Duty Vehicular Loads

Determine the maximum clear span “L” with simple supports for type R-37-5 size 5" x 1/4" A-36 Steel heavy duty riveted grating under AASHTO / HS-20 loading with maximum impact factor of 30%.

Design Formulas and Descriptions:                                                    Values

SH20b = Section modulus at bottom of grating under HS-20 loading. = 11.56 (see calculations below)

| IH20 = Moment of inertia of grating under HS-20 loading | = 32.305 (see calculations below) |
| --- | --- |
| F = Max allowable fiber stress for ASTM A36 steel | = 20,000 psi |
| a = Length of load distribution parallel to bearing bars | = 20" |
| c = Width of load distribution perpendicular to bearing bars.* | = 20" + 2 bearing bars |
| Ac = Distance center to center of main bearing bars = Ar + b | = 2.5625" |
| NbH20 = Number of main bearing bars under load H-20 | = 9.8 (see calculation below) |
| NcH20 = Number of connecting bearing bars under load | = 8.8 (see calculation below) |
| PUH20 = Partially distributed uniform load = Wheel load. | 20,800 lbs |
| E = Modulus of elasticity (psi) | 29,000,000 psi |
| M = Max moment applied to grating section | 231,200 in Ibs (see calculation below) |
| L = Maximum clear span (in inches) | 54.5 inches (see calculation be- low) |
| Max moment = M = SH20b x F | Reference formula number 4 |
| Maximum span L = ((8M+(a x PUH20)) / (2 PUH20) | Reference formula number 12 |
| Deflection at max span = PuH20 ((a/2)3 + L3 - a2 L/2) / (48 E IH20) | Reference formula number 17 |

* c = a + 2 Ac              Reference AASHTO Specification for Bridges 16th edition paragraph 3.27.3.1 NbH20  = (a / Ac) + 2

NcH20  = (a / Ac) + 1

Using the formulas above:

NbH20  = (20 / 2.5625) + 2 = 9.8 main bearing bars with each having thickness of 0.25" The depth of the main bearing bars is d main  = 5"

NcH20  = (20 / 2.5625) + 1 = 8.8 connecting bars with each having thickness of 0.1875" The depth of the connecting bars is d connecting   = 1.5"

The resulting section of riveted grating under load is modeled as a T section with dimensions shown below.

4.102 1.500 5.000 2.794 2.451 Distance to Centroid

In the T section the centroid distance and second moment of inertia “IH20” can be calculated using the paral- lel axis theorem.

IH20  = 32.305 in4  and the centroid distance to bottom is as shown 2.794" SH20b  = IH20  / centroid distance to bottom = 32.305 / 2.794 = 11.56 in3

The maximum moment that can be applied to the section is M = SH20b  x F = 11.56 x 20,000 = 231,200 in Ibs

Now using formula 12, Maximum Span is calculated as follows

L = ((8xM)+(a x PUH20)) / (2 PUH20) = ((8 x 231,200) +(20 x 20,800) / (2 x 20,800) = 54.5 inches.

Deflection Calculation:

Du = Pu  [(a/2)3  + L3  - a2  L/2] / (48 E IH20)

Du = 20,8000 [(20/2)3  + (54.5)3  - (20)2  x 54.5 / 2] / (48 x 29,000,000 x 32.305)

Du = 0.070 in (≤ .125” and L/400)

# METAL BAR GRATING DESIGN MANUAL METRIC PRACTICE

The system of metric measurement used in this manual is from IEEE/ASTM SI 10-2002, “Standard for Use of the International System of Units (SI): The Modern Metric System.”

SI prefixes:

k (kilo) = 103

M (mega) = 106

G (giga) = 109

Corresponding units:

Length - meter, m  millimeter, mm Force - newton, N

Stress - pascal, Pa (newton/square meter)

Bending Moment - newton meter, N-m Conversion factors:

1 in. = 0.0254 m = 25.4 mm

1 in2  = 645.16 mm2

1 in3 = 16,387 mm3

1 in4 =416,231 mm4

1 ft = 0.3048 m = 304.8 mm

1 ft2 = 0.092903 m2  = 92,903 mm2

1 Ib (force) (Ib) = 4.44822 N

1 kip = 4.44822 kN

1 psi = 4.44822N/645.16mm2  = 0.006894757 N/mm2  = 6,894.757 N/m2 = 6,894.757 Pa = 6.894757 kPa = 0.006894757 MPa

1 psf = 47.88026 Pa = 0.04788026 kPa

1 pfw = 4.44822N/0.3048m = 14.5939 N/m of width

Sample Calculations - Stainless Steel Bar Grating

Load bearing capabilities and corresponding midspan deflections of a W-19 (1 x 1/8) welded stainless steel grating are calculated.

Using ASTM A666 Type 304

Allowable stress, F = 137.90 MPa

Modulus of elasticity, E = 193,000 MPa

Aw= 1.1875 in. = 30.16 mm

K = 12/Aw  = 12/1.1875 = 12 x 25.4/30.16 = 10.105

Sg = Kbd2/6 = 10.105 x (0.125 x 25.4) x (1 x 25.4)2/6 = 3,450 mm3

Ig = Kbd3/12 = 10.105 x (0.125 x 25.4) x (1 x 25.4)3/12 = 43,800 mm4

If section modulus and moment of inertia are known in inch units, conversion is even simpler, and the result will be well within engineering accuracy.

If Sg = 0.21 in3  and Ig  = 0.1053 in4 , then Sg  = 0.21 x 16,387 = 3,440mm3

Ig = 0.1053 x 416,231 = 43,800 mm4

To calculate loads and deflections for a 0.762 m (2'-6") span, the following procedures may be used:

Maximum bending moment for grating per 0.3048 m (1 ft) width Mg  = F x Sg  = 137.90 MPa (N/mm2) x 3,450 mm3

= 475,728 N mm = 475.7 N m

per 0.3048 m of width

Concentrated Load at Midspan

Load, C = 4Mg/L = 4 x 475.7/0.762

= 2,497 N per foot of width

= 2,497/(103 x 0.3048)

= 8.19 kN per meter of width Defl, Dc  = CL3/48EI

= 2,497 x 0.7623 x 109/(48 x 193,000 x 43,800)

= 2.72 mm

Uniform Load

Load, U = 8Mg/L2   = 8 X 475.6/0.7622

= 6,553 N/m per foot of width

= 6,553/(103 x 0.3048)

= 21.50 kN/m2

Defl, Du  = 5UL 4/384EI

= 5 x 6,553 x 0.7624  x 109/(384 x 193,000 x 43,800)

= 3.40 mm

## TABLE A

SECTION MODULI AND MOMENTS OF INERTIA FOR RECTANGULAR  BEARING BARS

S = Section Modulus, in3 (mm3)

I = Moment of Inertia, in4  (mm4)

| Depth of Bar |  |  | Thickness of Bar in. (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1/8 | (3.2) | 3/16 (4.8) |  | 1/4 | (6.4) | 3/8 | (9.5) |
| in. | (mm) |  |  |  |  |  |  |  |  |  |
| 3/4 | (19.0) | S I | 0.0117 0.0044 | (192) (1 ,830) | 0.0176 0.0066 | (288) (2,750) | 0.0234 0.0088 | (383) (3,660) | 0.0352 0.0132 | (577) (5,490) |
| 1 | (25.4) | S I | 0.0208 0.0104 | (341) (4,330) | 0.0313 0.0156 | (513) (6,490) | 0.0417 0.0208 | (683) (8,660) | 0.0625 0.0313 | (1,024) (13,000) |
| 1-1/4 | (31.8) | S I | 0.0326 0.0203 | (534) (8,450) | 0.0488 0.0305 | (800) (12,700) | 0.0651 0.0407 | (1,067) (16,940) | 0.0977 0.0610 | (1 ,600) (25,400) |
| 1-1/2 | (38.1) | S I | 0.0469 0.0352 | (769) (14,650) | 0.0703 0.0527 | (1,152) (21,900) | 0.0938 0.0703 | (1,537) (29,300) | 0.1406 0.1055 | (2,300) (43,900) |
| 1-3/4 | (44.4) | S I | 0.0638 0.0558 | (1,045) (23,200) | 0.0957 0.0837 | (1,568) (34,800) | 0.1276 0.1117 | (2,090) (46,500) | 0.1914 0.1675 | (3,140) (69,700) |
| 2 | (50.8) | S I | 0.0833 0.0833 | (1,365) (34,700) | 0.1250 0.1250 | (2,050) (52,000) | 0.1667 0.1667 | (2,730) (69,400) | 0.2500 0.2500 | (4,100) (104,000) |
| 2-1/4 | (57.2) | S I | 0.1055 0.1187 | (1,728) (49,400) | 0.1582 0.1780 | (2,590) (74,100) | 0.2109 0.2373 | (3,460) (98,800) | 0.3164 0.3560 | (5,180) (148,200) |
| 2-1/2 | (63.5) | S I | 0.1302 0.1628 | (2,130) (67,800) | 0.1953 0.2441 | (3,200) (101,600) | 0.2604 0.3255 | (4,270) (135,500) | 0.3906 0.4883 | (6,400) (203,000) |
| 3 | (76.2) | S I | 0.1875 0.2813 | (3,070) (117,000) | 0.2813 0.4219 | (4,610) (175,600) | 0.3750 0.5625 | (6,140) (234,000) | 0.5625 0.8438 | (9,220) (351,000) |
| 3-1/2 | (88.9) | S I | 0.2552 0.4466 | (4,180) (186,000) | 0.3828 0.6699 | (6,270) (279,000) | 0.5104 0.8932 | (8,360) (372,000) | 0.7656 1.3398 | (12,550) (558,000) |
| 4 | (101 .6) | S I | 0.3333 0.6666 | (5,460) (277,000) | 0.5000 1.0000 | (8,190) (416,000) | 0.6667 1.3333 | (10,920) (555,000) | 1.0000 2.0000 | (16,390) (832,000) |
| 4-1/2 | (114.3) | S I | 0.4219 0.9492 | (6,910) (395,000) | 0.6328 1.4238 | (10,370) (593,000) | 0.8438 1.8984 | (13,830) (790,000) | 1.2656 2.8477 | (20,700) (1,185,000) |
| 5 | (127.0) | S I | 0.5208 1.3021 | (8,534) (542,000) | 0.7813 1.9531 | (12,803) (813,000) | 1.0417 2.6042 | (17,070) (1,084,000) | 1.5625 3.9063 | (25,605) (1,626,000) |

Note: The bearing bar sizes in this table are those which are covered in the NAAMM Metal Bar Grating Standards.